3GPP TSG-RAN WG2 Meeting #130	R2-2504533

St Julian’s, Malta, May 19th- 23th, 2025

Source:	KDDI Corporation

Title:	Discussion on On-demand SIB1

Tdoc Type:	Discussion

Agenda Item:	8.5.3

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#127b and RAN2#129, the following agreements were made.

Agreements on OD-SIB1@RAN2#127b

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#129

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will provide our further consideration on on-demand SIB1.

2	Discussion

2.1 whether SIB-x in cell A is essential SIB

It is agreed in RAN2 that the WUS configuration for OD-SIB1 to be included in SIB-x, a new system information block.

In legacy system, only SSB and SIB1 are considered as essential system information. For SIB-x, if it is considered as essential system information, Cell A has to broadcast it periodically, similar as SSB and SIB1, which will cause extra energy consumption of Cell A. From network energy saving perspective, we prefer not to have SIB-x as essential system information.

Proposal 1: SIB-x is not considered as essential system information.

If SIB-x is not considered essential system information, Cell A can configure it as "not broadcasted" from a network energy saving perspective, and only broadcast it upon explicit request from the UE. In this case, UEs can reuse the legacy SI-Request procedure.

Proposal 2: SIB-x can be configured as “not-broadcasting” for network energy saving.

Proposal 3: UE request SIB-x via legacy SI-Request procedure

2.2 Access related information in UL WUS

R2-2502916 On demand SIB1 handling Nokia, Nokia Shanghai Bell discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access. [Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information. Offline among interested companies. Comeback in CB session (Nokia). [Comeback session]: [Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia. Revisit it next meeting.

In the last meeting, access-related information in UL WUS was discussed, but no consensus was reached. From our perspective, we do not see a need to include cell access-related information in the WUS configuration. The CellAccessRelatedInfo provides access-related information for the cell, i.e., the serving cell. Therefore, if included in the WUS configuration, it would represent the access information for the NES cell. However, the UE can acquire this information from the NES SIB1 after completing the OD-SIB1 request procedure. Furthermore, as mentioned by several companies during the meeting, including CellAccessRelatedInfo in the WUS configuration would increase configuration overhead and reduce energy efficiency for Cell A.

Proposal 4: Cell access related information is not needed in the WUS configuration.

2.3 Automatic Neighbour Relation

gNB manages the NCR(neighbor cell relation) information to facilitate handover (HO) between gNBs. There are two main methods for creating the NCR: one is manual creation by the operator, and the other is automatic creation through ANR (Automatic Neighbor Relations).

For the automatic creation of NCR through ANR, UE needs to obtain the NCGI information of the target cell and report it to the source cell. In the on-demand SIB1 scenario, there is no periodic transmission of SIB1 from NES cell, so UE cannot acquire SIB1 for NES cell.

As a result, ANR cannot function properly, leading to the issue of being unable to automatically create NCR. Manually creating all NCRs would significantly increase operational costs, so it is necessary to consider the issues in the on-demand SIB1 scenario.

Proposal 5: RAN2 discuss the necessity of including ANR in the on-demand SIB1 scenario.

To achieve the coexistence of on-demand SIB1 and ANR, it is necessary to discuss specific methods. For example, One approach is that when gNB detects ANR activity, it could coordinate with other gNB or higher-level network equipment to stop the SIB1-less mode of NES cells through network interface and have NES Cell transmit SIB1 periodically at all times. Another approach could be for UE to obtain SIB1 information of the NES cell in advance before executing ANR operations. In the latter case, if the UE is to acquire the necessary SIB1 information for ANR in advance, it must first receive and store the required information internally, which would lead to a significant burden on UE's implementation. Therefore, it is considered more straightforward to first consider solution like stopping the SIB1-less mode on the network side in RAN3, rather than considering solution in RAN2.

Proposal 6: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3

3	Conclusion

We make the following observations and proposals:

Proposal 1: SIB-x is not considered as essential system information.

Proposal 2: SIB-x can be configured as “not-broadcasting” for network energy saving.

Proposal 3: UE request SIB-x via legacy SI-Request procedure

Proposal 4: Cell access related information is not needed in the WUS configuration.

Proposal 5: RAN2 discuss the necessity of including ANR in the on-demand SIB1 scenario.

Proposal 6: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3