3GPP TSG-RAN WG2 Meeting #130	R2-2504541

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.19

Source:				Samsung

Title:						Leftover issue on ANR reporting of HSDN cells in INM

measurement

Document for:		Discussion

1 Introduction

In RAN2#129 meeting, RAN2 agreed to have UE-based solution for solving the issue of ANR reporting of HSDN cells as per below:

For NR specifications, it is proposed RAN2 to agree on UE-based solution for solving the issue of ANR reporting of HSDN cells: - - allow the UE to report HSDN indication for the intra-frequency or inter-frequency neighbouring cell via CGI reporting procedure, and introduce a new UE capability bit (optional with signalling) for it - - allow the UE to report HSDN indication for the E-UTRA neighbouring cell via CGI reporting procedure, and introduce a new UE capability bit (optional with signalling) for it For LTE specifications, it is proposed RAN2 to agree on UE-based solution for solving the issue of ANR reporting of HSDN cells: - - allow the UE to report HSDN indication for the intra-frequency or inter-frequency neighbouring cell via CGI reporting procedure, and introduce a new UE capability bit (optional with signalling) for it - - allow the UE to report HSDN indication for the NR neighbouring cell via CGI reporting procedure, and introduce a new UE capability bit (optional with signalling) for it

Accordingly, the following CRs are in-principle agreed in the latest RAN2#129-bis meeting.

R2-2502996 	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.5.1	5318	1	B	TEI19

- The CR is in principle agreed

R2-2502784	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.306	18.5.0	1264	-	B	TEI19

- The CR is in principle agreed

R2-2502997 	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.331	18.5.0	5110	1	B	TEI19

- The CR is in principle agreed

R2-2502786	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.306	18.4.0	1911	-	B	TEI19

- The CR is in principle agreed

RAN2 haven’t discussed INM apsects for ANR reporting of HSDN cells at all. We would like to discuss whether the field hsdn-Cell needs to be exchanged between nodes.

2 Discussion

When network intends to instruct a certain UE to perform measurements on neighbour cells for ANR purpose, the UE can be configured with at most one measurement identity across all CGs using a reporting configuration with the reportType (or purpose) set to reportCGI. Hence, configuration of ANR requires coordination between MN and SN in INM when the UE is in MR-DC.

Observation 1: Since the UE can only be configured with a single reportCGI configuration from either MN or SN, configuration of ANR requires coordination between MN and SN for MR-DC.

When the UE is (NG)EN-DC and NR-DC, the reportCGI-RequestNR in CG-Config is used by SN to indicate to MN about configuring reportCGI procedure. If configured, the UE sends a measurement report to the network by including the IE CGI-InfoNR for the indicated/detected NR neighbour cell. Accordingly, the measResultReportCGI in CG-ConfigInfo is used by MN to provide SN with the received CGI-InfoNR for the cell as per SN’s request.

CG-ConfigInfo-v1540-IEs ::=     SEQUENCE {

ph-InfoMCG                      PH-TypeListMCG                      OPTIONAL,

measResultReportCGI             SEQUENCE {

ssbFrequency                    ARFCN-ValueNR,

cellForWhichToReportCGI         PhysCellId,

cgi-Info                        CGI-InfoNR

}

Note that hsdn-Cell-r19 will be also transferred from MN to SN via INM since it is added to the CGI-InfoNR thanks to the extension marker "...", even though RAN2 hasn’t discussed whether such HSDN indication needs to be transferred from MN to SN.

CGI-InfoNR ::=                    SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList               OPTIONAL,

frequencyBandList                   MultiFrequencyBandListNR            OPTIONAL,

noSIB1                              SEQUENCE {

ssb-SubcarrierOffset                INTEGER (0..15),

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1

}                                                                       OPTIONAL,

...,

[[

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16            OPTIONAL

]],

[[

cellReservedForOtherUse-r16         ENUMERATED {true}                   OPTIONAL

]],

[[

hsdn-Cell-r19         				ENUMERATED {true}                   OPTIONAL

]]

}

Observation 2: For (NG)EN-DC and NR-DC, HSDN indication for the NR neighbour cell is transferred from MN to SN if reported by UE as part of report CGI procedure.

When the UE is NE-DC, the measResultReportCGI-EUTRA in CG-ConfigInfo is used by MN to provide SN with the received CGI-InfoEUTRA for the cell as per SN’s request.

CG-ConfigInfo-v1560-IEs ::=  SEQUENCE {

(omitted)

measResultReportCGI-EUTRA               SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA           EUTRA-PhysCellId,

cgi-InfoEUTRA                           CGI-InfoEUTRA

}                                                                           OPTIONAL,

Unlike ANR reporting of NR neighbour cell, the new field hsdn-Cell-r19 is NOT added within the CGI-InfoEUTRA due to the absence of extension marker. It means that HSDN indication for the E-UTRA neighbour cell is NOT transferred from MN to SN even if reported by UE as part of report CGI procedure.

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...,

[[

hsdn-Cell-r19         				ENUMERATED {true}                   OPTIONAL

]]

}

Observation 3: For NE-DC, HSDN indication for the E-UTRA neighbour cell is NOT transferred from MN to SN even if reported by UE as part of report CGI procedure.

The main motivation for introducing ANR reporting of HSDN cells is to relieve the operator from the burden of manually managing NCRs. Given different outcomes in INM between (NG)EN-DC/NR-DC and NE-DC due to ASN.1 structure, we think that RAN2 should make concrete agreements what is correct network behavior. We generally think that there is no need for MN to filter part of information received from UE. Since any cell in SN can be operated as intra-frequency/inter-frequency/inter-RAT frequenct cell, there seems to be no remarkable reason to support only the transfer HSDN indication of the NR neighour cell between nodes.

Proposal 1: RAN2 to confirm that HSDN indication for the NR neighbour cell is transferred from MN to SN if reported by UE as part of report CGI procedure for (NG)EN-DC and NR-DC. No specification change is needed.

Proposal 2: RAN2 to discuss whether to transfer HSDN indication for E-UTRA neighour cell from MN to SN i.e. introduce hsdn-Cell-r19 in CG-ConfigInfo-v1900-IEs.

3 Conclusion

In section 2, the following observations are made:

Observation 1: Since the UE can only be configured with a single reportCGI configuration from either MN or SN, configuration of ANR requires coordination between MN and SN for MR-DC.

Observation 2: For (NG)EN-DC and NR-DC, HSDN indication for the NR neighbour cell is transferred from MN to SN if reported by UE as part of report CGI procedure.

Observation 3: For NE-DC, HSDN indication for the E-UTRA neighbour cell is NOT transferred from MN to SN even if reported by UE as part of report CGI procedure.

Based on above, RAN2 is requested to discuss and agree on the following proposal:

Proposal 1: RAN2 to confirm that HSDN indication for the NR neighbour cell is transferred from MN to SN if reported by UE as part of report CGI procedure for (NG)EN-DC and NR-DC. No specification change is needed.

Proposal 2: RAN2 to discuss whether to transfer HSDN indication for E-UTRA neighour cell from MN to SN i.e. introduce hsdn-Cell-r19 in CG-ConfigInfo-v1900-IEs.

4 Reference