3GPP TSG-RAN WG2 Meeting #130                                                              R2-2504546

St.Julians, Malta, May 19th- 23th, 2025

Agenda item:	8.13.2

Source:	Samsung

Title:	Considerations on relay discovery and (re)selection

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the relay discovery and (re)selection was extensively discussed for multi-hop SL relay, and reached the following agreement:

Agreement:

The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop.

Agreements:

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

Agreements:

Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected).

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item.  This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI.

Agreements:

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In this contribution, we will share our further thinking on the discovery and relay selection/re-selection.

Discussion

UE type

In multi-hop SL relay, four types of UE are defined, i.e., remote UE, first relay UE, intermediate relay UE, and last relay UE. The discovery among those UEs can be performed. If an UE can be acted as one or some types of UE, the connection establishment between two UEs depends on the UE type. For example, the remote UE cannot allow the access of other UEs, the first relay UE can only allow the access of remote UE. Thus, during the discovery procedure, each UE should be able to identify whether the discovered UE can be accessed or not. To achieve this purpose, one possible way is to indicate the capability in the discovery message. Another way is to indicate the supported number of hops in the discovery message.

Proposal 1-1: RAN2 is kindly asked to discuss how to determine whether the PC5 connection can be established with the discovered UE in multi-hop SL relay.

R19 supports multiple hops between remote UE and the gNB. However, the total number of hops are limited. Thus, the remote UE/relay UE should not connect with its parent node when the total number of hops exceeds the supported number of hops. To avoid this, the discovery message can contain upstream hop count related information.

Proposal 1-2: RAN2 is kindly asked to discuss whether the upstream hop count information is indicated to its child UE.

On the other hand, the capacity of each intermediate relay UE is limited. Thus, the number of supported child UEs or the number of supported downstream hops would be limited as well. In this sense, each intermediate relay UE can indicate its capacity information to the child UE so that each child UE can determine whether to access such intermediate relay UE.

Proposal 1-3: RAN2 is kindly asked to discuss whether the number of supported downstream hops is indicated to its child UE.

Notification message design

- Issue 1: transmission of the notification message

Each node may trigger its own notification message independently. After the initiation, the transmission of the notification message can be carried out by considering the following two options:

  - Option 1: transparent delivery

This option is the simplest one, i.e., each relay UE independently generates the content of the notification message, and each message is forwarded hop-by-hop transparently. This requires to include the notification message of each node as container of PC5-RRC message.

  - Option 2: non-transparent delivery

This option allows each intermediate relay UE to regenerate the content of the notification message, i.e., if the intermediate relay UE wants to add the state information (i.e., indication type) of itself, it can add its own state in the new generated notification message, and forward it to its child node.

Compared to Option 1, Option 2 is more signalling efficient. So, we prefer to Option 2.

Proposal 2-1: RAN2 is kindly asked to agree that each intermediate relay UE can regenerate the information in the notification message.

- Issue 2: content of the notification message

According to RAN2 agreements, the notification message may indicate the following state of different hops along the routing path:

- Between last relay UE and NW: cell reselection, handover, Uu RLF, Uu RRC connection establishment/resume failure

- Between an intermediate relay UE and NW: RRC connection release

- Between multi-hop U2N relay UE(s): PC5 RLF or PC5-S connection release

However, the RRC connect release may not need. The reason is that the RRC connection release is determined by the gNB. Before releasing an intermediate relay UE, the gNB can first perform the path switch to the remote UE, and then release the intermediate relay UE. Thus, compared to the legacy content in the notification message, the PC5-S connection release can be added.

Proposal 2-2: RAN2 is kindly asked to agree that the Notification message can be used to indicate the PC5-S connection release.

For each intermediate relay UE, two possible cases may trigger the transmission of the notification message:

- Case 1: receive notification message from the parent UE

- Case 2: its own link state change

Compared to Case 1, Case 2 may be more useful for the child UE/remote UE. The reason is that case 2 represents a real link problem of the intermediate relay UE; while case 1 does not reflect the link problem of the intermediate relay UE. Thus, it is beneficial to add the information of the real link problem in the notification message. Specifically, when receiving a notification message from parent UE, if the intermediate relay UE has a real link problem (e.g., PC5 RLF, PC5 RRC release), it can update the notification message by adding its own problem. With those additional information, the child UE/remote UE can be aware that how many link along the routing paths have the problem.

Proposal 2-3: RAN2 is kindly asked to agree that each intermediate relay UE can regenerate the notification message by adding its own link problem (e.g., PC5-RLF, PC5 connection Release), and the corresponding relay UE’s local ID.

- Issue 3: Notification message triggering

For connected state relay UE, the notification message transmission can be triggered either by receiving the notification message from its parent UE or by its own link state change (e.g., PC5 RLF, PC5 RRC Release). However, for the idle/inactive relay UE, if it receives the notification message from the parent UE, it can recover by itself first. For example, as shown in the following figure, Relay 3 triggers the relay reselection after PC5 RLF with Relay1. Since the reselected relay, i.e., relay2, is connecting to the same gNB/cell, Relay3 itself recovers the link for the remote UE, and it needn’t trigger the notification message to the remote UE.

Proposal 2-4: RAN2 is kindly asked to agree that the idle/inactive relay UE can trigger its own notification message when the following condition is satisfied:

- The reselected relay is served by a different cell from the one before PC5 RLF or PC5-S connection release

Fig.2 Fast recovery

Conclusion

In this contribution, we address some issues for the relay discovery and (re)selection, and have the following proposals:

Proposal 1-1: RAN2 is kindly asked to discuss how to determine whether the PC5 connection can be established with the discovered UE in multi-hop SL relay.

Proposal 1-2: RAN2 is kindly asked to discuss whether the upstream hop count information is indicated to its child UE.

Proposal 1-3: RAN2 is kindly asked to discuss whether the number of supported downstream hops is indicated to its child UE.

Proposal 2-1: RAN2 is kindly asked to agree that each intermediate relay UE can regenerate the information in the notification message.

Proposal 2-2: RAN2 is kindly asked to agree that the Notification message can be used to indicate the PC5-S connection release.

Proposal 2-3: RAN2 is kindly asked to agree that each intermediate relay UE can regenerate the notification message by adding its own link problem (e.g., PC5-RLF, PC5 connection release), and the corresponding relay UE’s local ID.

Proposal 2-4: RAN2 is kindly asked to agree that the idle/inactive relay UE can trigger its own notification message when the following condition is satisfied:

- The reselected relay is served by a different cell from the one before PC5 RLF or PC5-S connection release

References