3GPP TSG-RAN WG2 Meeting #130	R2-2504556

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                Qualcomm Incorporated

Title:	Open issues on LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 LP-WUS in DC/CA

In last RAN2 meeting, there is FFS on LP-WUS in dual DRX groups

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

For CA, from Rel-16, dual DRX groups has been supported. A UE can be configured with different values of drx-onDurationTimer and drx-InactivityTimer for one DRX group and for the other DRX group. Long DRX cycle is common for both DRX groups.

Dual DRX groups

Rel-16 DCP was enabled only on PCell or PSCell and indicates to start drx-onDurationTimer of all the DRX groups. In other words, it was not possible to trigger PDCCH monitoring of different DRX groups independently. This is not beneficial for UE power saving. For Rel-19 LP-WUS in connected mode.

In last RAN2 meeting, different views were expressed that whether one single LP-WUS configuration is applied for both DRX groups or separate LP-WUS configuration is applied for each DRX group. If one single LP-WUS configuration is applied for both DRX groups, the UE has to wake up main radios on both DRX group even though there may only DL data arrive on one DRX group, so that it is power wasteful for the UE to wake up the MR on the DRX group there is no DL data arrives.

Observation 1: If one single LP-WUS monitoring is applied for both DRX groups, it is power waste for the UE to wake up the MR on the DRX group in which there is no DL scheduling.

Observation 2: If separate LP-WUS monitoring is applied for each DRX group, UE only needs to wake up MR on the DRX group in which there is DL scheduling.

Then it is preferred that separate LP-WUS monitoring is applied for each DRX group.

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group. The LP-WUS configurations are independent for each DRX group. LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cells of the DRX group.

Considering UE may have different capabilities to support monitoring two LP-WUS signals for dual DRX group, then it is proposed to make it optional for the UE.

For option 1-2, lpwus_PDCCHMonitoringTimer should also be maintained per DRX group.

- Support maintenance of lpwus_PDCCHMonitoringTimer is per DRX group.

- Introduce UE capability to support separate LP-WUS monitoring on a serving cell per each DRX group.

## 2.2 Switching between LP-WUS and legacy PDCCH monitoring

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

Observation 3: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

Observation 4: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to gNB. And gNB should send switching command to the UE to enable LP-WUS. Then there is no misalignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 5: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, it can be FFS for other conditions, and it can be FFS on whether the entry/exist condition need to be configured to the UE or can be left UE implementation.

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

## 2.3 LP-WUS activation/deactivation

RAN1 decided to study activation/deactivation of LP-WUS monitoring as highlighted below in the RAN1#116 agreement.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

It could be possible that a network configures one or multiple LP-WUS configurations, and one of the LP-WUS configuration is activated by the MAC-CE. The MAC-CE can also deactivate LP-WUS monitoring (i.e., it activates none of the LP-WUS configurations), in which case the UE should switch to legacy C-DRX configuration.

In each of the LP-WUS configurations, various RRC parameters can be provided, then the MAC-CE activation can control the overhead/efficiency, associated beam, etc, in dynamic manner.

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

## 2.4 Paging monitoring in CONNECTED state

For paging, since the paging is cell specific and can be applicable for all RRC state. For CONNECTED state UE, it is possible to monitor paging DRX cycle for SI notification if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace. There are two options for paging DRX handling in CONNECTED state.

Option 1: UE in CONNECTED state monitors paging as legacy. That LP-WUS is not used for paging DRX in CONNECTED state.

Option 2: IDLE/Inactive LP-WUS can be used for the UE in CONNECTED state for paging monitoring. In this option, UE needs to monitor LP-WUS as in IDLE/Inactive state.

Option 2 has more power saving, but UE needs to support monitor two different LP-WUS signalling. For paging for SI update, gNB needs to send paging using both LP-WUS and legacy methods since there are LP-WUS UE and legacy UE in the network. Considering UE may have different capability to monitor two different LP-WUS signalling, then it should be left UE implementation to Proposal 9	It is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

- If the UE is in CONNECTED state, it is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring.

# Conclusion

LP-WUS in DC/CA

Observation 1: If one single LP-WUS monitoring is applied for both DRX groups, it is power waste for the UE to wake up the MR on the DRX group in which there is no DL scheduling.

Observation 2: If separate LP-WUS monitoring is applied for each DRX group, UE only needs to wake up MR on the DRX group in which there is DL scheduling.

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group. The LP-WUS configurations are independent for each DRX group. LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cells of the DRX group.

- Support maintenance of lpwus_PDCCHMonitoringTimer is per DRX group.

- Introduce UE capability to support separate LP-WUS monitoring on a serving cell per each DRX group.

Switching between LP-WUS and legacy PDCCH monitoring

Observation 3: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Observation 4: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Observation 5: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

LP-WUS activation/deactivation

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

Paging monitoring

- It is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)