3GPP TSG-RAN WG2 Meeting #130	R2-2504570

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.20.2

Source:	Nokia

Title:	UE usage of the RAT restrictions

WID/SID:	ECRATU - Release 19

Document for:	Discussion and Decision

# 1	Introduction

CT1 is approaching RAN2 on UE usage of RAT restrictions with LS C1-252546:

1. Overall Description:

CT1 thanks RAN2 for the reply LS on UE usage of the RAT restriction.

CT1 has discussed the question raised by RAN2 whether there is a need for the GERAN/UTRAN restrictions and reached consensus that these are required for the purpose of PLMN selection.

For the purpose of cell selection and reselection, CT1 believes the support of the GERAN/UTRAN restrictions is useful for GERAN/UTRAN network sunset and would also like to highlight that non-support of the GERAN/UTRAN restrictions can result in some implementations in an inferior user experience if the UE is wasting time with futile registration attempts on these RATs.

As an aside, CT1 would like RAN2 to note that CT1 has changed the terminology for the feature:

- RAT utilization control -> access technology utilization control

- list of "PLMNs with associated RAT restrictions" -> list of "PLMNs with associated access technology restrictions",

2. Actions:

To RAN WG2 group.

ACTION:

CT1 kindly asks RAN2 to take the above into consideration.

In this paper we analyse this LS and possible way forward in RAN2..

# 2	Discussion

As can be seen in the LS response from the CT1 they won’t limit usage to 4G/5G and they did not introduce separate capability for 4G/5G feature either.

So UE implementing ECRATU and supporting 2G/3G/4G/5G must implement also ECRATU for all the supported radio technologies before it can indicate NAS support for the feature

Observation 1: UE implementing ECRATU and supporting 2G/3G/4G/5G has to implement also ECRATU for all the supported radio technologies before it can indicate NAS support for the feature

One option to limit the impact is to have AS capability to differentiate ECRATU support for 2G/3G/4G/5G  but that does not make sense to have capabilities in both AS/NAS – Then NAS won’t be aware of the UE support and would falsely consider giving unsupported RAT technology restrictions

Observation 2: Adding separate capabilities in AS does not help as NW NAS would still consider UE supporting the feature completely

Another alternative is that NW just signals in NAS RAT restrictions and UE follows whatever it happens to support. This is not desired as then there is no way to verify which UEs support the feature or not

Observation 3: UE not implementing part of ECRATU (e.g. omitting 2G) does not work as there is no way for NW to know which UE will follow the requests (e.g., ECRATU message) and which UE cannot

So in our view in practice, we have following options:

- Implement ECRATU for 2G/3G by company contributions to plenary and then either

  - Only when UEs can test all the RATs they can indicate support for the feature. This is unfortunate as then most likely UE supporting 2G/3G will never indicate support for ECRATU making the feature pretty much useless

  - RAN2 indicates CT1 that please implement separate capability as otherwise we won’t likely ever have this feature deployed.

- Don’t implement CRs to 2G/3G—LS to CT1 and explain that due to IOT reasons it does not make sense to have CRs to 2G/3G as anyway it will very likely that no UE can ever test functionality for 2G/3G as for many UE vendors updating 2G/3G implementations is very difficult.

If 3GPP actually desires ECRATU to be deployed, then only valid options in our view is 1b. We should clarify that the capability should be purely for IOT reasons i.e. whenever there is testing possibility for 2G/3G then UE has to indicate support for corresponding capability if it indicates support for any ECRATU capability.

Proposal 1a: Work together with interested companies to bring CRs for 2G/3G ECRATU to plenary

Proposal 1b: Ask CT1 to implement separate capabilities purely for IOT reasons for ECRATU support for 2G and/or 3G and possibly also separate capabilities for 4G/5G

# 3	Conclusion

Observation 1: UE implementing ECRATU and supporting 2G/3G/4G/5G has to implement also ECRATU for all the supported radio technologies before it can indicate NAS support for the feature

Observation 2: Adding separate capabilities in AS does not help as NW NAS would still consider UE supporting the feature completely

Observation 3: UE not implementing part of ECRATU (e.g. omitting 2G) does not work as there is no way for NW to know which UE will follow the requests (e.g., ECRATU message) and which UE cannot

Proposal 1a: Work together with interested companies to bring CRs for 2G/3G ECRATU to plenary

Proposal 1b: Ask CT1 to implement separate capabilities purely for IOT reasons for ECRATU support for 2G and/or 3G and possibly also separate capabilities for 4G/5G