3GPP TSG-RAN WG2 Meeting #130	R2-2504571

St. Julians, Malta, May 19th-23rd, 2025

Agenda Item:	8.8.2

Source: 	CSCN, Huawei, HiSilicon, ZTE corporation, Sanechips, CATT

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

In RAN2#129b, some agreements of downlink coverage enhancement have been achieved as shown follow [1]:

Agreements; 1.From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits 2. We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring 3. RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we further discuss SMTC enhancements for idle/inactive and connected mode in NR NTN based on the agreements [1] and offline discussion [2] of RAN2#129b meeting.

Discussion

2.1 SMTC enhancements in RRC_IDLE/Inactive

RAN2 has agreed that network can provide assistance information regarding SMTC enhancements. In addition to UE implementation mentioned by some companies (e.g., choosing SMTC based on RSRP), two primary solutions are currently under consideration: SSB-based solution and the location-based solution.

Below we compared pros and cons of these 2 options.

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: SSB-based solution | Compatible with the existing UE beam selection process, which is easy to implement for UE. No restriction to UE behavior, and no impacts on beam selection. | - Provides no benefit for only one single SSB beam per cell. (2) In FR2 with 64 SSB indexes, may introduce signalling overhead. |
| Option 2: Location-based solution | For regular-shaped cells where the reference location is the cell center, the network implementation is straightforward. | (1) UE need to support GNSS and enable GNSS measurement in the idle mode, which increases the power consumption and complexity of UE. (2) When the cells coverage is irregular, it is hard to define the reference location. (3) For the earth moving cell, UE needs to calculate multiple moving reference locations. (4) Multiple reference location and distance threshold (48+16 bits for each) introduce large signalling overhead in both FR1and FR2. |

In terms of network deployment, both one single beam per cell and multiple beams per cell are possible and need to be considered. One single beam per cell can allocate limited active beams to more cells, thereby enabling simultaneous activation of more cells, so that network can employ the implementation method to configure no more than 4 SMTCs. As for multiple beams per cell, SSB-based SMTC configuration can applied appropriately for UE measurement.

Observation 1: There is no need for SMTC enhancement in the case of one single beam per cell. SSB-based SMTC configuration can only applied in the case of multiple beams per cell.

The benefit of SSB-based solution is that it can minimize the impact on the terminal, as the UE will detect the SSB and calculate the SSB index anyway, which is a procedure that will definitely take place at the UE, and how the UE selects SSB beam depends on the UE implementation as legacy. Once a SSB in the serving cell is selected, then the UE simultaneously determine a SMTC configuration corresponding to the SSB (i.e. the time domain occasion of SSB in the neighbour cell). While UE move to a new SSB beam, the UE can monitor another group of SMTC(s) corresponding to the new SSB UE select. How to select the SSB is depend on UE implementation, and UE behavior is the same as legacy.

Observation 2: Even the UE may at the edge of SSB or the overlapping area of SSBs, the UE can perform beam selection in a legacy way. Once a beam is selected, the associated SMTC of the serving cell SSB can be determined.

Companies may also concern about the relationship between SSB index and SMTC may change frequently. From our point of view, firstly, the network topology and connection relationship between serving cell and neighbour cell won’t change so frequently, NW deployment should avoid this unstable case. Otherwise, it will also lead to frequent changes of the neighboring cell, and the NW need to update the neighbour cell information frequently. In this case, relationship between SSB index/reference location and SMTC may both change frequently. Secondly, during the time of satellite covering (i.e. within t-Service), the relationships between serving cell and neighboring cells are predictable and stable. While taking into account earth moving cell, the neighbour cells are moving together with the serving cell to maintain a relatively stable mapping, and SSB beam is not required to fix to a specific geographical area on the ground, which means the NW can configure a consistent SMTC for the UEs within the serving cell.

Considering the singalling overhead of SSB-based solution, there are 2 possible methods of SMTC configurations:

a) a SSB list with multiple SSBs configured for each SMTC;

b) a SMTC list with multiple SMTCs for each SSB.

Even considering the maximum number of SSB index in FR2, additional 64 bits is required at most for SSB index indication in option a and additional 6 bits is required for SMTC indication. No matter which option is selected, the signalling overhead for SSB-based solution at least not more than location-based solution. In the FR1 scenario, the SSB-based solution is more efficient in signalling overhead.

Observation 3: For SSB-based SMTC configuration, a SSB list with multiple SSBs configured for each SMTC is more reasonable.

Proposal 1: RAN2 to introduce SSB-based solution for SMTC enhancement in RRC_Idle/Inactive.

2.2 SMTC/measure gap enhancements in RRC_CONNECTED

For intra-frequency and inter-frequency, the NW is aware of which beam the UE is utilizing for data transmission. This enables the NW to determine which neighboring cell is closer to the UE. By utilizing the beam management results, the 4 SMTCs in existing spec should be sufficient (i.e. no need to configure more than 4).

Observation 4: For intra-frequency and inter-frequency, by utilizing the beam management results, the 4 SMTCs in existing spec should be sufficient (i.e. no need to configure more than 4) for UE measurement in RRC_CONNECTED.

However, the configured 4 SMTCs may not be optimal from UE perspective. The situation of inter-frequency is more complicated. The implementation of SMTC and measurement gap in CONNECTED state not only disrupts data transmission continuity in the serving cell but also introduces scheduling restriction and complexity at the gNB. Therefore, ineffective SMTC and gap configuration should be avoided. There are two main scenarios: inter-frequency contiguous coverage (case 1) or overlapping coverage (case 2) shown in Figure 1. Beam footprints and coverage may show obvious variations in different frequency bands cells.

As shown in the Figure 1, if the gNB unaware of the UE's location, the majority of SMTC and gap parameters configured for the UE will become inaccurate, which will lead to unnecessary interruption of data transmission. More seriously, the number of gaps a UE needs to consider will more than 2, which contradicts the specification.

Figure 1 different UE with different detectable SSB

Observation 5: For inter-frequency, the inaccurate configuration of SMTC and gap becomes increasingly challenging in RRC_CONNECTED.

The Figure 2 below shown the typical triggered condition for Event D1 and Event D2: the green crescent-shaped section represents the area satisfying the entering condition, while the blue section represents the area satisfying the leaving condition.

Figure 2 the typical entering condition and leaving condition for event D1/D2

For event D1, the spec restricts the reference points with "The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to neighbour cell." meaning that referenceLocation1 can only be selected from serving cell-related points. For event D2, since both event D2 and location-based measurement initiation utilize movingReferenceLocation in SIB19, it is reasonable to configure this movingReferenceLocation within the serving cell rather than outside of it. For the case 1 (contiguous coverage) scenario, as shown in Figure 3, legacy event D1/D2 can be used by gNB to identify which neighboring cell that UE is closer to. For the case 2 (overlapping coverage) scenario, similar with eventA4, serving cell related configuration is unnecessary for UE to calculate for measuring the neighboring cell, legacy eventD1/D2 are no longer applicable, especially for load balancing

Observation 6: For the case 2 (overlapping coverage) scenario, similar with eventA4, serving cell related configuration is unnecessary for UE to calculate for measuring the neighboring cell, legacy eventD1/D2 are no longer applicable in some scenarios.

Figure 3 For case1, report the area of UEs by the legacy event D1

To avoid this issue, the gNB shall implement different configurations for UEs in different areas to reduce invalid configurations and the UE needs to send some information so that the gNB can know how to configure the SMTC and gap appropriately. There are two feasible methods in RRC_CONNECTED:

Option 1: Measurement report based solution

Based on the above analysis of the Event D1/D2, we present the following solutions for option1.

Option 1-1: event D1/D2 enhancement

In order to reduce the impact of the serving cell parameters during the measurement of neighbour cell, UE does not need to frequently calculate unnecessary values in some scenarios. The network can inform UE implicitly to invalidate the configuration of serving cell in both eventD1/D2 and condEventD1/D2.

According to the current specification, the minimum configuration value for distance threshold in eventD1/D2 is 1 (i.e. 50 meters), while the minimum value for condEventD1/D2 is 0.

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                            TimeToTrigger

}

eventD2-r18                                 SEQUENCE {

distanceThreshFromReference1-r18            INTEGER(1.. 65535),

distanceThreshFromReference2-r18            INTEGER(1.. 65535),

reportOnLeave-r18                           BOOLEAN,

hysteresisLocation-r18                      HysteresisLocation-r17,

timeToTrigger-r18                            TimeToTrigger

}

condEventD1-r17                            SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(0.. 65525),

distanceThreshFromReference2-r17            INTEGER(0.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                            TimeToTrigger

}

CondEventD2-r18                            SEQUENCE {

distanceThreshFromReference1-r18            INTEGER(0.. 65535),

distanceThreshFromReference2-r18            INTEGER(0.. 65535),

hysteresisLocation-r18                      HysteresisLocation-r17,

timeToTrigger-r18                            TimeToTrigger

}

So option 1-1 only modifies the field description of the event D1/D2 a with a clarification. For example, in the field descriptions of distanceThreshFromReference1, it is specified that “In NTN scenarios, when distanceThreshFromReference1 for eventD1/D2 is configured as 1, or when distanceThreshFromReference1 for condEventD1/D2 is configured as 0, the UE invalidate serving cell related configuration information and trigger MR reporting based on the neighbour cell’s configuration only.” This is equivalent to an implicit indication used to inform the UE.

Observation 7: The clarification of condEvent/eventD1/D2 in the specification of R19 has no impact on legacy UEs.

Under this condition, the area description triggering D1/D2 is as illustrated in the Figure 4:

Figure 4 Ignoring referenceLocation1 to define a circular entering condition

With this enhancement, the gNB can infer the UE located area (i.e. specific neighbor cell or beam coverage) via measurement reports indicating circular condition. The gNB can configure appropriate SMTC and gap for UE based on this measurement report.

Figure 5 For case2, report the area of UEs by the enhanced event D1/D2

Option 1-2: new event D3/D4

Define new Event/condEvent D3 and D4 aligns with Event A4 in TN, which utilize a single referenceLocation and distanceThreshFromReference to define a circular area. The benefit of new definition is more straightforward and clear and the design also aligns with legacy Event A specifications: Event D1/D2 aligns with Event A5 and Event D3/D4 aligns with Event A4.

Observation 8: New event D3/D4 is more straightforward and clear and the design also aligns with legacy Event A specifications: Event D1/D2 aligns with Event A5 and Event D3/D4 aligns with Event A4.

Option 2: UAI with multiple areas or locations

Configure multiple areas or referenceLocation points in OtherConfig. The gNB obtains information about the area in which the UE is located or identifies which reference location is closest via UEAssistanceInformation. The network configures "area list" to indicate each neighbour cell coverage or "reference location list" to describe the neighbour cell center. The UE determines the located area or top-N nearest reference location and reports the ID list to the network via UAI. The network performs refined configurations for SMTC and gap based on this information. If the UE’s current area changes from the last report, it triggers a subsequent report of new located arealist or top-N nearest reference locations change via UAI.

Figure 6: UE reporting multi area/reference location relationships via UAI

Below we summarize the pros and cons and applicability of different solutions:

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: Measurement report based solution | Option 1-1: event D1/D2 enhancement - Only minor change to IE descriptions (2) No impacts on legacy UEs due to the irrelevance of the 50m threshold in NTN. Option 1-2: new event D3/D4 The design is straightforward and aligns with event A specifications: Event D1/D2 aligns with Event A5; Event D3/D4 aligns with Event A4. | Option 1-1: event D1/D2 enhancement UEs located at the edge of multiple target areas may trigger multiple measurement reports (MRs), leading to excessive SMTC/gap reconfiguration. Option 1-2: new event D3/D4 This involves more modifications. (i.e. new definition of event D in NTN.) |
| Option 2: UAI with multiple locations or areas | Utilizing a single UAI reporting for multiple locations/areas can reduce signalling interactions. | Compared to Option 1: (1) Cannot associate with HO or CHO directly. (2) It is hard to associate location/area with frequency. |

Proposal 2: RAN2 to consider the following options for enhanced SMTCs and measurement gaps configurations for UEs in RRC_CONNECTED:

Option 1: Measurement report based solution (D1/D2 enhancement, new event D)

Option 2: UAI with multiple areas or locations.

Conclusions

Based on above analyses, observations and proposals are summarized as follows.

Observation 1: There is no need for SMTC enhancement in the case of one single beam per cell. SSB-based SMTC configuration can only applied in the case of multiple beams per cell.

Observation 2: Even the UE may at the edge of SSB or the overlapping area of SSBs, the UE can perform beam selection in a legacy way. Once a beam is selected, the associated SMTC of the serving cell SSB can be determined.

Observation 3: For SSB-based SMTC configuration, a SSB list with multiple SSBs configured for each SMTC is more reasonable.

Observation 4: For intra-frequency and inter-frequency, configuring 4 SMTCs by NW is sufficient for UE measurement in RRC_CONNECTED.

Observation 5: For inter-frequency, the inaccurate configuration of SMTC and gap becomes increasingly challenging in RRC_CONNECTED.

Observation 6: Legacy events D1/D2 are no longer applicable for inter-frequency overlapping coverage scenario.

Observation 7: The clarification of condEvent/eventD1/D2 in the specification of R19 has no impact on legacy UEs.

Observation 8: New event D3/D4 is more straightforward and clear and the design also aligns with legacy Event A specifications: Event D1/D2 aligns with Event A5 and Event D3/D4 aligns with Event A4.

Proposal 1: RAN2 to introduce SSB-based solution for SMTC enhancement in RRC_Idle/Inactive.

Proposal 2: RAN2 to consider the following options for enhanced SMTCs and measurement gaps configurations for UEs in RRC_CONNECTED:

Option 1: Measurement report based solution (D1/D2 enhancement, new event D)

Option 2: UAI with multiple areas or locations.

Reference

- Report of 3GPP TSG RAN WG2 meeting #129b

- R2-2503055, Report of [AT129bis][303][R19 NR NTN] SMTC enhancements, Xiaomi