3GPP TSG RAN WG2#130 	R2-2504577                                                                            St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.2.4

Source:	III

Title:	Discussion on A-IoT data segmentation and transmission

Document for:	Discussion

Introduction

In the previous RAN2#129-bis meeting, some related agreements were achieved in the following:

Agreements < Validity of AS ID > - AS ID is applied for Inventory + command case; - AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed. - For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. - Do not specify the reader behaviour on how exactly the ASID is generated. - The device releases the AS ID upon power off (no stage 3 specification impact); - The device only keeps one AS ID at a time. - For CFRA, command message is used for AS ID assignment - For CBRA, Msg 2 is used for AS ID assignment - The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) FFS other cases for release ASID to avoid keeping it indefinitely. < Segmentation > 1. For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. 2 FFS whether the reader always includes the command for retransmission of segments. 3 1-bit indication is sufficient to indicate whether more D2R data will be sent 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption.

Based on above RAN2#129-bis agreement, this contribution discusses AS ID for CFRA and segmentation transmission/retransmission.

Discussion

AS ID for CFRA

The discussion about AS ID for CFRA had reached agreements in RAN2#129.

Agreements: For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: • Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. • Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 • Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

In Rel-19, only one device is paged by one paging message for CFRA. Option 3 will introduce extra new MSG steps between MSG1 and MSG2, these extra new MSGs will cause more power consumption in device. In order to be consistent with CBRA, option 2 / option 4 is preferred.

Observation 1: For CFRA case, option 2 / option 4 is preferred.

Segmentation

In the MSG3 transmission of A-IoT random access procedure, to support data transmission under limited resource conditions, the A-IoT MAC layer has studied the segmentation mechanism to segment data blocks. Each segmentation contains corresponding control information to ensure that the reader can correctly assemble all segments to complete MSG3.

Two segmentation-based transmission processes are proposed:

- One reader’s response after all segments transmitted: The device continuously transmits all segments data without waiting the response for each segment. Until all segments were transmitted, the device wait for the response from the reader. The response will indicate which segments should be re-transmitted.

- One reader’s response after each segment transmit: The device transmits each segment data and waiting the response for this segment. The response will indicate this segment how to re-transmit.

Proposal 1: Two segmentation-based transmission processes are proposed:

- Unified response after full segment transmission,

- Send instant responses segment by segment.

- Conclusion

We have the following proposals:

Observation 1: For CFRA case, option 2 / option 4 is sufficient.

Proposal 1: Two segmentation-based transmission processes are proposed:

- Unified response after full segment transmission,

- Send instant responses segment by segment.