3GPP TSG-RAN WG2 Meeting #130	R2-2504596

# St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.7.4.1

Source:	Ericsson, InterDigital, LG Electronics, Fujitsu

Title:	Discussion on Bj enhancement

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has been working on scheduling enhancements to the LCP mechanism according to the objective in the XR WID [1] for Rel-19:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In latest meeting (RAN2 #129bis) RAN2 discussed enhancement to the Bj part in the LCP mechanism. It was agreed that unless gains are proven and there is common understanding these enhancements will not be considered.

- Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

- It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

In this contribution we provide simulation results investigating the impacts of doing Bj enhancement and also provide a TP how this enhancement can be implemented in specification.

# 2	Discussion of LCP and Bj enhancement

Enhancement to the Bj handling in LCP for delay critical data was proposed in previous meetings. As input to these discussions reasons for doing the Bj enhancement and details on how it can work was provided. An example of such was provided in R2-2502934 with diagrams outlining various multi-flow scenarios showing the impacts on the delay with different LCP behaviours. In short, the idea presented is that during priority adjusted state all the time critical data (LCH priority adjusted data) should be selected in the resource allocation procedure before moving to the next LCH in the list. The intention with the enhancement is that the highest prioritized time critical data will be transmitted without any extra delay. Counter arguments has been raised that situations where this enhancement is needed will occur seldomly and that current LCP mechanism offer enough configurations possibilities to make further LCP enhancements unnecessary. However, proponents’ views have been that these situations can indeed occur often, as it has been shown in earlier simulation campaigns during Rel17 and Rel18 XR Study Items that a few XR users can use up all the radio resources in the system, and that current LCP parameters will not be enough to limit the delays, which then necessitates the Bj enhancement to have a properly working solution. Thus, to understand the impact and need of Bj enhancements a new simulation campaign has been undertaken to investigate both the current options with the legacy LCP mechanisms and impact of the new Rel19 LCP enhancements including the Bj enhancement option.

## 2.1 Simulations of LCP enhancement

Simulations has been performed with two uplink traffic flows mapped to different LCH. One traffic flow consists of AR video (XR type of traffic) while the other traffic flow consists of Object file uploads (eMBB type of traffic). The AR video has a clearly defined periodicity (60 fps) while the Object uploads varies both in size and inter arrival time based on exponential distributions (1 second mean value of inter arrival time). Simulations are focused on users that experience a data rate of around 2-3 Mbps. AR video is rate adaptive and will adjust to a suitable bit rate for the available data rate.

Simulation results are presented in three steps. First solution evaluated is the legacy LCP mechanism, then the delay enhanced LCP without Bj enhancement and last is the delay enhanced LCP with Bj enhancement.

### 2.1.1 Legacy LCP performance

In this section the legacy LCP is simulated without any enhancements. Different settings on the LCP parameters Prioritized Bit Rate (PBR), Bucket Size Duration (BSD) and LCH priority are compared. First results of these evaluations are shown in Figure 1 where only the PBR and priority is varied, while BSD is fixed at 50ms. In the left plot the 90th percentile delay of the AR video is shown while in the right plot the mean delay of the object file uploads is presented. Iteration parameter is the average object size. Settings on PBR is varied from 64kbps up to 4096kbps. Either AR or Object has higher priority (indicated with “Priority” in the legend). Mean object size is iterated from 10000 to 20000 bytes. BSD is kept fixed at 50ms in all of the configurations in these simulations.

Figure 1: Legacy LCP mechanism with various priority and PBR settings. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays.

It is clear from the figures that when utilizing the legacy LCP either the AR video delays will be low or alternatively the Object delays, but neither both delays at the same time. The settings that keep AR video low and stable (priority of AR traffic with high PBR) will instead create very large Object delays. Other settings that bring down the Object delays will create large AR delays that is also unstable and rapidly increase with the increased object load.

There is one more parameter setting in the LCP to evaluate and that is the BSD. Figure 2 shows comparisons when setting the BSD to a lower value (either 10ms or 5ms). With high PBR this setting doesn’t have any impact but when PBR is low on both LCH then lowering the BSD have some effect. When both PBR and BSD are set to low values there is a reduction in the AR delays. However, the AR delays are still increasing with increased object load and the object delays are larger.

- All configurations of legacy LCP that provide low and stable XR video delays significantly increase the Object delays.

Figure 2: Legacy LCP mechanism with adjusted BSD settings. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays. BSD is set to either 5ms or 10ms. AR has higher priority.

### 2.1.2 Delay enhanced LCP without Bj enhancement

In this section Rel19 LCP enhancements is simulated. Only the behaviour of adjusting the priority of LCH with delay critical data is evaluated, i.e. traffic mapped to this LCH can be delay prioritized. For simplicity the delay threshold is implemented so that when data has spent more time in buffer than the threshold (time spent in buffer exceeds threshold value) the priority is adjusted.

It should be noted that adjusting the priority of a LCH that already has higher priority doesn’t have any impact on performance, e.g. with AR traffic having higher default priority adding a delay threshold will not provide any gains. To have an impact either AR traffic has higher default priority and the Object traffic is delay prioritised or the Object traffic has a higher default priority and the AR traffic is delay prioritized. Figure 3 shows a few comparisons of such different priority configurations together with delay thresholds (priority is adjusted when delay of data exceeds this threshold). The configuration when AR has high priority and Object is delay prioritized, enables to keep the AR delays stable but the reduction in Object delay is limited. Larger delay threshold gives better AR performance but Object delays gets worse. The opposite configuration performs better, here the delays on the AR can be kept low and stable but also a large reduction in Object delay is seen. However it is necessary that the PBR value on the AR traffic is set high, to make the AR flow take all the resources when it gets delay prioritized. An example with lower PBR setting on the AR traffic shows that the AR delays increase significantly. Thus the two possible configurations with delay enhanced LCP without Bj enhancement is to either set AR PBR high with a delay threshold and lower priority than the other traffic or set AR priority high with a delay threshold on other traffic. It can be noticed that this LCP solution limits the possible configurations.

- LCP enhancements that only adjust the priority requires to set the delay threshold on the less time critical traffic and limit the possible configurations.

Figure 3: Delay enhanced LCP without Bj enhancement. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays. Several different delay thresholds (in legend “Delay XXms”) and priority configurations are evaluated.

### 2.1.3 Delay enhanced LCP with Bj enhancement

In this section the Bj enhancement to the delay enhanced LCP is simulated. The enhancement is implemented so that whenever there is data exceeding the delay threshold in an LCH, there is no Bj limitation on how much data that can be selected from this LCH during the resource allocation procedure.

Figure 4 show the impact of this enhancement in various configurations of PBR and delay thresholds. It is apparent that the Bj enhancement enables multiple configurations that manages to keep the AR delays low and stable. Furthermore a large reduction in Object delay is achievable similar to the best performing of the other solutions that manages to keep the AR delays low.

Figure 4: Delay enhanced LCP with Bj enhancement. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays.

The effects can be more clearly seen if the Object load if further increased by iterating to higher average object sizes. This is shown in Figure 5 with a selection of the best performing configurations from each LCP solution. At higher load larger differences between the configurations can be seen. The solutions that minimise the AR delays can also perform the best on Object delays.

- Bj enhancements enable multiple configurations to keep AR delays low and stable while also provide large reduction in Object delays.

Figure 5: Selected configurations from all variants of LCP solutions evaluated to higher Object load. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays.

To verify that the LCP solutions doesn’t interfere with the rate adaption of the AR video, i.e. that the traffic mix is kept the same in all scenarios, a check of the AR video rate can be performed. Figure 6 shows the mean average rate of the AR video and it is seen that similar rates are achieved in all the LCP solutions.

Figure 6: AR video rate for selected LCP solutions.

As a last check, evaluations at higher user rates can be done. This is shown in Figure 7, where user bit rates of around 5 Mbps is targeted. The same scenarios as in Figure 5 are used. What can be seen is that in these higher rates all solutions are able to achieve low AR delays. However, the Bj enhancement solution achieves much lower Object delays. Thus, it can be concluded that the same LCP parameter configuration with Bj enhancement can perform well over varying user bit rates.

- Bj enhancements allow a single LCP configuration to perform well over varying user bit rates.

Figure 7: Selected configurations from all variants of LCP solutions evaluated at higher user bit rates. Left plot shows the 90th percentile of AR delays, right plot shows the mean of Object delays.

### 2.1.4 Conclusions of evaluations

Legacy LCP mechanism has a problem to support multiple flows and keep XR traffic (AR video) delays low when load from other traffic increases, without starving the other traffic (e.g. eMBB) and thus impose huge delays on that traffic.

Delay enhanced LCP without Bj enhancement can only keep the AR delays low if the AR PBR is set to a high value together with a delay threshold or if the delay threshold is configured on the less time critical traffic.

Bj enhanced LCP can keep the AR delays low and stable but also provide the high reduction in Object delays with multiple possible PBR configurations.

Bj enhancement shall be incorporated in the Rel19 LCP enhancements to achieve the most flexible and high performing LCP solution.

## 2.2 Bj enhancement in specification

The simplest solution to implement the Bj enhancement is to add a new condition in the resource allocation procedure so that logical channels which have applied additionalPriority are no longer limited by the Bj > 0 condition. A TP for this is provided in Appendix 1.

Implement the TP provided in Appendix 1.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	All configurations of legacy LCP that provide low and stable XR video delays significantly increase the Object delays.

Observation 2	LCP enhancements that only adjust the priority requires to set the delay threshold on the less time critical traffic and limit the possible configurations.

Observation 3	Bj enhancements enable multiple configurations to keep AR delays low and stable while also provide large reduction in Object delays.

Observation 4	Bj enhancements allow a single LCP configuration to perform well over varying user bit rates.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Bj enhancement shall be incorporated in the Rel19 LCP enhancements to achieve the most flexible and high performing LCP solution.

Proposal 2	Implement the TP provided in Appendix 1.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024

# Appendix 1

Changes to specification TS 38.321 in latest agreed version R2-2501761, additions highlighted in yellow:

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	additionalPriority which is applied instead of priority for the LCP procedure when the conditions specified in clause 5.4.3.1.3 are met;

-	priorityAdjustmentThreshold which is used to determine whether additionalPriority or priority is used for the LCP procedure;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2> for a logical channel configured with priorityAdjustmentThreshold, if the smallest remaining value of the running PDCP discardTimers among its data available for this new transmission, evaluated at the time of the first symbol of this new transmission, is below its priorityAdjustmentThreshold:

3>  apply additionalPriority of this logical channel;

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or logical channels with additionalPriority applied are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

Editor’s Note:	FFS whether priority-adjustable data in an LCH configured with additionalPriority can be transmitted even if Bj is negative.

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain:

3> if a logical channel has applied additionalPriority above; and

3> if none of its remaining data available for this new transmission has a running PDCP discardTimer whose remaining value, evaluated at the time of the first symbol of this new transmission, is below its priorityAdjustmentThreshold; and

3> if the UE supports lch-PriorityAdjustmentRound2:

4> apply priority of this logical channel;

Editor’s Note: 	FFS whether network configuration instead of UE capability should be used here. The name of the UE capability, lch-PriorityAdjustmentRound2, is tentative and can be updated after the 38.306 running CR is agreed.

3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. For this allocation of resources, logical channels with equal priority, either configured or applied, should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query, or MAC CE for UL rate query;

Editor’s Note:	FFS whether a new rate control MAC CE will be defined. If so, the rapporteur assumes it has the same priority as the legacy Recommended Bit Rate Query MAC CE.

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.