3GPP TSG-RAN2#130	R2-2504599

St. Julian, Malta, May 19 – May 23, 2025

Agenda item:		8.7.6

Source:		Lenovo

Title:		 XR Rate Control

Document for:		Discussion and Decision

- Introduction

For XR traffic, it would be beneficial to enable the application to adapt the codec rate based on the network conditions to avoid dropping of packets over the air interface. Such discussion has been ongoing in the RAN Plenary with the following being agreed as the way forward in the last meeting, RAN #105 [1].

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Additionally, the following agreements were made at the RAN2 #128and RAN2 #129 meetings, as follows:

Agreements from RAN2 #128 1. RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. 2. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN. 3. Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 4. RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. 5. RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE. 6. FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful. Agreements from RAN2 #129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed. - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this Agreements from RAN2 #129bis - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

This contribution looks at how the XR rate control information may be signaled in the downlink to support source rate adaption to uplink congestion.

- Discussion

For XR applications that have very strict requirements in terms of data rate and latencies, it is beneficial for the application to be able to adapt the source rates based on the congestion and network conditions on the uplink. During the CR reviews, some open issues have been identified regarding the design and specification of XR Rate Control procedure.

During the RAN2#129bis meeting, RAN2 had agreed that UL rate indication will be on a per QoS flow basis unless any issues are identified by RAN3. And it was FFS how this flow ID is indicated in the MAC CE. In our view, it is sufficient to go with the DRB ID and the QFI ID to be able to differentiate the different flows across PDU sessions. Furthermore, since the DRB ID is already indicated in the legacy RBR MAC CE, this is a simpler approach.

(MAC-04) The UL rate recommendation on a per QoS flow is indicated using the DRB ID and QFI ID.

Another open issue that remained was whether the new MAC CE can include the bit rate recommendation for only a single QoS flow or if it can include the bitrate recommendation for multiple QoS flows. Since a single DRB may consist of multiple QoS flows, it may be infeasible to include only the recommendation for a single QoS flow in the MAC CE since the gNB would then need to send multiple MAC CEs for a single DRB if more than one QoS flow can be rate adapted. This would lead to a lot of signalling overhead which should be avoided. Additionally, to make the MAC CE future proof, the gNB should be able to include the bitrate recommendation for multiple QoS flows within a single MAC CE.

(MAC-05) The Rate Control MAC CE can include multiple QoS flows.

In the last meeting, it was also agreed that only a single table will be designed for the bitrate recommendation with 8 bits, exponential distribution and no multiplier. Accordingly, RAN2 also needs to discuss the format of the Rate Control MAC CE. In our view, the MAC CE needs to at least consist of the following fields:

- DRB ID

- QoS Flow ID (QFI)

- Bit Rate Recommendation

Additionally, we think that a multiplier field is not needed, i.e., we do not use the legacy multiplier values in the new MAC CE, since the new table will be specified without a multiplier and intended for the range of data rates provided by SA4. The number of bits to indicate QFI and DRB ID should be at least 6 and 5 respectively.

(MAC-06) The Rate Control MAC CE consists of the following fields: DRB ID, QoS Flow ID (QFI), Bit Rate Recommendation.

- The number of bits for DRB ID shall be at least 5 bits.

- The number of bits for QFI shall be at least 6 bits.

- The number of bits for Bit Rate Recommendation shall be 8 bits.

(MAC-07) The Rate Control MAC CE does not include a bit rate multiplier field.

For UL rate querying, it was already agreed that a UE can be configured with a rate query MAC CE. It was also further discussed during the CR review that the triggering of the rate query is controlled by a prohibit timer similar to legacy procedure. Hence, we think that the design of UL rate query MAC CE should follow the same principle as the legacy RBR query MAC CE. That is, the UE need not be specified with any additional transmission or cancellation procedures for the rate query MAC CE and can be specified within the section for Recommended Bit Rate (5.18.10). For multiplexing, we think that the UL Rate Control Query MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE.

(MAC-08) The Rate Control Query MAC CE is specified with the same principles as legacy RBR Query MAC CE, i.e., within section 5.18.10.

(MAC-08) The Rate Control MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE.

Lastly, for the configuration of XR Rate Control with Dual Connectivity, it needs to be discussed how the UE should handle the reception of a Rate Control MAC CE from MCG and SCG. RAN2 had a similar discussion for PSI-based discard procedure, and it was concluded that the UE follows the latest received indication. We think that same behaviour can be reused for this case as well. Hence, we think that the UE should apply the latest received bitrate recommendation.

(MAC-09) UE applies the latest received UL bitrate recommendation for a QoS flow if UE can receive Rate Control MAC CE from both MCG and SCG for split DRB.

- Conclusion

- Proposal 1: (MAC-04) The UL rate recommendation on a per QoS flow is indicated using the DRB ID and QFI ID.

- Proposal 2: (MAC-05) The Rate Control MAC CE can include multiple QoS flows.

Proposal 3: (MAC-06) The Rate Control MAC CE consists of the following fields: DRB ID, QoS Flow ID (QFI), Bit Rate Recommendation.

-	The number of bits for DRB ID shall be at least 5 bits.

-	The number of bits for QFI shall be at least 6 bits.

-	The number of bits for Bit Rate Recommendation shall be 8 bits.

- Proposal 4: (MAC-07) The Rate Control MAC CE does not include a bit rate multiplier field.

Proposal 5: (MAC-08) The Rate Control Query MAC CE is specified with the same principles as legacy RBR Query MAC CE, i.e., within section 5.18.10.

Proposal 6: (MAC-08) The Rate Control MAC CE can be multiplexed with the same priority as the legacy RBR Query MAC CE.

Proposal 7:  (MAC-09) UE applies the latest received UL bitrate recommendation for a QoS flow if UE can receive Rate Control MAC CE from both MCG and SCG for split DRB.

- References

[1] RP-242390: Way Forward on WID Scope Revision, Nokia

[2] R3-250805, Reply LS on XR UL Rate Control, RAN3