3GPP TSG-RAN WG2 #130	R2-2504613

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.10.2

Source:	Ericsson

Title:	Further considerations on MRO for LTM and CHO with candidate SCG

Document for:	Discussion/Decision

# Introduction

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

•	Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

•	Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

This contribution discusses considerations regarding SON enhancements for LTM and CHO with candidate SCGs.

# Discussion

## SON for LTM

Per the agreements from previous meetings, we will focus on MCG LTM, and discuss the enhancement for the RLF report, the SHR and the RA report for LTM optimization. Considering that the LTM cell switch procedure highly relies on underlying features such as early DL/UL synchronization and a RACH-less access procedure, we will also discuss the possible MRO enhancements from those aspects.

### Enhancement of the RLF report

Logging the correct TA value for UE-based TA estimation

A well-functioning early UL sync procedure is important for well-functioning LTM, and in the context of SON, this makes it natural to design SON mechanisms for optimization of the early UL sync procedure. In the previous RAN2 and RAN3 meetings, early UL sync related MRO enhancement has been discussed.

When connection failure happens due to wrong TA estimated by PDCCH ordered early UL sync, both the NW and the UE has the related information of the TA estimation. Therefore, this case can be left to RAN3 to decide whether to adopt a network based or a UE based solution.

However, if the TA applied for the failed LTM cell switch is estimated by the UE, the NW is not able to figure out if this failed LTM cell switch is due to wrong TA estimation or some other reasons without knowing whether the TA estimated by the UE is accurate enough for the following LTM cell switch execution or not.

RAN2 to consider the case of LTM cell switch failure due to outdated/wrong TA calculated by UE.

To this end, it has been proposed that the TA value in the RAR during a random-access procedure in conjunction with a reestablishment procedure or LTM recovery procedure following a failed LTM cell switch can be reported by the UE or be remembered by the NW. Then the TA value from the RAR can be used for all LTM cell switches towards the target cell for later LTM cell switch. Moreover, this TA value from the RAR can be used by the NW to check if it is better to keep the UE based TA estimation method for this LTM candidate cell or not.

The UE logs the TA received from RAR in the target cell in the RLF report.

### Enhancement of the SHR

In RAN2#127bis meeting, the following was agreed:

UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

According to the agreement above, L1 measurements are logged in SHR only when triggered by a successful LTM cell switch. However, in scenarios where a UE is configured with a list of LTM candidate cells and an L3 handover occurs, triggering an SHR, the L1 measurements are not logged or reported. Consequently, the gNB may be unable to determine why the LTM cell switch was not triggered. Therefore, it would be advantageous to log the L1 measurements in SHR regardless of whether the SHR is triggered by a successful LTM cell switch or an L3 handover.

UE logs available L1 measurement results in the SHR for the serving cell, the target cell and other LTM candidate cells when the UE is LTM configured.

## SON features for CHO with candidate SCGs

### Time information when execution conditions are not fulfilled

In RAN2#127-bis meeting, the following agreements were taken-

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

- Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

However, if none of the execution condition are fulfilled and UE declares RLF, it remains unclear what UE should log in the RLF report. In legacy, UE uses timeSinceCHO-Reconfig IE to log time information related to the reception of the CHO configuration and RLF.

2> if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure: 3> if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover; 3> else: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig; 3> set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResultNeighCells;

We first note that the timeSinceCHO-Reconfig will be logged also for the case of CHO with candidate SCGs because, as it is specified currently, this field is “set to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message”.

- No matter if the HO is a pure Rel.16 CHO or the Rel.18 CHO with candidate SCG, the UE will always log the timeSinceCHO-Reconfiguration as the time gap since receiving the condRRCReconfig, according to the existing specification.

Hence, to avoid ambiguity and confusion with the legacy timeSinceCHO-Reconfig, and erroneous interpretation at the gNB, it should be introduced a similar field just capturing the time gap between the CHO with candidate SCG and the RLF. Otherwise, the gNB would interpret this as the time gap since the last CHO configuration, whereas in fact this should be the gap between the last CHO with candidate SCG and RLF.

UE reports the time gap between the reception of the last configuration of the CHO with candidate SCG configuration and the RLF/HOF.

### Inclusion of target PSCell information in SHR

In RAN2#129bis meeting, the following were agreed:

• UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

In the current running CR solution, UE includes the list of target PCell and PSCell pairs that were configured as CHO with candidate SCG configuration within the ChoWithCandidateSCGInfo IE. However, it was mentioned that if the UE has two configurations where the PCell is same but PSCell is different, from the current logged information it is not possible to derive which is the target PSCell ID that was eventually executed for the HO. This is because the legacy SHR only contains information related to the source PCell and target PCell, nothing about the target PSCell. Hence in this scenario, by just looking at the legacy SHR and at the new ChoWithCandidateSCGInfoI IE, it will not be possible for the network to understand which pair target PCell – target PSCell was eventually executed by the UE for the HO.

In below, we discuss multiple alternative solutions to the problem-

Solution-1: Add the target PSCell information, i.e., PSCell ID and measurements along with the measurement results, in the SHR.

Solution-2: Include a flag in the ChoWithCandidateSCG indicating the target PCell and PSCell pair for which the UE executed the CHO with candidate SCG configuration.

In our view, the first solution 1 is simple, and it can be adopted.

RAN2 to agree on the following alternatives to include target PSCell ID in the SHR:

  - Explicit inclusion of the target PSCell ID in the SHR.

  - Include a flag in the ChoWithCandidateSCG IE to indicate the target PCell and PSCell pair to which UE executed the CHO with candidate SCG procedure.

# Conclusion

In the previous sections we made the following observations:

Observation 1	No matter if the HO is a pure Rel.16 CHO or the Rel.18 CHO with candidate SCG, the UE will always log the timeSinceCHO-Reconfiguration as the time gap since receiving the condRRCReconfig, according to the existing specification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to consider the case of LTM cell switch failure due to outdated/wrong TA calculated by UE.

Proposal 2	The UE logs the TA received from RAR in the target cell in the RLF report.

Proposal 3	UE logs available L1 measurement results in the SHR for the serving cell, the target cell and other LTM candidate cells when the UE is LTM configured.

Proposal 4	UE reports the time gap between the reception of the last configuration of the CHO with candidate SCG configuration and the RLF/HOF.

Proposal 5	RAN2 to agree on the following alternatives to include target PSCell ID in the SHR:

a.	Explicit inclusion of the target PSCell ID in the SHR.

b.	Include a flag in the ChoWithCandidateSCG IE to indicate the target PCell and PSCell pair to which UE executed the CHO with candidate SCG procedure.