3GPP TSG-RAN WG2 Meeting #130	R2-2504624

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:        ZTE Corporation

Title:           Clarification on the maxMIMO-LayersForMulti-DCI-mTRP-r16

Agenda item:   5.1.3.2

Document for:  Discussion and Decision

- Introduction

The maxMIMO-LayersForMulti-DCI-mTRP-r16 was introduced for the mTRP case.

| maxMIMO-LayersForMulti-DCI-mTRP-r16 Indicates the interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP. If this field is included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. NOTE 1: For data rate calculation in clause 4.1.2, if this feature is indicated, each multi-DCI based multi-TRP CC is counted two times toward J. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

With this capability, there are 3 cases for the mDCI mTRP:

- Case 1: This capability is present, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH

- Case 2: This capability is absent and if having at least one RE overlapped, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs

- Case 3: This capability is absent and no RE overlapped, not specified yet.

In this contribution, we would share our views on the Case 3.

- Discussion

In this chapter, we first give a short description on RAN1’s discussion history about maxMIMO-LayersForMulti-DCI-mTRP-r16, then share our views on the case 3.

According to the RAN1 chairman note [1], this capability was introduced in [2] based on the email discussion [3]. In the summary part of the email discussion “ [105-e-NR-UEFeatures-eMIMO-01]”, it said “After further discussions during RAN1 #105-e on the RAN1 email reflector, the following was agreed”

After further discussions during RAN1 #105-e on the c, the following was agreed: Agreement: - Introduce the following new FG 16-2a-9; Interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP; For multi-DCI multi-TRP operation, if this FG is indicated, “maxNumberMIMO-LayersPDSCH” is interpreted as the maximum number of layers per PDSCH.; 16-2a-0; Yes; N/A; Per band; No; No; Note1: For multi-DCI multi-TRP operation, if this FG is not indicated, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped. Note2: For data rate calculation in Section 4.1.2 of 38.306, if this FG is indicated, each multi-DCI based multi-TRP CC is counted two times toward J.; Optional with capability signalling.

Thus we check the RAN1 email reflector on this topic with the link below and find a summary table：

https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG1;e12af580.2105D_2&S=

| Capability bit | maxNumberMIMO-LayersPDSCH | single TRP case | 2 TRPs configured, one TRP transmitting | 2 TRPs , no PDSCH overlap | 2 TRPs , at least one RE overlap |
| --- | --- | --- | --- | --- | --- |
| not supported | 4 | 4 | 4 | 4+4 | 2+2, 3+1, 1+3 |
| supported | 4 | 4 | 4 | 4+4 | 4+4 |

Note: we delete the red question part from the original table (for that it has been finally clarified)

Furthermore, in the RAN1 reflector, the most controversial part is for the case 1 and case 2, i.e.

- Case 1: This capability is present, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH (as highlighted in green)”

- Case 2: This capability is absent and if having at least one RE overlapped, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs (as highlighted in blue)

For the case 3, it seems that RAN1 companies share the same understanding that “maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH (as highlighted in yellow above)”. However this case was not included in the RAN1 feature list table and thus not included in the RAN2’s field description. From the network configuration aspect, network needs to know how many layers can be configured for this case. Thus, it’s better to have clear clarification on this case in the field description to make sure that the NW vendors and UE vendors share the same understanding.

Proposal 1: RAN2 to clarify that for the mDCI mTRP:

If maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH if no RE overlapped.

A TP was also provided in the Annex.

Proposal 1a: If the proposal 1 was confirmed, agree the TP in the Annex.

From RAN2 aspect, the maxNumberMIMO-LayersPDSCH was an input of the maximum data rate calculation, thus it’s better to have a clear understanding on the maxNumberMIMO-LayersPDSCH for each case.

### 4.1.2 Supported max data rate for DL/UL For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by maxNumberMIMO-LayersPDSCH for downlink and maximum of maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink.

Based on the proposal 1, we need to further discuss how to calculate the maximum data rate for this case. For the case 1 (i.e. the maxMIMO-LayersForMulti-DCI-mTRP-r16 is present), there is a Note as below for the maximum data rate calculation.

| maxMIMO-LayersForMulti-DCI-mTRP-r16 Indicates the interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP. If this field is included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. NOTE 1: For data rate calculation in clause 4.1.2, if this feature is indicated, each multi-DCI based multi-TRP CC is counted two times toward J. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

However for the above case 3, even the maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH, it happened only for the case without RE overlapping, thus this NOTE1 should not be used for the case 3, i.e. for the case 3, each multi-DCI based multi-TRP CC is counted 1 time toward J.

Proposal 2: RAN2 to confirm that for the case 3 (maxMIMO-LayersForMulti-DCI-mTRP-r16 is absent and no RE overlappedHo), each multi-DCI based multi-TRP CC is counted 1 time toward J and thus no spec change is needed for the maximum data rate calculation.

However, if companies can’t achieve the consensus based the above RAN1 discussion history, an LS can be sent to RAN1 to confirm the above proposal 1 and proposal 2.

Proposal 3: If companies can’t achieve the consensus, an LS can be sent to RAN1 to confirm the above proposal 1 and proposal 2.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to clarify that for the mDCI mTRP:

If maxMIMO-LayersForMulti-DCI-mTRP-r16 is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH if no RE overlapped.

Proposal 1a: If the proposal 1 was confirmed, agree the TP in the Annex.

Proposal 2: RAN2 to confirm that for the case 3 (maxMIMO-LayersForMulti-DCI-mTRP-r16 is absent and no RE overlappedHo), each multi-DCI based multi-TRP CC is counted 1 time toward J and thus no spec change is needed for the maximum data rate calculation.

Proposal 3: If companies can’t achieve the consensus, an LS can be sent to RAN1 to confirm the above proposal 1 and proposal 2.

- Reference

[1] R1-2106402 Final Report of 3GPP TSG RAN WG1 #105-e v1.0.0 (Online meeting, 10th - 27th May 2021)

[2] R1-2106160	Updated RAN1 UE features list for Rel-16 NR after RAN1#105-e	Moderators (AT&T, NTT DOCOMO, INC.)

[3] R1-2106164	Summary of email discussion/approval [105-e-NR-UEFeatures-eMIMO-01]	Moderator (AT&T)

- Annex

TP for the 38.306

| maxMIMO-LayersForMulti-DCI-mTRP-r16 Indicates the interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP. If this field is included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped, for multi-DCI multi-TRP operation. If this field is not included, maxNumberMIMO-LayersPDSCH is interpreted as the maximum number of layers per PDSCH if no RE overlapped, for multi-DCI multi-TRP operation. The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16. NOTE 1: For data rate calculation in clause 4.1.2, if this feature is indicated, each multi-DCI based multi-TRP CC is counted two times toward J. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |