3GPP TSG-RAN WG2 Meeting # 130	R2-2504646

# St. Julians, Malta, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Introduction of IoT NTN phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is a draft of support for IoT NTN phase 3 functionality. The store and forward functionality is captured in the latest RAN3 endorsed baseline CR R3-250861 and therefore not included here. NOTE three open issues: 1 For the UL capacity enhancements contention based Msg3 EDT we added two alternative implementations 1st is described in 7.3b as additions to existing MO-EDT and the 2nd is in a new section 7.3x. In the final CR we will need to select one of these options. 2 Further, how shall the agreement “We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.” be included in the specification when we already have a the statement “ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. ” in 10.1.4. Also, is a corresponding liint needed to be stated for eMTC, for example in 23.3? |  |  |  |  |  |  |  |  |
| Summary of change: |  | An [updated section 7.3b or new section 7.3x] is used to describe the contention based Msg3 EDT procedure. The public warning system for NB-IoT is added to stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN phase 3 has no stage 2 description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.10, 7.3b, 7.3c, 7.3x, 7.4, 8.1, 23.3.1, 23.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CRxxxx, TS 36.321 CRyyyy, TS 36.304 CRzzzz, TS 36.306 CRåååå |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | All sections are included where changes may be needed, unaffected parts will be removed before approval. This document is the result of the post RAN2#129bis discussion. |  |  |  |  |  |  |  |  |

Editor’s Note: All sections are included where changes may be needed, unaffected parts will be removed before approval.

START OF CHANGE

## 3.1	Definitions

***************** unaffected part omitted ***************

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Carrier frequency: center frequency of the cell.

Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

Cell Group: in dual connectivity, a group of serving cells associated with either the MeNB or the SeNB.

***************** unaffected part omitted ***************

START OF CHANGE

## 3.2	Abbreviations

***************** unaffected part omitted ***************

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CB-Msg3-EDT	Contention Based Msg3 Early Data Transmission

CB-RNTI	CB-Msg3 EDT RNTI

CC	Component Carrier

CG	Cell Group

***************** unaffected part omitted ***************

START OF CHANGE

## 4.10	NB-IoT

NB-IoT provides access to network services using physical layer optimized for very low power consumption (e.g. full carrier bandwidth is 180 kHz, subcarrier spacing can be 3.75 kHz or 15 kHz).

As indicated in the relevant clauses in this specification, a number of E-UTRA protocol functions supported by all Rel-8 UEs are not used for NB-IoT and need not be supported by eNBs and UEs only using NB-IoT.

In this version of the specification, a number of functions including inter-RAT mobility, handover, measurement reports, GBR, CSG, support of HeNBs, relaying, carrier aggregation, dual connectivity, NAICS, real-time services, interference avoidance for in-device coexistence, RAN assisted WLAN interworking, sidelink communication/discovery, V2X sidelink communication, MDT, emergency call, CS fallback, ACB, EAB, ACDC, SSAC, aerial UE Communication, EN-DC and RRC_INACTIVE are not supported for NB-IoT. This is not further stated in the corresponding procedures. In NTN, public warning system requirements may not be supported for some scenarios.

Editor’s Note: RAN2 made an agreement to state in 4.10 that ETWS, CMAS and PWS requirements may not be supported for some scenarios. However, “ETWS, CMAS and PWS requirements may not be supported for some scenarios” is covered by the legacy text in 10.1.4: 
“ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX.”

Editor’s Note: Do we need to state the corresponding limits for eMTC, for example in 23.3?

START OF CHANGE

## 4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figure 4.12-1 below illustrates an example of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Figure 4.12-1: Overall illustration of an NTN

NOTE:	Figure 4.12-1 illustrates an NTN; RAN4 aspects are out of scope.

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The following connectivity is supported by the NTN payload:

-	An eNB may serve multiple NTN payloads;

-	An NTN payload may be served by multiple eNBs.

NOTE:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

START OF CHANGE

NOTE: The changes in this section are an alternative implementation for adding CB-Msg3-EDT, that is only one of 7.3b and 7.3x shall be selected for the final CR

## 7.3b	MO-EDT

### 7.3b.1	General

MO-EDT allows one uplink data transmission optionally followed by one downlink data transmission either during the random access procedure or during the CB-Msg3-EDT procedure.

MO-EDT is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data (i.e., not signalling or SMS) and the uplink data size is less than or equal to a TB size indicated in the system information. MO-EDT is not used for data over the control plane when using the User Plane CIoT EPS/5GS optimisations.

MO-EDT is only applicable to BL UEs, UEs in enhanced coverage and NB-IoT UEs.

MO-EDT using CB-Msg3-EDT procedure is only applicable in NTNs to Control Plane CIoT EPS optimisation and User Plane CIoT EPS optimisation in NTNs for BL UEs, UEs in enhanced coverage mode A and NB-IoT UEs.

Contention based Msg3 EDT procedure (CB-Msg3-EDT procedure) allows EDT from RRC_IDLE using contention based uplink resources without random access preamble transmission and without random access response reception. The CB-Msg3-EDT uplink transmission utilize a diversity slotted aloha (DSA) method where the uplink transmission is replicated k number of times at randomly selected occasions within a configurable window and the value of k is configured by the network. Following a CB-Msg3-EDT uplink transmission, the UE monitors PDCCH for a response and the network may respond to multiple UEs in one Msg4. A UE shall at most have one ongoing CB-Msg3-EDT procedure at any time.

### 7.3b.2	MO-EDT for Control Plane CIoT EPS/5GS optimisations

MO-EDT for Control Plane CIoT EPS optimisation, as defined in TS 24.301 [20], and Control Plane CIoT 5GS Optimisation, as defined in TS 24.501 [91], are characterized as below:

-	Uplink user data are transmitted in a NAS message concatenated in UL RRCEarlyDataRequest message on CCCH;

-	Downlink user data are optionally transmitted in a NAS message concatenated in DL RRCEarlyDataComplete message on CCCH;

-	There is no transition to RRC CONNECTED.

The MO-EDT procedure for Control Plane CIoT EPS optimisation and Control Plane CIoT 5GS Optimisation are illustrated in Figure 7.3b-1 and Figure 7.3b-1a respectively.

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

Figure 7.3b-1a: MO-EDT for Control Plane CIoT 5GS Optimisation

0.	Upon connection establishment request for Mobile Originated data from the upper layers, the UE either initiates the MO-EDT procedure and selects a random access preamble configured for EDT or the UE initiates CB-Msg3-EDT procedure.

1.	UE sends RRCEarlyDataRequest message concatenating the user data on CCCH. For EPS if enabled in the cell, or for 5GS, the UE may indicate AS Release Assistance Information.

2.	For EPS, the eNB initiates the S1-AP Initial UE message procedure to forward the NAS message and establish the S1 connection. For 5GS, the ng-eNB initiates the NG-AP Initial UE message procedure to forward the NAS message. The (ng-)eNB may indicate in this procedure that this connection is triggered for EDT.

3.	For EPS, the MME requests the S-GW to re-activate the EPS bearers for the UE. For 5GS, the AMF determines the PDU session contained in the NAS message.

4.	For EPS, the MME sends the uplink data to the S-GW. For 5GS, the AMF sends the PDU Session ID and the uplink data to the SMF and the SMF forwards the uplink data to the UPF.

5.	For EPS, if downlink data are available, the S-GW sends the downlink data to the MME. For 5GS, if downlink data are available, the UPF forwards the downlink data to SMF and the SFM forwards the downlink data to AMF.

6.	If downlink data are received from the S-GW or SMF, the MME or AMF forwards the data to the eNB or ng-eNB via DL NAS Transport procedure and may also indicate whether further data are expected. Otherwise, the MME or AMF may trigger Connection Establishment Indication procedure and also indicate whether further data are expected.

7.	If no further data are expected, the (ng-)eNB can send the RRCEarlyDataComplete message on CCCH to keep the UE in RRC_IDLE. If downlink data were received in step 6, they are concatenated in RRCEarlyDataComplete message.

Editor’s Note: Exact how to capture msg4 for CB-Msg3-EDT is pending further agreements, it may at least affect step 7 above and the figure.

8.	For EPS, the S1 connection is released and the EPS bearers are deactivated. For 5GS, the AN release procedure is started.

NOTE 1:	If the MME/AMF or the (ng-)eNB decides to move the UE in RRC_CONNECTED mode, RRCConnectionSetup message is sent in step 7 to fall back to the legacy RRC Connection establishment procedure and the eNB assigns a new C-RNTI for CB-Msg3-EDT; the (ng-)eNB will discard the zero-length NAS PDU received in RRCConnectionSetupComplete message.

NOTE 2:	If neither RRCEarlyDataComplete nor, in case of fallback, RRCConnectionSetup is received in response to RRCEarlyDataRequest, the UE considers the UL data transmission not successful.

Editor’s Note: Exact how to capture not successful UL data transmission in CB-Msg3-EDT is FFS, the NOTE 2 above may need to be updated.

### 7.3b.3	MO-EDT for User Plane CIoT EPS/5GS optimisations

MO-EDT for User Plane CIoT EPS optimisation, as defined in TS 24.301 [20], and for User Plane CIoT 5GS Optimisation, as defined in TS 24.501 [91], are characterized as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

-	Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	There is no transition to RRC CONNECTED.

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2.

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE either initiates the MO-EDT procedure and selects a random access preamble configured for EDT or the UE initiates CB-Msg3-EDT procedure.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. If enabled in the cell, the UE may indicate AS Release Assistance Information.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH. The procedure ends with the reception of the HARQ feedback (ARQ) acknowledging the successful DL transmission.

Editor’s Note: Exact how to capture msg4 for CB-Msg3-EDT is pending further agreements, it may at least affect step 8 above and the figure above.

The MO-EDT procedure for User Plane CIoT 5GS Optimisation is illustrated in Figure 7.3b-2a.

Figure 7.3b-2a: MO-EDT for User Plane CIoT 5GS Optimisation

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the MO-EDT procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the ng-eNB, including its I-RNTI, the resume cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. The UE may indicate AS Release Assistance Information.

2.	The uplink data are delivered to the UPF.

3.	The ng-eNB sends a NG-AP Context Resume Request message to the AMF to resume the connection. If the UE included AS Release Assistance information indicating No further UL/DL higher layer PDU in step 1, ng-eNB may request for immediate transition to RRC IDLE with Suspend.

4.	If the AMF does not receive a request for immediate transition to RRC IDLE with Suspend in step 3 or the AMF is aware of downlink data or signalling pending, the AMF requests the SMF to resume the PDU session.

5.	The AMF sends a NG-AP Context Resume Response to the ng-eNB. If the AMF receives a request for immediate transition to RRC IDLE with Suspend in step 3 and there is no downlink data or signalling pending, the AMF includes a Suspend indication, and keeps the UE in CM-IDLE with Suspend.

6.	If the AMF includes Suspend indication in step 5, the ng-eNB proceeds to step 8. If the AMF does not include Suspend indication and the UE included AS Release Assistance information indicating Only a single Downlink Data transmission subsequent to the Uplink transmission in step 1, the ng-eNB may wait for the DL data to arrive, and proceeds to step 7.

7	The ng-eNB initiates the NG-AP UE Context Suspend procedure to inform the AMF that the RRC connection is being suspended. The AMF requests the SMF to suspend the PDU session and the SMF requests the UPF to release the tunnel information for the UE.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the I-RNTI, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH. The procedure ends with the reception of the HARQ feedback (ARQ) acknowledging the successful DL transmission.

NOTE 1:	If the MME/AMF or (ng-)eNB decides to move the UE to RRC_CONNECTED mode, RRCConnectionResume message, including a new C-RNTI for CB-Msg3-EDT, is sent in step 8 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 8 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest for MO-EDT, the UE considers the UL data transmission not successful.

For MO-EDT for User Plane CIoT EPS Optimisation and User Plane CIoT 5GS Optimisation, an RRC connection can also be resumed in an (ng-)eNB (the new (ng-)eNB) different from the one where the connection was suspended (the old (ng-)eNB). Inter (ng-)eNB connection resumption is handled using context fetching, whereby the new (ng-)eNB retrieves the UE context from the old (ng-)eNB over the X2 (Xn) interface. The new (ng-)eNB provides the Resume ID for EPS or I-RNTI for 5GS which is used by the old (ng-)eNB to identify the UE context. This is illustrated in Figure 7.3b-3 and Figure 7.3b-3a for the case of User Plane CIoT EPS Optimisation and for the case of User Plane CIoT 5GS Optimisation respectively.

Figure: 7.3b-3: MO-EDT for User Plane CIoT EPS Optimisations in different eNB

Figure: 7.3b-3a: MO-EDT for User Plane CIoT 5GS Optimisation in different ng-eNB

1.	Same as step 1 in the intra (ng-)eNB connection resumption.

2.	The new (ng-)eNB locates the old (ng-)eNB using the Resume ID (for EPS) or I-RNTI (for 5GS) and retrieves the UE context by means of the X2-AP (for EPS) or Xn-AP (for 5GS) Retrieve UE Context procedure.

3.	The old (ng-)eNB responds with the UE context associated with the Resume ID (for EPS) or I-RNTI (for 5GS).

4.	For EPS, the new eNB initiates the S1-AP Path Switch procedure to establish a S1 UE associated signalling connection to the serving MME and to request the MME to resume the UE context. For 5GS, the new ng-eNB initiates the NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF and to request the AMF to resume the UE context.

5.	For EPS, the MME requests the S-GW to activate the S1-U bearers for the UE and updates the downlink path. For 5GS, the AMF requests requests the SMF to resume the PDU session and the SMF requests the UPF to create the tunnel information for the UE and update the downlink path.

6.	MME/AMF Acks step 5.

7.	For EPS, after the S1-AP Path Switch procedure the new eNB triggers release of the UE context at the old eNB by means of the X2-AP UE Context Release procedure. For 5GS, after the NG-AP Path Switch procedure the new ng-eNB triggers release of the UE context at the old ng-eNB by means of the Xn-AP UE Context Release procedure.

8.	For EPS, same as step 5 in the intra eNB connection resumption. For 5GS, the uplink data are delivered to the UPF.

9.	Same as step 6 in the intra (ng-)eNB connection resumption.

10.	Same as step 7 in the intra (ng-)eNB connection resumption.

11.	Same as step 8 in the intra (ng-)eNB connection resumption.

START OF CHANGE

## 7.3c	MT-EDT

### 7.3c.1	General

MT-EDT is intended for a single downlink data transmission either during the random access procedure or during the CB-Msg3-EDT procedure.

MT-EDT is initiated by the MME if the UE and the network support MT-EDT and there is a single DL data transmission for the UE.

MT-EDT for Control Plane CIoT EPS Optimisation and for User Plane CIoT EPS Optimisation, as defined in TS 23.401 [17], is characterised as below:

-	Support for MT-EDT for the Control Plane CIoT EPS Optimisation and/or for the User Plane CIoT EPS Optimisation is reported by UE at NAS level;

-	DL data size is included in the S1-AP Paging message for the UE;

-	MT-EDT indication is included in the Paging message for the UE over the Uu interface;

-	For User Plane CIoT EPS Optimisation, the UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	In response to the Paging message including MT-EDT indication, the UE triggers the MO-EDT procedure for Control Plane CIoT EPS Optimisation or for User Plane CIoT EPS Optimisation if the upper layers request the establishment or resumption of the RRC Connection for Mobile Terminated Call;

-	There is no transition to RRC CONNECTED.

MT-EDT is only applicable to BL UEs, UEs in enhanced coverage and NB-IoT UEs.

MT-EDT using CB-Msg3-EDT procedure is only applicable in NTNs for BL UEs, UEs in enhanced coverage mode A and NB-IoT UEs.

### 7.3c.2	MT-EDT for Control Plane CIoT EPS Optimisation

The MT-EDT procedure for Control Plane CIoT EPS Optimisation is illustrated in Figure 7.3c-1.

Figure 7.3c-1: MT-EDT for Control Plane CIoT EPS Optimisation

1.	Upon arrival of downlink data, the SGW may send the DL data size information to the MME for MT-EDT consideration by the MME.

2.	The MME includes the DL data size information in the S1-AP PAGING message to assist eNodeB in triggering MT-EDT.

3.	If the data can fit in one single downlink transmission according to the UE category included in the UE Radio Capability for Paging provided in the S1-AP Paging message, the eNB includes mt-EDT indication in the Paging message for the UE.

4.	The UE initiates the MO-EDT procedure for the Control Plane CIoT EPS Optimisation as described in clause 7.3b.2 or clause 7.3x.2 with the following differences:

-	In step 1, the UE sends RRCEarlyDataRequest message with the establishment cause mt-Access and without user data.

-	In step 7, in case of fallback to the RRC Connection establishment procedure, the downlink data may optionally be included in RRCConnectionSetup message.

### 7.3c.3	MT-EDT for User Plane CIoT EPS Optimisation

The MT-EDT procedure for User Plane CIoT EPS Optimisation is illustrated in Figure 7.3c-2.

Figure 7.3c-2: MT-EDT for User Plane CIoT EPS Optimisation

1.	Upon arrival of downlink data, the SGW may send the DL data size to the MME for MT-EDT consideration by the MME.

2.	The MME includes the DL data size in the S1-AP PAGING message to assist eNodeB in triggering MT- EDT.

3.	If the data can fit in one single downlink transmission according to the UE category included in the UE Radio Capability for Paging provided in the S1-AP Paging message, the eNB includes mt-EDT indication in the Paging message for the UE.

4.	The UE initiates the MO-EDT procedure for the User Plane CIoT EPS Optimisation as described in clause 7.3b.3/ figure 7.3b-2 or in clause 7.3x.3/ figure 7.3x-2 for CB-Msg3-EDT with the following differences:

-	In step 0, the UE either initiates MO-EDT procedure and selects a random access preamble not configured for EDT or the UE initiates the CB-Msg3-EDT procedure;

-	In step 1, the UE sends RRCConnectionResumeRequest message with the resume cause mt-EDT and without user data.

-	In step 4, the MME may include the Pending Data Indication in the S1AP UE Context Resume Response message to notify the eNB of further data traffic in excess of that initially signalled in step 2. The eNB may use this indication to decide whether to release the UE.

START OF CHANGE

NOTE: The changes in this section are an alternative implementation for adding CB-Msg3-EDT, that is only one of 7.3b and 7.3x shall be selected for the final CR

## 7.3x	CB-Msg3-EDT procedure

### 7.3x.1	General

The contention based Msg3 EDT (CB-Msg3-EDT) procedure allows EDT from RRC_IDLE using contention based uplink resources without random access preamble transmission and without random access response reception. The CB-Msg3-EDT uplink transmissions utilize a diversity slotted aloha (DSA) method where the uplink transmission is replicated k number of times at randomly selected occasions within a configurable window and the value of k is configured by the network. Following a CB-Msg3-EDT transmission, the UE monitors PDCCH for a response and the network may respond to multiple UEs in one Msg4.

The CB-Msg3-EDT procedure is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data (i.e., not signalling or SMS) and the uplink data size is less than or equal to a TB size indicated in the system information and the measured RSRP is higher than a RSRP level indicated in the system information for CB-Msg3-EDT. The CB-Msg3-EDT procedure is not used for data over the control plane when using the User Plane CIoT EPS optimisation.

The CB-Msg3-EDT procedure is only applicable to NTNs for BL UEs, UEs in enhanced coverage mode A and NB-IoT UEs.

A UE shall at most have one ongoing CB-Msg3-EDT procedure at any time.

### 7.3x.2	CB-Msg3-EDT for Control Plane CIoT EPS optimisation

The CB-Msg3-EDT for Control Plane CIoT EPS optimisation, as defined in TS 24.301 [20], is characterised as below:

-	Uplink user data is transmitted using the CB-Msg3-EDT resource in a NAS message concatenated in RRCEarlyDataRequest message on CCCH;

-	Downlink user data is optionally transmitted in a NAS message concatenated in RRCEarlyDataComplete message on CCCH;

-	There is no transition to RRC CONNECTED.

The CB-Msg3-EDT procedure for the Control Plane CIoT EPS optimisation is illustrated in Figure 7.3x-1.

Figure 7.3x-1: CB-MSG3-EDT for Control Plane CIoT EPS optimisation

0.	Upon connection establishment request for CB-Msg3-EDT Mobile Originated data from the upper layers, the UE initiates the CB-Msg3-EDT procedure.

1	Same as step 1 in MO-EDT for Control Plane CIoT EPS optimisation in Figure 7.3b-1 except that the UE transmits over the CB-Msg3-EDT resources instead of a resource allocated in the random access response.

2..6 Same as MO-EDT for Control Plane CIoT EPS optimisation in Figure 7.3b-1.

7	Same as step 7 in MO-EDT for Control Plane CIoT EPS optimisation in Figure 7.3b-1.

8	Same as step 8 in MO-EDT for Control Plane CIoT EPS optimisation in Figure 7.3b-1.

Editor’s Note: Exact how to capture msg4 is pending further agreements, it may at least affect step 7 above.

NOTE 1:	If the MME or the eNB decides to move the UE to RRC_CONNECTED mode, RRCConnectionSetup message is sent in step 7 to fall back to the legacy RRC Connection establishment procedure, a new C-RNTI is assigned; the eNB will discard the zero-length NAS PDU received in RRCConnectionSetupComplete message.

NOTE 2:	If neither RRCEarlyDataComplete nor, in case of fallback, RRCConnectionSetup is received in response to RRCEarlyDataRequest, the UE considers the UL data transmission not successful.

Editor’s Note: Exact how to capture not successful UL data transmission is FFS, the NOTE 2 above may need updates.

### 7.3x.3	CB-Msg3-EDT for User Plane CIoT EPS optimisation

The CB-Msg3-EDT procedure for User Plane CIoT EPS optimisation, as defined in TS 24.301 [20], is characterised as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data is transmitted on DTCH multiplexed with RRCConnectionResumeRequest message on CCCH;

-	Downlink user data is optionally transmitted on DTCH multiplexed with RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink is ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	There is no transition to RRC CONNECTED.

The CB-Msg3-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3x-2.

Figure 7.3x-2: CB-Msg3-EDT for User Plane CIoT EPS optimisation

0.	Upon connection resumption request for CB-Msg3-EDT Mobile Originated data from the upper layers, the UE initiates the CB-Msg3-EDT procedure.

1.	Same as step 1 in MO-EDT for User Plane CIoT EPS optimisation in Figure 7.3b-2 except that the UE transmits over the CB-Msg3-EDT resources instead of a resource allocated in the random access response.

2..7 Same as MO-EDT for User Plane CIoT EPS optimisation in Figure 7.3b-2.

8.	Same as step 8 in MO-EDT for user Plane CIoT EPS optimisation in Figure 7.3b-2.

Editor’s Note: Exact how to capture msg4 is pending further agreements, it may at least affect step 8 above.

NOTE 1:	If the MME or eNB decides to move the UE to RRC_CONNECTED mode, RRCConnectionResume message is sent in step 8 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 8 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest for CB-Msg3-EDT, the UE considers the UL data transmission not successful.

CB-Msg3-EDT procedure for User Plane CIoT EPS optimisation, an RRC connection can also be resumed in an eNB (the new eNB) different from the one where the connection was suspended (the old eNB). Inter eNB connection resumption is handled using context fetching, whereby the new eNB retrieves the UE context from the old eNB over the X2 interface. The new eNB provides the Resume ID for EPS which is used by the old eNB to identify the UE context. This is illustrated in Figure 7.3x-3.

Figure: 7.3x-3: CB-Msg3-EDT for User Plane CIoT EPS optimisation in different eNB

1	Same as step 1 in MO-EDT for User Plane CIoT EPS optimisation in different eNB in Figure 7.3b-3 except that the UE transmits over the CB-Msg3-EDT resources instead of a resource allocated in the random access response.

2..10 Same as MO-EDT for User Plane CIoT EPS optimisation in different eNB in Figure 7.3b-3.

11.	Same as step 11 in MO-EDT for User Plane CIoT EPS optimisation in different eNB in Figure 7.3b-3.

Editor’s Note: Exact how to capture msg4 is pending further agreements, it may at least affect step 11 above.

START OF CHANGE

## 7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters). It can also contain information about E-UTRA and NR idle/inactive measurements;

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency), which can also be used for NR idle/inactive measurements;

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockType26a contains information related to NR bands list which can be used for EN-DC operation with the serving cell;

-	SystemInformationBlockType27 contains assistance information for inter-RAT cell selection to NB-IoT;

-	SystemInformationBlockType28 contains information related to NR sidelink communication;

-	SystemInformationBlockType29 contains information related to common resource reservation;

-	SystemInformationBlockType30 contains information related to disaster roaming;

-	SystemInformationBlockType31 contains information required for accessing an NTN cell;

-	SystemInformationBlockType32 contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33 contains assistance information related to neighbouring NTN cells for measurements.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType10-NB contains an ETWS primary notification;

-	SystemInformationBlockType11-NB contains an ETWS secondary notification;

-	SystemInformationBlockType12-NB contains a CMAS warning notification;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockType27-NB contains assistance information for inter-RAT cell selection to E-UTRAN and/or GERAN;

-	SystemInformationBlockType31-NB contains information required for accessing an NTN cell;

-	SystemInformationBlockType32-NB contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33-NB contains assistance information related to neighbouring NTN cells for measurements.

On MBMS-dedicated cell, only system information relevant for receiving MBMS service is broadcasted. MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS (SIB1-MBMS) are used instead of MIB and SIB1 respectively:

-	MasterInformationBlock-MBMS defines the most essential physical layer information of the cell required to receive further system information on MBMS-dedicated cell;

-	SystemInformationBlockType1-MBMS contains information relevant for receiving MBMS service and defines the scheduling of other system information blocks on MBMS-dedicated cell;

The MIB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and BR-BCCH, and carried on DL-SCH. Except for BL UEs, UEs in enhanced coverage and NB-IoT UEs, all other SI messages than the MIB which are dynamically carried on DL-SCH, can be identified through the SI-RNTI (System Information RNTI). Both the MIB and SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage) use a fixed schedule with a periodicity of 40 and 80 ms respectively. The scheduling of other SI messages is flexible and indicated by SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage, and SystemInformationBlockType1-NB for NB-IoT). For NB-IoT, the MIB-NB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and carried on DL-SCH. Both the MIB-NB and SystemInformationBlockType1-NB use a fixed schedule with a periodicity of 640 and 2560 ms respectively. The MIB-NB contains all information required to acquire SIB1-NB and SIB1-NB contains all information required to acquire other SI messages.

On MBMS-dedicated cell, the MIB-MBMS and SIB1-MBMS use a fixed schedule with a periodicity of 160 ms. Additionally, SIB1-MBMS may be scheduled in additional non-MBSFN subframes indicated in MIB-MBMS.

For NB-IoT, in TDD mode, the MIB-TDD-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS, SystemInformationBlockType1-NB can be transmitted on NB-IoT carrier other than the MIB-NB, and the SI messages can be transmitted on a NB-IoT carrier other than the MIB-NB. At most two NB-IoT carriers are used to transmit the MIB-NB, SystemInformationBlockType1-NB and the SI messages.

Except for NB-IoT, the eNB may schedule DL-SCH transmissions concerning logical channels other than BCCH or BR-BCCH in the same subframe as used for BCCH or BR-BCCH. The minimum UE capability restricts the BCCH or BR-BCCH mapped to DL-SCH e.g. regarding the maximum rate.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).

Except for NB-IoT, system information may also be provided to the UE by means of dedicated signalling e.g. upon handover.

START OF CHANGE

## 8.1	E-UTRA related UE identities

The following E-UTRA related UE identities are used at cell level:

-	C-RNTI: unique identification used for identifying RRC Connection and scheduling;

-	Semi-Persistent Scheduling C-RNTI: unique identification used for semi-persistent scheduling;

-	Temporary C-RNTI: identification used for the random access procedure;

-	TPC-PUSCH-RNTI: identification used for the power control of PUSCH;

-	TPC-PUCCH-RNTI: identification used for the power control of PUCCH;

-	SL-RNTI: identification used for sidelink communication scheduling;

-	SL-V-RNTI: identification used for V2X sidelink communication scheduling;

-	Random value for contention resolution: during some transient states, the UE is temporarily identified with a random value used for contention resolution purposes;

-	SRS-TPC-RNTI: identification used for triggering group SRS and power control of SRS for SRS-only SCells;

-	SL Semi-Persistent Scheduling V-RNTI: identification used for semi-persistent scheduling for V2X sidelink communication;

-	UL Semi-Persistent Scheduling V-RNTI: identification used for multiple semi-persistent scheduling for UE capable of V2X communication;

-	AUL C-RNTI: unique identification used for autonomous uplink scheduling;

-	PUR-RNTI: identification used for transmission using PUR;

-	CB-RNTI: identification used for the CB-Msg3-EDT procedure.

In DC, two C-RNTIs are independently allocated to the UE: one for MCG, and one for SCG.

The following UE identity is only used for E-UTRA connected to EPC:

-	Resume ID: unique identification used for the RRC connection resume procedure;

The following UE identity is only used for E-UTRA connected to 5GC:

-	I-RNTI: unique identification used for the RRC connection resume procedure in RRC_INACTIVE or for the User Plane CIoT 5GS Optimisation as specified for NR connected to 5GC in TS 38.300 [79];

START OF CHANGE

### 10.1.4	Paging and C-plane establishment

Paging groups (where multiple UEs can be addressed) are used on PDCCH:

-	Precise UE identity is found on PCH;

-	DRX configurable via BCCH and NAS;

-	Only one subframe allocated per paging interval per UE;

-	The network may divide UEs to different paging occasions in time;

-	There is no grouping within paging occasion;

-	One paging RNTI for PCH.

When extended DRX (eDRX) is used in idle mode, the following are applicable:

-	The DRX cycle is extended up to and beyond 10.24s in idle mode, with a maximum value of 2621.44 seconds (43.69 minutes); For NB-IoT, the maximum value of the DRX cycle is 10485.76 seconds (2.91 hours);

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging DRX during a Paging Time Window (PTW) used in ECM-IDLE. The PH is determined based on a formula that is known by the MME/AMF, UE and (ng-)eNB as a function of eDRX cycle and UE identity;

-	During the PTW, the UE monitors paging for the duration of the PTW (as configured by NAS) or until a paging message is including the UE's NAS identity received for the UE, whichever is earlier. The possible starting offsets for the PTW are uniformly distributed within the PH and defined in TS 36.304 [11];

-	MME/AMF uses the formulas defined in TS 36.304 [11] to determine the PH as well as the beginning of the PTW and sends the S1 paging request just before the occurrence of the start of PTW or during PTW to avoid storing paging messages in the (ng-)eNB;

-	ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection;

-	When the eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before establishing an RRC connection. Paging message can be used for system information change notification, when including systemInfoModification-eDRX, for a UE configured with eDRX cycle longer than the system information modification period.

NB-IoT UEs, BL UEs or UEs in enhanced coverage can use (G)WUS, when configured in the cell, to reduce the power consumption related to paging monitoring. (G)WUS is only applicable in RRC_IDLE.

When GWUS is used in RRC_IDLE, the following are applicable:

-	Multiple WUS groups, possibly distributed over multiple WUS resources, can be configured in the cell;

-	If the UE supports WUS assistance information, the MME/AMF may provide the UE with UE paging probability information (see TS 24.301 [20] and TS 24.501 [91]);

-	UE selects one WUS group based on its UE paging probability information and /or its UE NAS identity as defined in TS 36.304 [11];

-	A common WUS group may be used to wake up all UEs monitoring the same WUS resource.

When (G)WUS is used in RRC_IDLE, the following are applicable:

-	The UE monitors (G)WUS only in the last used cell as defined in TS 36.304 [11];

-	The WUS or WUS group is used to indicate that the UE shall monitor MPDCCH or NPDCCH to receive paging in that cell;

-	For a UE not configured with extended DRX, the WUS or WUS group is associated to one paging occasion (N = 1);

-	For a UE configured with extended DRX, the WUS or WUS group can be associated to one or multiple paging occasion(s) (N ≥ 1) in a PTW;

-	If UE detects the WUS or WUS group, the UE shall monitor the following N paging occasions unless it has received a paging message;

-	The paging operation in the MME/AMF is not aware of the use of the WUS in the (ng-)eNB;

-	To reduce WUS use in cells not monitored by the UE, WUS-capable (ng-)eNBs provide UE's last used cell information to MME/AMF in the S1-AP/NG-AP UE Context Release Complete or UE Context Suspend Request messages for all UEs, as described in TS 23.401 [17] and TS 23.501 [82]. In case of immediate suspension of a UE, the WUS-capable ng-eNB also provides the UE's last cell information to the AMF in the UE Context Resume Request message, as described in TS 23.501 [82].

The timing between WUS and the paging occasion (PO) is illustrated in Figure 10.1.4-1. The timing between GWUS and the paging occasion (PO) is illustrated in Figure 10.1.4-2 and Figure 10.1.4-3. The UE can expect WUS repetitions during "Configured maximum WUS duration" but the actual WUS transmission can be shorter, e.g. for UE in good coverage. The UE does not monitor WUS during the non-zero "Gap".

Figure 10.1.4-1: Illustration of WUS timing

Figure 10.1.4-2: Illustration of GWUS timing for NB-IoT UEs

Figure 10.1.4-3: Illustration of GWUS timing for BL UEs and UEs in enhanced coverage

NOTE:	WUS1/WUS3 could be higher or lower frequency than WUS0/WUS2.

For NB-IoT, UE in RRC_IDLE receives paging on the anchor carrier or on a non-anchor carrier based on system information.

When coverage based paging is used in NB-IoT:

-	Some paging carriers may be configured for lower levels of coverage enhancements;

-	The eNB can configure a UE during RRC connection release to select one of these paging carriers according to coverage information provided by the eNB;

-	If configured for coverage based paging, the UE only selects one of the coverage based paging carriers if its NRSRP is suitable according to the coverage based paging configuration;

-	Coverage based paging is only applicable in the last cell where the coverage information was received;-	The eNB includes the coverage based paging information provided to the UE within the information on the coverage enhancement (CE) level described in clause 23.13.2.

START OF CHANGE

## 23.3	E-UTRAN Support for Warning Systems

### 23.3.0	General

The E-UTRAN provides support for warning systems through means of system information broadcast capability. The E-UTRAN performs scheduling and broadcasting of the "warning message content" received from the CBC, which is forwarded to the E-UTRAN by the MME. The schedule information for the broadcast is received along with the "warning message content" from the CBC. The E-UTRAN is also responsible for paging the UE to provide indication that the warning notification is being broadcast. The "warning message content" received by the E-UTRAN contains an instance of the warning notification. Depending on the size, E-UTRAN may segment the secondary notification before sending it over the radio interface.

### 23.3.1	Earthquake and Tsunami Warning System

ETWS is a public warning system developed to meet the regulatory requirements for warning notifications related to earthquake and/or tsunami events. ETWS warning notifications can either be a primary notification (short notifications delivered within 4 seconds, see TS 22.168 [37]) or secondary notification (providing detailed information). The ETWS primary notification is broadcast in SystemInformationBlockType10 (SystemInformationBlockType10-NB for NB-IoT) while the secondary notification is broadcast in SystemInformationBlockType11 (SystemInformationBlockType11-NB for NB-IoT).

### 23.3.2	Commercial Mobile Alert System

CMAS is a public warning system developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [34]). The CMAS warning notifications are short text messages (CMAS alerts). The CMAS warning notifications are broadcast in SystemInformationBlockType12 (SystemInformationBlockType12-NB for NB-IoT). The E-UTRAN manages the delivery of multiple, concurrent CMAS warning notifications to the UE and is also responsible for handling any updates of CMAS warning notifications.

START OF CHANGE

## 23.21	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

### 23.21.1	General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers.

UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment.

In this release of the specification, NTN is only applicable to FDD system.

In NTN, the distance refers to Euclidean distance.

### 23.21.2	Timing and synchronization

#### 23.21.2.1	Scheduling timing

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 8 of TS 36.211 [4]).

To accommodate the long propagation delays in NTN, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets: Koffset and Kmac:

-	Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	Kmac is a configured offset (RRC parameter k-Mac) that is approximately equal to the RTT between the RP and the eNB.

The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 36.213 [6].

The offset Kmac is used to delay the application of a downlink configuration indicated by a MAC CE received on NPDSCH/PDSCH, see TS 36.213 [6], and to determine the UE-eNB RTT, see TS 36.321 [13]. It may be provided by the network when downlink and uplink frame timing are not aligned at eNB. The Kmac is also used in the random access procedure, to determine the start time of random access response window after a random access preamble transmission (see TS 36.213 [6]).

The Service link RTT, Feeder link RTT, the RP, the Common TA, Kmac and TTA (see clause 23.21.2.2) are illustrated in Figure 23.21.2.1-1.

Figure 23.21.2.1-1: Illustration of timing relationship (for collocated eNB and NTN Gateway)

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (by dedicated RRC signalling and/or DCI based indication). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled;

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 36.321 [13]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled. HARQ mode configuration is not applicable for PUR transmissions.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by a downlink SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by an uplink SPS configuration.

#### 23.21.2.2	Timing Advance and Frequency Pre-compensation

For the serving cell, the network broadcast ephemeris information and common Timing Advance (common TA) parameters.

The UE shall have valid GNSS position as well as the ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to a cell, the UE shall pre-compensate the Timing Advance (TTA, see TS 36.211 [4] clause 8.1), see Figure 23.21.2.2-1, by considering the common TA, UE position and the NTN payload position through the ephemeris.

The UE computes the frequency Doppler shift of the service link, and pre-compensates for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have valid ephemeris and Common TA, it shall not transmit until they are regained. If the GNSS position becomes out-dated, it shall not transmit unless configured with uplink transmissions extension that is active.

In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. The UE can be triggered to perform, or configured to autonomously perform, GNSS acquisition. In connected mode, upon outdated ephemeris and common Timing Advance, the UE shall acquire the broadcasted parameters. Upon failed GNSS acquisition, the UE shall move to idle mode if the GNSS position is outdated and uplink transmission extension is not active. Upon outdated GNSS position the UE shall move to idle mode, unless GNSS acquisition was triggered or uplink transmission extension is active. Upon completing the GNSS acquisition, the UE shall trigger remaining validity duration reporting (see TS 36.321 [13]).

NOTE:	The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed.

The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

Figure 23.21.2.2-1: Illustration of Uplink/Downlink Radio Frame Timing at the UE

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link and transponder frequency error, whether introduced in Downlink or Uplink, is left to network implementation.

### 23.21.3	Support of discontinuous coverage

As an NTN payload moves on a specified orbit, for example in case of a NGSO satellite, the NTN payload beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the NTN payload. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g. a sparse satellite constellation deployment.

To enable the UE, in RRC_IDLE, to save power during periods of no coverage, the network provides NTN payload assistance information (e.g. ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage) to enable the UE to predict when coverage will be provided by upcoming NTN payloads. Predicting out of coverage and in coverage is up to UE implementation.

If the eNB detects that the UE is out of coverage due to discontinuous coverage, it may initiate a UE Context Release Request procedure towards the serving MME.

### 23.21.4	Mobility Management

#### 23.21.4.1	Mobility Management in ECM-IDLE

The principles described in clause 10.1.1 apply in NTN unless specified otherwise hereafter.

The network may broadcast more than one TAC per PLMN in an NTN cell. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. The UEs may, by UE implementation, check whether a TAC has been removed.

For quasi-Earth-fixed cells, timing information on when the cell is going to stop serving the area may be broadcast by the network. This may be used by the UE to start measurements on neighbour cells before the broadcast stop time of the serving cell, while the exact start of the measurements is up to UE implementation.

Initiation of measurements based on location and time is supported for cell reselection.

#### 23.21.4.2	Mobility Management in ECM-CONNECTED

Radio link failure and RRC connection re-establishment are supported in NTN. The principles described in clause 10.1.6 apply unless specified otherwise. The principles described in clause 10.1.2 apply to NTN unless specified otherwise.

To enable mobility in NTN, the network provides target cell NTN payload assistance information needed to access the NTN cell in the handover command.

Conditional handover is supported for BL UEs and UEs in enhanced coverage.

When operating in NTNs the following additional trigger conditions upon which UE may execute CHO to a candidate cell are supported, as defined in TS 36.331 [16]:

-	The RRM measurement-based event A4;

-	A time-based trigger condition;

-	A location-based trigger condition.

It is up to UE implementation how the UE evaluates the time-based or location-based trigger condition together with the RRM measurement-based event.

When time-based trigger condition is used, the source eNB should consider the time indicated to the UE to decide when to start the early data forwarding to the target eNB.

When a time-based trigger condition is used, the source eNB may signal the corresponding parameters to a single target eNB via the Source eNB to Target eNB Transparent Container in an S1-based handover, see TS 23.401 [17], or X2 based handover. The source eNB signals the corresponding CHO configuration to the UE in the RRC Connection Reconfiguration message during handover.

#### 23.21.4.3	Measurements

The principles described in clause 10.1.3 apply in NTN unless specified otherwise.

To enable measurements, terrestrial and non-terrestrial networks may provide neighbouring cell assistance information via system information.

The following can optionally be used for measurements on neighbour cells in RRC_IDLE as specified in TS 36.331 [16]:

-	The timing and location information associated to the serving cell provided in SIB3 and SIB31;

-	Timing information when the neighbour cell starts serving the current geographical area;

-	Location information refers:

-	In the quasi-Earth fixed cell scenario, to the reference location of the serving cell and a distance threshold to the reference location;

-	In the Earth moving cell scenario, to the reference location of the serving cell at the epoch time and a distance threshold to the reference location.

The following measurement triggers can be configured in RRC_CONNECTED for the purpose of RRC reestablishment or handover as specified in TS 36.331 [16]:

-	A time-based trigger condition;

-	A location-based trigger condition.

The time-based measurement initiation may be applicable for the feeder link switchover case for cell (re)selection.

For a UE in idle mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is indicated in SIB3/SIB5 but without corresponding satellite information in SIB33.

For a UE in connected mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is included in the measurement configuration but without corresponding satellite information in SIB33.

START OF CHANGE

Annex P (informative):
Example implementation of Non-Terrestrial Networks

The following figure illustrates an example implementation of a Non-Terrestrial Network for transparent NTN payload:

Figure P-1: NTN based E-UTRAN

The eNB depicted in Figure P-1 may be subdivided into non-NTN infrastructure eNB functions and the NTN Service Link provisioning System. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning System and the NTN Control function. The NTN Service Link provisioning System may consist of one or more NTN payloads and NTN Gateways.

The NTN payload is embarked on a spaceborne (or airborne) vehicle, providing a structure, power, commanding, telemetry, attitude control for the satellite (resp. high altitude platform station) and possibly an appropriate thermal environment and radiation shielding.

The NTN Service Link provisioning System maps the Uu radio protocol over radio resources of the NTN infrastructure (e.g. beams, channels, Tx power).

The NTN control function controls the spaceborne vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure eNB functions of the eNB.

Provision of NTN control data to the eNB is out of 3GPP scope.

NOTE:	The transport of Uu protocol between the NTN Service Link provisioning system and the non-NTN infrastructure eNB functions is out of 3GPP scope.

At least the following NTN related parameters are expected to be provided by O&M to the eNB for its operation.

a) Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) mapped to the beam;

-	The Cell's reference location (e.g. cell's center and range).

b) Quasi Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) and time window mapped to a beam;

-	The Cell's/beam's reference location (e.g. cell's center and range);

-	The time window of the successive switchovers (feeder link, service link);

-	The identifier and time window of all serving NTN payloads and NTN-Gateways.

c) Earth moving beams: for each beam provided by a given NTN-payload:

-	The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam's foot print on Earth;

-	Its elevation with regard to NTN-payload;

-	Schedule of successive serving NTN-Gateways/eNBs;

-	Schedule of successive switchovers (feeder link, service link).

END OF CHANGE