3GPP TSG-RAN WG2 Meeting #130								R2-2504647

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.13.3

Source:	OPPO, vivo, Qualcomm, Huawei, CATT, MediaTek Inc., KT Corp., China Telecom, ASUSTek, Sharp

Title:	Parallel RRC setup for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

Parallel RRC setup procedure is discussed in the RAN2 129 and 129bis meeting, this paper will further discuss the way forward of this issue.

Agreements:

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left.

Strive to avoid additional RAN3 impact specific to fast forwarding.

FFS if applicable to DL.

FFS what level of gNB awareness of the path information would be needed.

FFS if fast forwarding is optional/mandatory for UEs to support.

Discussion

Based on the conclusion from RAN2#129bis, the scope of the fast-forwarding enhancement is limited to SRB0 only, which means SRB1 forwarding should be based on the legacy/baseline approach, i.e., SRB1 can only be forwarded based on dedicated configuration from the network.

Agreements:

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

The scope of the fast-forwarding enhancement is limited to SRB0 only.

Then, there are several issues identified for supporting UL parallel and DL parallel SRB0 respectively.

Table 1 Issue and Spec Impact for Parallel UL/DL SRB0 Message Delivery

|  | Issue | Spec Impact |
| --- | --- | --- |
| For parallel UL SRB0 transmission | Current local ID based SRAP PDU requires SRAP configuration before forwarding by Relay UE | Either rely on L2 ID based SRAP header, so one additional SRAP header (with L2 ID) format needs to be defined. Or rely on pre-configured local ID. |
|  | Currently UL egress RLC channel determination requires SRAP configuration before forwarding by Relay UE | One additional specified RLC channel needs to be defined for UL SRB0 |
|  | How to solve the collision of L2 ID of faraway UEs, e.g., 2 remote UE with same L2 ID but connects to different Relay UE, so L2 ID cannot be used as the base for UE differentiation for SRB0 message forwarding | SRAP PDU discard if more than one SRAP PDU for UL SRB0 message received with the same L2 ID, and further notification to child UE(s). |
| For parallel DL SRB0 transmission | Currently DL egress link determination requires SRAP configuration before forwarding by Relay UE | Relay UE has to remember the ingress link for the uplink packet and apply that for the downlink packet. The additional SRAP header (with L2 ID) has to be applied to downlink as well. |
|  | Currently DL egress RLC determination requires SRAP configuration before forwarding by Relay UE | The specified RLC channel needs to be applied to DL SRB0 |
|  | It is not clear how to handle the case that RRC setup of the child UE succeeds while RRC setup of parent UE fails (e.g., due to authentication reason) | Last Relay has to report the path/hierarchical information to network, e.g., via SUI. Which further requires solution for child UE(s) to notify the path/hierarchical information to parent UE(s) and further forward to Last Relay. |
|  | Child UE would already start UL SRB1 transmission to parent UE upon receiving DL SRB0, yet parent UE has not acquired SRAP configuration of child UE when receiving the UL SRB1 message, the parent UE shall discard the packet following current error case handling rule. | Change of error case handling, i.e., the parent UE should buffer the problematic SRAP PDU for SL SRB1. FFS when to discard it. |

Given all these impact listed above, and the multiple options on the table (which may lead further combinations between options), it is not possible to fully discuss and solve all the above issues in the limited TU (only two meetings), especially considering the running CR works and open issues (on relay reselection, SI/Paging forwarding, service continuity, etc.) to close. So, it is preferred to focus on the work of baseline procedure to close the WI in time.

Spec impact is huge to enable parallel SRB0 message, it is not possible to fully discuss and solve all the issues listed in Table-1 within the limited TU before R19 closing (only two meetings), especially considering the running CR works.

Even with this parallel SRB0 enhancement, the following SRB1/SRB2/DRB transmission has to wait till dedicated SRAP configuration being acquired, and all the SRB1/2 procedures after SRB0 including RRCsetupComplete, SMC, SUI, NAS messages (delivered via RRC container), RRCReconfiguration should be transmitted/forwarded in a cascaded way, i.e., parent first and child afterwards. Which means the gain for parallel SRB0 forwarding is marginal.

The effort on SRB0 is not justified by the potential gain which is marginal.

(SRAP-4) RAN2 prioritize the work of baseline procedure to close the WI in time and down-prioritizes fast forwarding using parallel procedure.

Conclusion

We have the following observations:

Observation 1	The scope of the fast-forwarding enhancement is limited to SRB0 only.

Observation 2	Spec impact is huge to enable parallel SRB0 message, it is not possible to fully discuss and solve all the issues listed in Table-1 within the limited TU before R19 closing (only two meetings), especially considering the running CR works.

Observation 3	The effort on SRB0 is not justified by the potential gain which is marginal.

We have the following proposals:

Proposal 1	(SRAP-4) RAN2 prioritize the work of baseline procedure to close the WI in time and down-prioritizes fast forwarding using parallel procedure.