3GPP TSG-RAN WG2 #130	R2-2504653

# St. Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# Introduction

During RAN2#129bis, RAN2 made significant progress in the necessary changes to the SMTC framework to accommodate the newly introduced extended initial SSB periodicity and link-level beam hopping. In this contribution, we continue the analysis for RRC_CONNECTED state and address one of the open issues listed for RRC_IDLE/INACTIVE states.

# 2	Discussion

## 2.1	SMTC selection in RRC_CONNECTED

In a typical NTN deployment, the neighbour cells that a UE can physically measure depend on the position the UE occupies within the serving cell. The relatively large cell size (e.g., 50 km) makes it unlikely that a UE needs to measure all possible NTN neighbour cells, let alone all its potential TN neighbour cells. Figure  presents two UEs in opposite areas of an NTN serving cell. While one UE can potentially only measure cells 1, 2, and 3, the second UE can potentially only measure cells 4, 5, and 6. The introduction of beam hopping, and the extended SSB default periodicity makes evident an issue which is well-known since Release 17: neighbour cell measurement configurations do not take UE location into consideration.

- In a typical NTN deployment due to the relatively large cell size, a UE does not need to be configured to measure all potential surrounding neighbour cells.

Figure 1. Relative position of 2 UEs w.r.t. to NTN neighbour cells. All cells operate in the same frequency, colours are illustrative.

During RAN2#129bis, RAN2 reached an initial agreement to the abovementioned problem. A UE will be provided with multiple SMTC configurations (e.g., 6 different combinations of SMTC periodicity and offset) which are associated to a still undecided location-based assistance information (e.g., cell SSB index, reference location, etc). This decision applies to UEs in RRC_IDLE and RRC_INACTIVE state. Nonetheless, several companies raised at the last meeting that the problem is also present for UEs in RRC_CONNECTED state. When the network is not aware of the UE’s whereabouts, the network needs to blindly configure the Measurement Object(s) and the associated SMTC offset and periodicity. Referring to the previous example, the network must guess that UE 2 should be configured to measure cells 4, 5, and/or 6. This is highly undesirable since the UE may receive an unsuitable measurement configuration for the area where it is located and waste energy and time on useless measurements. Therefore, RAN2 also needs to find a solution for RRC_CONNECTED state and, as many companies expressed last meeting, it would be beneficial that this is aligned with the already agreed solution for other RRC states.

- Without location information, the network may need to guess which is the most appropriate measurement configuration (e.g., SMTC) for a UE.

If RAN2 supports multiple SMTC periodicities and offsets, introduce a common solution for measurement configuration/selection in all RRC states.

Regarding potential solutions for RRC_CONNECTED, some companies have suggested that the network may request the UE to report its coarse location using UE Information procedure. In our understanding, this is undesirable since it is an optional capability that may require explicit user consent (up to local regulations – see the excerpt in the annex), only includes the coarse location if available, and involves the exchange of two additional messages (UEInformationRequest and UEInformationResponse). Another proposed alternative was the Measurement Event D1 which does not entirely solve the problem. Firstly, the network still needs to guess which reference locations the UE should be configured with, unless it configures all neighbours which would increase the complexity of the measurement configuration. Secondly, the UE will need to be re-configured multiple times. In conclusion, this solution is also unsuitable for an efficient operation.

- The UE Information procedure and reporting with multiple D1 events are suboptimal solutions to configure measurements (e.g., SMTCs) relevant to the UE’s current location.

One potential solution that does not entail blind configurations, potentially missing user consent, a large signalling overhead or high complexity is for the network to configure the UE with multiple measurement configurations (e.g., SMTCs). Given that the network does not know the UE’s location, the network can configure the UE with multiple measurement configurations and let the UE choose based on related location assistance information (e.g., a reference location). This is exactly the same mechanism that RAN2 has already agreed for RRC_IDLE and RRC_INACTIVE. The only difference between the RRC_CONNECTED case and other RRC states is that in RRC_CONNECTED state the network needs to know which the UE’s final selection of configuration is so that it knows the time windows in which it must refrain from scheduling the UE. The UE would preferably report its choice in the RRCReconfigurationComplete message sent in response to the RRCReconfiguration message including the multiple configurations. We think this alternative does not require the network to obtain the UE location, does not introduce any additional overhead in terms of extra messages, and reaps the benefit of having a single solution for all RRC states.

Adopt the same principle of SMTC selection for RRC_CONNECTED as for RRC_IDLE/INACTIVE.

The UE is provided with multiple measurement configurations (e.g., SMTCs) to be selected from based on location criteria. After the decision, the UE reports its selection to the network.

## 2.2	Cell reselection enhancements

In the past several meetings, there was a discussion on the necessity of preventing legacy (pre-Rel-19 NTN) UEs from accessing cells operating with DL CE, e.g., via access barring. At RAN2#129bis however, the following agreements were captured:

- From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

- We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

Hence RAN2 did not identify a need for access barring of legacy UEs in cells operating with DL CE. It was instead agreed that, for now, RAN2 does not introduce new barring bits for DL CE, and that the discussion on access barring for DL CE is pending further progress in RAN1.

- RAN2 did not identify a need for access barring of legacy UEs in cells operating with DL CE and has agreed that the discussion is pending further progress in RAN1.

In this discussion, initially in [3] and again in [4], it was suggested to reuse the “excluded cell lists” to prevent (legacy) UEs from even measuring on neighbouring cells operating with DL CE, to prevent wasting UE energy. This is, however, not the case, as was already clarified in [5]. The intra- and inter-frequency excluded cell list in SIB3 and SIB4, respectively, comprise PCIs, not frequencies. The UE is measuring certain frequencies at certain times (according to the provided SMTC information). Only after capturing the SSB of a cell does the UE know the PCI of the cell and can check whether the cell is on an excluded cell list. Hence, the “excluded cell lists” are not a suitable tool for preventing a UE to perform measurements on cells, which was the suggested motivation.

- The “excluded cell lists” are not a suitable tool for preventing a UE to perform measurements on cells.

From RAN2 perspective, legacy UEs may access cells operating with DL CE. Therefore, if a legacy UE is able to cope with the extended SSB periodicity, obtain sufficient measurement samples, and comply with the cell reselection criterion, then it should not be prevented by any other mechanisms to perform the procedure. In case some network implementation wants to prevent cell reselection, this can still be achieved through legacy mechanisms such as the modification of the R criterion for specific cells (e.g., Qoffset). In our view, even though this mechanism was introduced in the NES work item, there is not enough motivation in NTN scenarios using DL CE. We therefore propose not to consider the (re)use of “excluded cell lists”.

No enhancements to prevent legacy UEs from performing cell reselection to DL CE cells are pursued in Rel-19 (e.g., “excluded cell lists”).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In a typical NTN deployment due to the relatively large cell size, a UE does not need to be configured to measure all potential surrounding neighbour cells.

Observation 2	Without location information, the network may need to guess which is the most appropriate measurement configuration (e.g., SMTC) for a UE.

Observation 3	The UE Information procedure and reporting with multiple D1 events are suboptimal solutions to configure measurements (e.g., SMTCs) relevant to the UE’s current location.

Observation 4	RAN2 did not identify a need for access barring of legacy UEs in cells operating with DL CE and has agreed that the discussion is pending further progress in RAN1.

Observation 5	The “excluded cell lists” are not a suitable tool for preventing a UE to perform measurements on cells.

Based on the discussion in the previous sections we propose the following:

Proposal 1	If RAN2 supports multiple SMTC periodicities and offsets, introduce a common solution for measurement configuration/selection in all RRC states.

Proposal 2	Adopt the same principle of SMTC selection for RRC_CONNECTED as for RRC_IDLE/INACTIVE.

Proposal 3	The UE is provided with multiple measurement configurations (e.g., SMTCs) to be selected from based on location criteria. After the decision, the UE reports its selection to the network.

Proposal 4	No enhancements to prevent legacy UEs from performing cell reselection to DL CE cells are pursued in Rel-19 (e.g., “excluded cell lists”).

# References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

R2-2408655, Consideration on DL coverage enhancements, ZTE corporation, Sanechips, RAN2#127bis, October 2024.

R2-2410365, Consideration on DL coverage enhancements, ZTE corporation, Sanechips, RAN2#128, November 2024.

R2-2410806, Downlink coverage enhancement access control, Sequans Communications, RAN2#128, November 2024.

S3-221268, LS reply on Reply LS on NTN specific User Consent and UE location in connected mode in NTN, SA3#107, May 2022.

# Annex

2) With respect to the coarse-grained GNSS location reporting by the UE with an accuracy of 2km without explicit consent, it is up to local regulations whether it is acceptable or not. In case explicit user consent is required by local regulation it is up to proprietary mechanisms for Rel-17.

3) With respect to the implicit user consent approach which is considered by RAN2 LS R2-2204257 for Rel-17, SA3 would like to suggest that it shall be the network that decides whether there is a user consent for the aforementioned location request (based on subscription-based means or proprietary mechanisms) and not the UE. In other words, the network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) and the network should not request for the location if there is no user consent. The UE should provide the location information (if available) if the network requests for it.

Excerpt from S3-221268.