# 3GPP TSG RAN WG2 Meeting #130      	                                                 R2-2504704
St. Julian, Malta, 19th – 23rd 2025

Agenda item:	8.5.1

Source:	Ericsson

Title:	[AT130][104] [NES] (Ericsson)

Document for:	Discussion and Decision

# 1	Introduction

R2-2503608	Report of [POST129b][111][NES] (Ericsson)	Ericsson	discussion	Rel-19

5.2.1

Noted.

[AT130][104][NES] (Ericsson)

Scope: Discuss and attempt to make conclusions on the following issues:

- FFS on SIBX

  - FFS: if list of cells is ARFCN&PCI or only PCI.

- FFS on DownlinkConfigCommonSIB

  - FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

  - FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

- P6 in R2-2504037

Intended outcome: Discussion summary in R2-2504704.

Deadline: Comeback in CB session. 30min f2f offline discussion.

# 2 Discussion

- FFS on SIBX

  - FFS: if list of cells is ARFCN&PCI or only PCI.

- FFS on DownlinkConfigCommonSIB

  - FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

  - FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

- P6 in R2-2504037

## 2.1 FFS in SIBx

- FFS: if list of cells is ARFCN&PCI or only PCI.

The following agreement was made during the WG2 Meeting #129bis meeting:

RAN WG2 Meeting #129bis

Agreements on OD-SIB1

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Current RRC CR has the following way to capture OD-SIB1 config interpreting list of cells as list of PCIs

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                           SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                       OD-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

Related to this topic, RAN1 made the following agreements in RAN#117 and RAN#118bis meetings, respectively:

RAN1#117

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

RAN1#118bis

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

RAN1#120bis

Agreement

In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion.

According to RAN1, NES-CellId is defined by PhysCellId and ARFCN-ValueNR. Therefore, to address the “FFS Whether the UL-WUS configuration applies to one cell or several cells” that was raised in [6], we propose the following:

Include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config.

- => P1 is not proceeded.

## 2.2 P6 in R2-2504037

There is another issue with the pasted text regarding OD-SIB1 request behavior in running CR to TS 38.331. According to the legacy UE behavior, if the UE receives SI change notification, the UE will receives SIB1 according to the CORESET#0 configuration parameters provided in MIB of the CD-SSB [8]. As for an OD-SIB1 cell, RAN2 has previously agreed that SIB1 update in OD-SIB1 cell also relies on SI change notification [4][5]:

RAN2#126 (202405) SIB1 acquisition upon SIB change notification in NES cell Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell. RAN2#127 (202408) OD-SIB1 Validity: 1.Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

However, according to the running CR to TS 38.331 [6], if the OD-SIB1 UE receives short message notifying SI change in the OD-SIB1 cell, the UE will be requesting OD-SIB1, rather than monitoring SIB1 according to the OD-SIB1 request configuration, which is against with the previous RAN2 agreement. Because the kssb is indicating SIB1 is not scheduled in the cell and the UE will request OD-SIB1:

5.2.2.2.2 SI change indication and PWS notification If the UE receives a Short Message, the UE shall: 1> if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceSIB1 and searchSpaceOtherSystemInformation on the active BWP or the initial BWP: 2> immediately re-acquire the SIB1; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6: 3> acquire SIB6, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7: 3> acquire SIB7, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8: 3> acquire SIB8, as specified in clause 5.2.2.3.2, immediately; NOTE: In case SIB1, SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB1, SIB6, SIB7, or SIB8. 1> if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period; 1> if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary. 5.2.2.3 Acquisition of System Information 5.2.2.3.1 Acquisition of MIB and SIB1 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: (Note: legacy UE behavior when it receives SI change notification) 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. (Note: OD-SIB1 UE behavior when it receives SI change notification in the OD-SIB1 cell) 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5.

Observation 3: According to the running CR to TS 38.331, the UE will trigger OD-SIB1 request when it receives SI change notification if kssb indicates that SIB1 is not scheduled in the cell, which is against previous RAN2 agreement “Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.”.

Therefore, we propose RAN2 to discuss how the OD-SIB1 UE receives SIB1 if the UE receives SI change notification and the kssb is indicating SIB1 is not scheduled in the cell. Our understanding is that the UE should use the CORESET#0 configuration parameters provided by the OD-SIB1 request configuration to receives SIB1. If RAN2 does not share this understanding or cannot reach a consensus on this issue, we suggest to send LS to RAN1 if needed.

Proposal 6: RAN2 to discuss how the OD-SIB1 UE receives SIB1 if the UE receives SI change notification and the kssb is indicating SIB1 is not scheduled in the cell (e.g. the UE uses the CORESET#0 parameters in OD-SIB1 request configuration to receive SIB1), considering the UE behavior will be falsely requesting OD-SIB1 according to the running CR to TS 38.331. Send LS to RAN1 if needed.

It is noticed that it is interpreted that UE should not make OD-SIB1 request. The below is the current RRC CR and UE checks first if SIB1 is broadcasted and if not it will make the request:

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3> if the UE has a stored valid version of od-SIB1-Config for this cell:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5> if SIB1 is not broadcasted

6> perform the actions as specified in clause 5.2.2.3.3x;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

If the network sends the short message, network can also start to provide SIB1 and the current RRC procedure should work.

RAN2 to consider the case when network send a short message and whether the current RRC procedure where UE checks for SIB1 before requesting it is enough.

=> Keep RRC CR as it is for the short message and UE behaviour

## 2.3 FFS on DownlinkConfigCommonSIB

- FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions.

One suggestion on how to handle this:

pagingAdaptation-NS

Number of paging occasions per paging frame for paging adaptation. The UE supporting paging adaptation ignores this field in RRC_CONNECTED.

pagingAdaptationNAndPagingFrameOffset

Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. The UE supporting paging adaptation ignores this field in RRC_CONNECTED.

Another wording suggestion to above is

“UE supporting paging adaptation ignores this field in RRC_CONNECTED and uses ns and nAndPagingFrameOffset instead when monitoring paging occasions.” so that the proposed text (in red) above is not interpreted as if a UE supporting paging adaptation is not required to monitor paging occasions when in connected mode.

Discussion:

- FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging

- => Agreed

The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- => Agreed

The maximum offset value is extended to 32 radio frames.

- => Agreed

The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- => Agreed

Discuss whether the maximum offset value for firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames

- at symbol level

- at slot level

- by restricting the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO field parameter for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N)

- by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

- FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

Discuss if a new pei-ConfigBWP IE is introduced for Rel-19 UEs that support adaptive paging, i.e., if a new PEI search space is needed.

- => A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging

Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging

- => Agreed

The maximum number of PEI occasion per paging frame is extended to 8.

- => The maximum number of PEI occasion per paging frame is not extended to 8.

The maximum offset value is extended to 32 radio frames.

- => Agreed

Discuss how to extend the maximum offset value for firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames.

For Rel-19 PEI configuration, introduce the following parameters: -po-NumPerPEI-r19, -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19,  -firstPDCCH-MonitoringOccasionOfPEI-O-R19

For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19

# 3	Conclusion

- Include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config.

- => P1 is not proceeded.

RAN2 to consider the case when network send a short message and whether the current RRC procedure where UE checks for SIB1 before requesting it is enough.

=> Keep RRC CR as it is for the short message and UE behaviour

Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging

- => Agreed

The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- => Agreed

The maximum offset value is extended to 32 radio frames.

- => Agreed

The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- => Agreed

- NOT discussed:

Discuss whether the maximum offset value for firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames

- at symbol level

- at slot level

- by restricting the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO field parameter for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N)

- by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

- FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

Discuss if a new pei-ConfigBWP IE is introduced for Rel-19 UEs that support adaptive paging, i.e., if a new PEI search space is needed.

- => A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging

Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging

- => Agreed

The maximum number of PEI occasion per paging frame is extended to 8.

- => The maximum number of PEI occasion per paging frame is not extended to 8.

The maximum offset value is extended to 32 radio frames.

- => Agreed

- NOT discussed

Discuss how to extend the maximum offset value for firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames.

For Rel-19 PEI configuration, introduce the following parameters: -po-NumPerPEI-r19, -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19,  -firstPDCCH-MonitoringOccasionOfPEI-O-R19

For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19