3GPP TSG-RAN WG2 Meeting #130		R2-2504921

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Offline summary on the expire of target cell TAT during RAHC-less CLTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

- Introduction

This paper aims to capture the offline discussion summary on the below issue:

3. How to handle if TAT of the target cell expires during RAHC-less CLTM execution? R2-2503931 Remaining Open issues for CLTM NEC discussion Rel-19 NR_Mob_Ph4-Core Proposal 2 (modified). In case of timeAlignmentTimer associated with PTAG expiry during RACH-less LTM cell switch, discuss the following options: - Opt1) the UE terminates the CLTM cell switch procedure - Opt2) the UE fallbacks from RACH-less LTM to RACH-based C-LTM - Opt3) the UE does nothing (i.e. count on T304). [LG, CATT, Apple, Lenovo, Huawei]: Understand this issue can also happen in R18 LTM and we count on T304. Support option 3. [Samsung, Qualcomm, Ericsson, MediaTek, Nokia, Rakuten, Ofinno, IDC]: There is difference between R18 LTM and R19 C-LTM. In R18 LTM, TAT timer starts upon the reception of LTM cell switch command while in R19, TAT timer starts much earlier so there would be more chance it expires early. Also note the current running MAC CR is already somewhat different compared to R18 MAC. [Vivo]: To the current running CR, the UE checks based on whether TAT runs when the first CG resource is available, which is different in R18. [Samsung]: Note in R18 discussion, some companies understand it can be controlled by NW, e.g. TAT is set as similar value of T304, however it cannot be applied to CLTM case. Comeback in CB session (Vivo wants to have more time for offline discussion).

- Discussion

  - In case of before the first transmission

Upon CLTM execution, before first transmission, there are two options as below:

- Option A: (Current MAC running CR) If TA is valid (TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less will be performed. The corresponding TP is as below in current running CR.

1> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 2> acquire the Target Configuration ID from upper layers, if any, for the satisfied L3 measurement based event; 2> if the ltm-Candidate-TimeAlignmentTimer associated with the Target Configuration ID is running in the first available CG occasion for initial CG transmission according to clause 5.8.2 in case two TAGs are not configured for the CLTM candidate cell: 3> process the stored Timing Advance Command associated with the ltm-Candidate-TimeAlignmentTimer (see clause 5.2); 3> consider the RACH-less CLTM cell switch to be ongoing;

Offline information: following companies support this option A: [NEC, Apple, vivo, ASUSTEK, MTK, OPPO, QC, ZTE, Ericsson, Xiaomi, LG]

- Option B: (MAC running CR need update) If TA is valid (TAT timer is running), and there is CG resource configured, RACH-less will be performed. (i.e. UE doesnot need to check TAT in the first CG occasion). The corresponding TP is as below in current running CR.

1> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 2> acquire the Target Configuration ID from upper layers, if any, for the satisfied L3 measurement based event; 2> if the ltm-Candidate-TimeAlignmentTimer associated with the Target Configuration ID is running in the first available CG occasion for initial CG transmission according to clause 5.8.2 in case two TAGs are not configured for the CLTM candidate cell: 3> process the stored Timing Advance Command associated with the ltm-Candidate-TimeAlignmentTimer (see clause 5.2); 3> consider the RACH-less CLTM cell switch to be ongoing;

Offline information: following companies support this option B: [Samsung, Nokia, LG(if option 2 is agreed)]

After offline with some companies, rapporteur suggests to follow the clear majority:

Proposal 1: Upon CLTM execution, before the first transmission, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

  - In case of after the first transmission

During CLTM is ongoing, after the first transmission, there are below three options:

- (option 1) If TAT timer expires while rachless LTM is ongoing, UE terminates the CLTM cell switch procedure. //supporting companies: [NEC1st, vivo]

  - The subsequent behaviour is to perform re-establishment: cell selection, and RACH-based LTM.

- (option 2) If TAT timer expires while rachless LTM is ongoing, fallback to RACH-based CLTM.// supporting companies: [Samsung, Nokia, NEC2nd, CMCC, Apple 2nd, ASUSTEK, ZTE, Ericsson, QC, MTK 2nd]

- (option 3) If TAT timer expires while rachless LTM is ongoing, there is no fallback to RACH-based CLTM.  The UE wait for T304 expires. // supporting companies: [Apple, OPPO, vivo2nd, ASUSTEK2nd, MTK, Xiaomi 1st, CATT, LG]

  - Clarify: UE just waits. And there is no UL tx as TAT expires.

  - Clarify: no spec change comparing with option 1.

- Based on the offline discussion, Rapporteur suggests to try option 2. If we can not conclude that, we will continue to discuss option 2 and 3.

Proposal 2: RAN2 to discuss the below options:

- Option 2 (Fallback): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- Option 3 (HO failure): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, there is no fallback to RACH-based CLTM. The UE waits for T304 expires.

Rapporteur suggests to try option 2 during CB session.

- Conclusion

In this contribution, we discuss the case that TAT of target cell expires during RAHC-less CLTM. Based on the discussion, we have the following proposals:

Proposal 1: Upon CLTM execution, before the first transmission, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

Proposal 2: RAN2 to discuss the below options:

- Option 2 (Fallback): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- Option 3 (HO failure): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, there is no fallback to RACH-based CLTM. The UE waits for T304 expires.

Rapporteur suggests to try option 2 during CB session.