3GPP TSG-RAN WG2 Meeting #130	R2-2504706

Malta, May 19th – 23rd, 2025

Agenda item:	8.5.1

Source: 	InterDigital (Rapporteur)

Title: 	[AT130][106][NES] Open issues for 38.321 CR (InterDigital)

Document for:	Discussion and Decision

# Introduction

This document collects the comments received during the following offline discussion on the MAC open issues for the R19 NES work item:

* [AT130][106][NES] (IDC)

Scope: Discuss and attempt to make conclusions on the following issues:

-        Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

Intended outcome: Discussion summary in R2-2504706.

Deadline: 30min f2f offline discussion

# MAC Issue 2: availability of additional PRACH resources

From the set of open issues identified in the pre-meeting email discussion for the MAC CR, the following open issue is to be discussed in this offline:

-        Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

Relevant proposals for issue 2:

R2-2504420	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 3: (MAC-2)

At least, the following aspects should be captured in MAC spec. for additional PRACH resources.

- MAC layer would apply new PRACH mask configured by RRC to additional PRACH resources

- MAC layer can use additional PRACH resources when lower layer indicates that they are available until indicated that they are unavailable.

Discussion:

Conclusion:

[Proposal 1:	At least, the following aspects should be captured in MAC spec. for additional PRACH resources.

- MAC layer would apply new PRACH mask configured by RRC to additional PRACH resources

- MAC layer can use additional PRACH resources when lower layer indicates that they are available until indicated that they are unavailable.]

R2-2504279	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2:	From a MAC perspective, additional PRACH resources are considered available if a PDCCH activation indication is received by lower layers; otherwise not considered available.

Text proposal:

### 5.1.1c Availability of the set of Random Access resources The MAC entity shall for each set of configured Random Access resources: 1> if eRedCap is set to true for a set of Random Access resources: 2> consider the set of Random Access resources as not available for a Random Access procedure for which eRedCap is not applicable. 1> if redCap is set to true for a set of Random Access resources: 2> consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable. 1> if smallData is set to true for a set of Random Access resources: 2> consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT by MO-SDT as specified in TS 38.331 [5]. 1> if NSAG-List is configured for a set of Random Access resources: 2> consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List. 1> if msg3-Repetitions is set to true for a set of Random Access resources: 2> consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable. 1> if msg1-Repetitions is set to true for a set of Random Access resources: 2> if Msg1 repetition is not applicable to the current Random Access procedure; or 2> if the set of Random Access resources is not associated with any of the Msg1 repetition number that is applicable to the current Random Access procedure: 3> consider the set of Random Access resources as not available for the Random Access procedure. 1> if a set of Random Access resources is not configured with FeatureCombination: 2> consider the set of Random Access resources to not associated with any feature. 1> if a set of Random Access resources is configured with [RRC name TBD: additional PRACH resources]: 2> if PDCCH addressed for the P-RNTI is received from lower layers indicating the availability of the Random Access resources; or 2> if a PDCCH order is received from lower layers with the [TBD DCI field name] indicating the availability of the Random Access resources: 3> consider the set of Random Access resources as available.

Discussion:

Conclusion:

[Proposal 2:	From a MAC perspective, additional PRACH resources are considered available if:

- PDCCH addressed for the P-RNTI is received from lower layers indicating the availability of the Random Access resources; or

- PDCCH order is received from lower layers with the [TBD DCI field name] indicating the availability of the Random Access resources]

R2-2503553	Adaptation of common signal or channel 	Fujitsu	discussion	Rel-19 Netw_Energy_NR_enh-Core

Proposal 4: RAN2 to discuss how to handle the RA resource selection when there are different preamble assignments for additional PRACH resource configuration and legacy PRACH resource configuration.

Relevant agreements:

- RAN2#129: “Will follow legacy mechanism regarding how to select RACH resource”.

- RAN1#120: “Separate configuration of number of SSB per RO is supported”

- RAN1#120bis: “Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported: CB-PreamblesPerSSB”

Discussion:

Conclusion:

Proposal 3:	TBD

Other proposals (If time allows), not directly related to issue 2

R2-2503712	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 9: If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

R2-2503945	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Discussion:

Conclusion:

Proposal 4:	TBD

# Conclusion

After the offline discussion, it is proposed to agree to the following:

# References

[1] R2-2504282, “Running 38.321 CR for network energy saving”, InterDigital

[2] R2-2504281, “Summary of Comments to 38.321 CR for NES”, InterDigital