3GPP TSG-RAN WG2 Meeting #130	R2-2504718

Malta, May 19th – 23rd, 2025

Agenda item:	8.5.4

Source: 	OPPO (Rapporteur)

Title: 	[AT130][112][NES] (OPPO)

Document for:	Discussion and Decision

- Introduction

This document collects the comments received during the following offline discussion:

[AT130][112][NES] (OPPO)

Scope: Discuss whether there is further specification impact to support feature and feature combination, and how to solve the issue if any.

Intended outcome: Discussion summary in R2-2504718

Deadline: Comeback in CB session. 30min f2f offline discussion.

- Discussion

The issue comes from the following agreement

R2-2501088	Common signal/channel adaption	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: RACH partitioning with all the features, i.e. RedCap, SDT, Slicing, and CE, and feature combinations, are supported for PRACH adaption in time domain.

[Apple]: We have two discussion points. 1) RACH adaptation is not modelled as RA feature(s), 2) Whether additional RACH resource can be applied to other existing feature(s). Seems all companies agreed with 1) [LG]: For 2), consider at least preamble partitioning using common RACH configuration should be supported. [OPPO, LG, ZTE, Apple]: Agree with 1). [Session chair]: What would be additional spec impact to support 2)? [Samsung]: No RAN2 spec impact is foreseen. With additional RACH resource, whether to configure legacy RACH resource or additional RACH resource is up to NW. Even for ASN.1, we can reuse what we have. [Ericsson]: Agree with Samsung. Do not see the need of the NW restriction.

From R2 perspective, RACH adaptation is not modelled as RA feature(s).

From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

Observation-1: R2 excluded the support of CE based feature when it comes to RACH partitioning for additional RACH.

And then based on the latest R2 agreement

R2-2503712	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

•	Alt-1: addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

•	Alt-2: addl-RACH-Config-Adaptation is placed under rach-ConfigGeneric

[Nokia, Ericsson]: Prefer Alt-2 since rach-ConfigGeneric is applied to all cases. [LG, OPPO]: It doesn’t make sense to include ssb-perRACH-OccasionAndCB-PreamblesPerSSB under rach-ConfigGeneric. [Apple]: RAN2 already agreed in some cases (e.g. IAB), RACH adaptation is not applied, but with alt-2, it can be applied to them as well if we follow the current spec. [Ericsson]: Understand the raised issues for Alt-2, however consider it’s not complicated to fix it. This decision may impact CFRA discussion. [OPPO]: We need this decision first, otherwise it will be difficult to make progress on CFRA. [Samsung]: Do not see any blocking issue with this agreement even for CFRA. If needed, additional RO configuration can be included in CFRA.

- Alt-1: LG, Samsung, OPPO, Apple, Qualcomm, CATT, Vivo, Huawei

- Alt-2: Nokia, Ericsson, NTTDCM, ZTE

addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

Since additional RACH is to be configured within RACH-ConfigCommon, together with legacy RACH, while there is only one feature(-combination) based partitioning parameter in the RACH-ConfigCommon, it is not clear whether/how to apply this to additional RACH.

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

<Removed>

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]]

}

Observation-2: It is not clear how for a same featureCombinationPreamblesList-r17 to apply to two different RACH resources in a same RACH-ConfigCommon, i.e., legacy and additional RACH, given that the former allows partition of CE while the latter one disallow.

So would like to check companies view on that.

Option-1a: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 revisit the conclusion so that CE is also applicable to additional RACH. And LS to R1 to inform conclusion on that to check if any concern.

Option-1b:  A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, so clarify in the spec that the preambles allocated to feature of CE is not usable for the ROs of additional RACH in the RACH-ConfigCommon.

Option-2: Two different featureCombinationPreamblesList-r17/ featureCombinationPreamblesList-v19xy, while the former is used for legacy RACH, and the latter is for additional RACH.

| Company | Which Option? | Comment |
| --- | --- | --- |

Discussion:

Conclusion:

Proposal 1:	TBD

- Conclusion

After the offline discussion, it is proposed to agree to the following:

- References

[1] R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core