3GPP TSG RAN WG2#130	R2-2504961

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121	R1-2504888

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	Reply LS on LP-WUS in RRC_CONNECTED

Response to:	R1-2503616/R2-2503202

Release:	Rel-19

Work Items:	NR_LPWUS

Source:	RAN WG1

To:	RAN WG2

CC:

Contact Person:

#### Name:	Shinya Kumagai

E-mail Address:	shinya.kumagai.yw@nttdocomo.com

Attachment:

- Overall Description:

RAN1 would like to thank RAN2 for the LS on LP-WUS in RRC_CONNECTED in R1-2503616/R2-2503202.

Related to the information/questions in the LS, in RAN1#121, RAN1 made following conclusion and agreements:

LP-WUS monitoring Agreement: As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS. Agreement: As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified - LR operation is the UE operation for LP-WUS monitoring - MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode Conclusion From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity, - It is up to RAN2 to further discuss and finalize the specification support, if any. UE time offset preference Agreement: For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types - V1=5ms - V2=13ms - V3=37ms Agreement: For the UAI of preferred time offset for LP-WUS monitoring in RRC connected mode, - The candidate values are same as those for the UE capability of minimum time gap, i.e., {V1=5, V2=13, V3=37}ms - The reported UAI value is equal to or longer than the minimum time gap reported by UE capability for a UE Note: There is no change to the RAN1 agreed definition for UAI

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully requests RAN2 take the above replies into account during the future work

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #122	25th – 29th August 2025	Bengaluru, IN

TSG-RAN WG1 Meeting #122bis	13th – 17th October 2025	Prague, CZ