3GPP TSG-RAN WG2 Meeting #131	R2-2505071

Bangaluru, India, 25-29 August 2025

Agenda item:	8.7.4

Source:	Qualcomm Incorporated

Title:	Discussion on DSR enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the remaining issues on DSR.

# Discussion

[MAC-1]	Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

We acknowledge that this scenario can happen, especially if one of the reporting thresholds is higher than the triggering threshold. However, we do not consider delay reporting data alone in the above scenario is worth reporting by DSR, because they are not delay critical and can be reported by BSR. It makes more sense to keep the current agreement that DSR is cancelled once there is no more delay critical data or all delayed critical data has been reported.

Proposal 1.	[MAC-1] Keep the current agreement that a pending DSR is canceled once the UE no longer has delay critical SDUs or all delay critical SDUs have been reported. No additional conditions are needed.

[MAC-2]	Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

We acknowledge that this scenario can happen. For example, a BSR first is triggered, then a DSR is triggered. If the time gap between the BSR and the DSR is long enough, then it is possible that all the buffer status up to (and including) the last event that triggered the BSR, can be reported by DSR.

In such cases, it indeed seems wasteful to send both BSR and DSR MAC CEs in the same MAC PDU. As DSR is more critical than BSR, BSR thus can be omitted, and the pending BSRs triggered prior to the MAC PDU assembly can be canceled.

This cancelation does not need to be mandatory, to avoid additional implementation complexity, e.g. when the DSR is triggered very close to the MAC PDU assembly and UE needs to cancel the BSRs at very last minute.

Proposal 2.  [MAC-2] It is up to UE implementation whether to cancel a BSR when all the buffer status up to the last event that triggered the BSR is reported by a DSR MAC CE.

[UE capability-1] Whether Rel-18 DSR is a pre-requisite for Rel-19 DSR

We first note that network can easily implement the Rel-18 UE behavior through a simple configuration of Rel-19 DSR. More specifically, it only needs to configure one reporting threshold for an LCG and set this reporting threshold the same as the triggering threshold. Therefore, from network’s perspective, it does not make any difference whether UE supports Rel-19 only or both.

However, it is simpler for UE implementation and IoT test if Rel-18 is not required as a pre-requisite of Rel-19. That would allow UE to focus on a single implementation of the reporting procedure and MAC CE format.

Our view therefore is that it is more desirable not to require any dependency between the two releases.

Proposal 3.	[UE capability-1] Support of Rel-18 DSR is not a prerequisite for support of Rel-19 DSR.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	[MAC-1] Keep the current agreement that a pending DSR is canceled once the UE no longer has delay critical SDUs or all delay critical SDUs have been reported. No additional conditions are needed.

Proposal 2.  [MAC-2] It is up to UE implementation whether to cancel a BSR when all the buffer status up to the last event that triggered the BSR is reported by a DSR MAC CE.

Proposal 3.	[UE capability-1] Support of Rel-18 DSR is not a prerequisite for support of Rel-19 DSR.