3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505089

Bengaluru, India, 25th – 29th Aug, 2025

Source:              CATT

Title:	             Leftover Issues on Random Access in SBFD

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In the past RAN2 meetings, random access in SBFD were discussed and reached many agreements, but there are some leftover issues, listed below:

- [MAC-1] How to handle the Msg1 repetition number in case of RO type switch?

- [MAC-2] When CFRA fallbacks to CBRA, whether SBFD-aware UE can use the same RO type indicated in CFRA only when the RACH resources for the same RO type is provided for CBRA?

- [MAC-3] How to derive the SBFD RO locations for CFRA?

- [MAC-4] Whether RA parameter initialization is needed after RO type switch?

- [MAC-5] Whether power ramping offset compensation is needed after RO type switch?

- [RRC-6] Whether to support SBFD for LTM?

- Whether early identification in Msg3 needs to be supported when legacy RO is selected?

- Whether RA-RNTI collision needs to be solved?

- Whether the working assumption of RO type selection order can be confirmed?

Discussion

[MAC-1] Msg1 repetition number handling in case of RO type switch

Regarding to the RO type fallback, in RAN2#130 meeting, the following agreements were reached:

Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

Based on the above agreements, there are two leftover issues regarding to the msg1 repetition number in case of RO type switch:

- Issue 1: Whether UE is allowed to switch to an RO type that is configured with a higher Msg1 repetition number if the same is not available?

- Issue 2: After RO type switch, how to determine the applied Msg1 repetition number?

For Issue 1, allowing UE to switch to an RO type that is configured with a higher Msg1 repetition number associated with the same feature combination can increase the RA successfully probability. There is no strong motivation to prohibit it.

Proposal 1: [MAC-1] UE is allowed to switch to an RO type that is associated with the same feature combination and configured with a higher Msg1 repetition number if the same Msg1 repetition number is not available.

For issue 2, if Proposal 1 is agreed, after RO type switch, UE is allowed to select a next higher Msg1 repetition number within the same feature combination. But if UE already reached the highest Msg1 repetition number before RO type switch, RO type switch is also allowed and the UE can continue use the current Msg1 repetition number.

Proposal 2: Once both the conditions of RO type switch and Msg1 repetition number fallback are met, UE shall select the next higher Msg1 repetition number after RO type switch if the highest Msg1 repetition number on the new RO type has not been reached; otherwise, select the same Msg1 repetition number.

[MAC-2] CFRA to CBRA fallback

During the email discussion of MAC open issues, the email rapporteur raised one open issue that when CFRA fallbacks to CBRA, whether SBFD-aware UE can use the same RO type indicated in CFRA only when the RACH resources for the same RO type is provided for CBRA?

Regarding to this issue, we share the same view as rapporteur, it is quite strange that gNB chooses not to configure SBFD for CBRA but to configure UE to perform CFRA in SBFD symbols. This issue can be solved by smart gNB implementation. No MAC spec modification is needed.

Proposal 3: [MAC-2] No MAC spec impact is needed to address the issue that when CFRA fallback to CBRA but the RACH resources of the RO type indicated in CFRA is not provided for CBRA.

[MAC-3] SBFD RO location derivation in CFRA

During the email discussion of MAC open issues, one company pointed that when NW indicates RO type as SBFD RO in CFRA, but it is unclear how to derive the locations of SBFD RO. This issue is tightly associated with MAC-2. In our understanding, if the NW indicates RO type as SBFD RO for CFRA, there should be SFBD RACH configuration configured for CBRA. This can be ensured by smart gNB implementation. If companies have concern on it, the solution suggested during the email discussion can also be acceptable that is to clarify that NW can indicate RO type as SBFD RO for CFRA only if an SBFD RACH configuration is configured for CBRA. This can be implemented in TS38.331 as a NOTE or field description.

Proposal 4: [MAC-3] NW can indicate the RO type as SBFD RO for CFRA only when SBFD RACH configuration is provided for CBRA. This can rely on gNB implementation. But if companies have concern, this restriction can be added as a NOTE or added in field description.

[MAC-4] Parameter initialization after RO type switch

For 2-step to 4-step fallback, parameter initialization should be performed after RA type switch, the corresponding description is as below:

4> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 5> set the RA_TYPE to 4-stepRA; 5> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

For RO type switch, whether parameter initialization should be performed can be discussed for RACH configuration option 1 and RACH configuration option 2 separately:

- For RACH configuration option 1, based on the following RAN1 agreements and conclusions, the preambleReceivedTargetPower is different between additional RO and legacy RO. Hence, when RO type switch, preambleReceivedTargetPower should be initialized.

Agreements in RAN1#119: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority Conclusion in RAN1#120: There is no RAN1 consensus on the following proposal: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, - also support separate powerRampingStep and powerRampingStepHighPriority for additional-ROs. Conclusion in RAN1#120bis: For RACH configuration Option 1, there is no consensus in RAN1 to support separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs under the same FeatureCombinationPreambles configuration and it’s up to RAN2 to decide whether to support separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs.

- For RACH configuration option 2, since RACH-ConfigCommon is separately configured for legacy RO and additional RO, all parameters can be configured separately. Hence, when RO type switch, all the parameters can be initialized.

Proposal 5: [MAC-4-1] After RO type switch, RA parameter initialization should be performed.

Proposal 6: [MAC-4-2] For RACH configuration option 1, after RO type switch，preambleReceivedTargetPower shall be initialized. And for RACH configuration option 2, all parameters configured in RACH-ConfigCommon shall be initialized.

[MAC-5] Power offset for RO type switch

If RACH configuration option 2 is used, power ramping steps for additional RO and legacy RO are configured separately. This may result different power ramping step for different RO types. Hence, some companies suggest introducing power offset to compensate the power ramping step difference between the two RO types when RO type is switched. Regarding to the issue, RAN1 has already discussed it in RAN1#121 and reached the following conclusion:

Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference.

Based on the above conclusion, RAN1 has no consensus to support it. Since, it only impacts RAN2, this topic can be re-discussed in RAN2.

In TS38.321, when UE performs 2-step RA to 4-step RA fallback, the power offset was considered. The corresponding description is as below:

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

For RO type switch, it is natural to reuse this mechanism if the power ramping steps for additional RO and legacy RO are different. Two power offset can be introduced in RAN2 spec, one for additional RO to legacy RO switch and another one for legacy RO to additional RO switch.

Proposal 7: [MAC-5] Two power offsets for RO type switch shall be introduced if the power ramping steps for additional RO and legacy RO are different, one for additional RO to legacy RO switch and another one for legacy RO to additional RO switch.

[RRC-6] Whether to support SBFD for LTM?

In RAN2#127 meeting, regarding to the supported RACH triggers, the following working assumption was reached:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Furthermore, in RAN2#129bis meeting, the above working assumption was discussed and reached the following agreement:

Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

After RAN2#130meeting, this issue was further discussed in the email discussion of [Post130] [216]. And the following was proposed after email discussion:

[Proposal for RRC-6] (Only) support RACH-based LTM cell change in SBFD symbols [10/11]. Add RO type indication in LTM cell switch command MAC CE.

For RACH-based LTM cell switch, the latency is shown as below:

Figure-1 Mobility Latency for RACH-based LTM cell switch

Based on the above figure, it is obvious that if RA is allowed in SBFD symbols, it can reduce both the UL synchronization latency and the interruption time.

Furthermore, for RACH-based LTM cell switch, since RACH-ConfigDedicated already contained the RO type, there is no spec impact in RAN3 and only one RO type indication in LTM cell switch command MAC CE is enough. Hence, we agree with the proposal reached in the email discussion.

Furthermore, considering Rel-19 LTM is still under discussion, hence, it is suggested that our discussion mainly based on Rel-18 LTM. The combination of Rel-19 LTM and SBFD can be postponed similar as the treatment for Rel-19 NES and Rel-19 SBFD.

Proposal 8: [RRC-6] Support RACH-based LTM cell switch in SBFD symbols and RO type indication in LTM cell switch command MAC CE specified in Rel-18. The combination of Rel-19 LTM and SBFD can be postponed.

Whether early identification is needed when legacy RO is selected?

In RAN2#127bis meeting, the following agreements were reached:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreements, it is obvious that if additional RO is selected, early identification via Msg1 is possible. If legacy RO is selected, since SBFD is not introduced as a feature combination, early identification via Msg1 is not supported.

Based on the above background, what needs to be discussed next is whether early identification via Msg3 needs to be supported in case of legacy RO is selected.

In RAN1#120bis meeting, it was agreed that:

For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure

In RAN1#121 meeting, it was further updated as the following:

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the RACH attempt.

This means Msg3 will not be enhanced. Hence, early identification via Msg3 cannot be supported in case of legacy RO is selected.

Proposal 9: Follow RAN1 agreement, if legacy RO is selected, early identification via Msg3 is not supported.

RA-RNTI collision

In the RAN2#129bis meeting, some companies mentioned the RA-RNTI collision issue in case of the additional RO and legacy RO overlapped. In RAN1#120 meeting, the following conclusion was reached in RAN1:

Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

Based on the above conclusion, RA-RNTI collision can be solved by gNB implementation, no additional specification effort is needed.

Proposal 10: The RA-RNTI collision issue shall be left to gNB implementation, no spec impact.

Working assumption confirmation on the order of RO type selection

Regarding to the RO type selection, in RAN2#129bis meeting, the following working assumption was reached:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

As discussed in the RAN2#129bis meeting, this solution has less spec impacts and complexity compared with other solutions. Hence, suggest RAN2 to confirm the working assumption.

Proposal 11:  Confirm the working assumption that for SBFD-aware UE, the selection of RO type is performed before the selection of the set of Random Access resources.

Conclusion

According to the analysis in section 2, it is proposed:

Msg 1 repetition number handling during RO type switch

Proposal 1: [MAC-1] UE is allowed to switch to an RO type that is associated with the same feature combination and configured with a higher Msg1 repetition number if the same Msg1 repetition number is not available.

Proposal 2: Once both the conditions of RO type switch and Msg1 repetition number fallback are met, UE shall select the next higher Msg1 repetition number after RO type switch if the highest Msg1 repetition number on the new RO type has not been reached; otherwise, select the same Msg1 repetition number.

CFRA/CBRA fallback

Proposal 3: [MAC-2] No MAC spec impact is needed to address the issue that when CFRA fallback to CBRA but the RACH resources of the RO type indicated in CFRA is not provided for CBRA.

SBFD RO location derivation in CFRA

Proposal 4: [MAC-3] NW can indicate the RO type as SBFD RO for CFRA only when SBFD RACH configuration is provided for CBRA. This can rely on gNB implementation. But if companies have concern, this restriction can be added as a NOTE or added in field description.

RACH parameter initialization after RO type switch

Proposal 5: [MAC-4-1] After RO type switch, RA parameter initialization should be performed.

Proposal 6: [MAC-4-2] For RACH configuration option 1, after RO type switch，preambleReceivedTargetPower shall be initialized. And for RACH configuration option 2, all parameters configured in RACH-ConfigCommon shall be initialized.

Power offset for RO type switch

Proposal 7: [MAC-5] Two power offsets for RO type switch shall be introduced if the power ramping steps for additional RO and legacy RO are different, one for additional RO to legacy RO switch and another one for legacy RO to additional RO switch.

Support of LTM in SBFD symbol

Proposal 8: [RRC-6] Support RACH-based LTM cell switch in SBFD symbols and RO type indication in LTM cell switch command MAC CE specified in Rel-18. The combination of Rel-19 LTM and SBFD can be postponed.

Early identification in Msg3

Proposal 9: Follow RAN1 agreement, if legacy RO is selected, early identification via Msg3 is not supported.

RA-RNTI collision

Proposal 10: The RA-RNTI collision issue shall be left to gNB implementation, no spec impact.

Order of RA type selection

Proposal 11:  Confirm the working assumption that for SBFD-aware UE, the selection of RO type is performed before the selection of the set of Random Access resources.

Reference

- R2-2504743 Summary of [AT130][209][SBFD] Proposals to address MAC-2 and MAC-3 (Samsung)