3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505092

Bangalore, India Aug 25th – 29th, 2025

Source:	CATT

Title:	Discussion on A-IoT random access

Agenda Item:	8.2.4

Document for:	Discussion

1.	Introduction

This paper further discusses the random access related issues for Ambient IoT. In details, the following aspects are covered by our contribution,

- CBRA failure detection

- Frequency index in Msg2

Section 2 gives the detailed discussion, and followed by the Section 3 that summarizes the conclusion of this paper.

2.	Discussion

## 2.1	CBRA failure detection (Issue 2-4)

In RAN2#130 post email discussion [1], there is an open issue on CBRA failure detection:

| Subgroup: CBRA procedure related |  |  |
| --- | --- | --- |
| Issue 2-4: CBRA failure detection | Further down selection between option B and C for msg2 monitor window in CBRA. - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. - 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - Status in running CR: the common part of option B and C is captured, while the difference part is in [] and highligted. | To be discussed by company contributions |

The intention is to define a boundary for the device to detect the reception of Msg2. According to the online discussion, the controversy between companies is whether we allow enough reader flexibility along the time domain to confirm the contention resolution for a certain device. From our perspective, the expected reader implementation is to echo the device for contention resolution as early as possible, therefore we prefer option B. If we go with option C, at least we do not see much motivation to make a configurable K value for this purpose. Therefore, we proposed that,

Proposal 1: 	(2-4) RAN2 agrees to support Option B to define the boundary of Msg2 reception, i.e., a device assumes CBRA failure if its expecting Msg2 is not received before the reception of either the next Access Trigger message or the subsequent paging message. If we go with Option C by the kth Access Trigger message or the subsequent paging message, a fixed K value is preferred.

## 2.2	Frequency index in Msg2 (Issue 2-5)

Regarding the content of Msg2, there is an open issue on whether to include frequency index along with the echoed RN16 in Msg2 to reduce collisions of Msg1 between different devices.

| Subgroup: CBRA procedure related |  |  |
| --- | --- | --- |
| Issue 2-5: random ID differentiation in Msg2 | Whether to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - RAN2#130 agreement: Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Status in running CR: not captured yet. | To be discussed by company contributions |

The cases can be summarized as follows (take two devices for example):

- Case 1: the device 1 and device 2 send the same RN16 to the reader in different access occasions, and the reader successfully decodes the two same RN16. If the reader includes one or two same RN16 in the Msg2 associated with the corresponding AS ID, it will confuse the devices 1 and 2 whether or which one echoed RN16 with the associated AS ID is used for itself.

- Case 2: the device 1 and device 2 send the same RN16 to the reader in different access occasions, but the reader only decodes one of them (for example, device 1) due to the fact another one is conflicted with the other devices. In this case, the reader does not know there is another device 2 sharing the same RN16 with the echoed RN16 in the Msg2, which also confuses the device 2 to finish contention resolution.

To solve above issue, we think this can be left to reader implementation with the following reasons,

1. The reason why we use RN16 is that we think this is a large enough number to avoid generating the same RN16 for different devices, as much as possible.

2. Even if above cases occur, for case 1 we can leave it to reader implementation, i.e., the reader does not echo the RN16 in Msg2 if it decodes two same RN16 in different access occasions; for case 2, the reader will echo the RN16 in Msg2 because it is not aware of the device 2 sharing the same RN16 in Msg1 transmission, which will cause the device 1 and device 2 share the same AS ID and Msg3 D2R resource. However, the reader can use the NACK Feedback message to trigger the re-access for device 1 and device 2 due to decoding failure caused by this resource conflict.

In summary, there is no critical problem for both cases based on the reader implementation or the NACK mechanism to trigger re-access.

Even if we introduce frequency index in the Msg2, it still does not solve this issue. This is because the device 1 and device 2 may send the same RN16 with the same frequency shift but with different time-domain resources. At that case, it still confuses the devices which echoed RN_16 is associated with itself.

Based on above arguments, we proposed not to make optimization on this issue. Therefore, we proposed that,

Proposal 2:	(2-5) No need to include frequency index in Msg2.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

CBRA failure detection (Issue 2-4)

Proposal 1: 	(2-4) RAN2 agrees to support Option B to define the boundary of Msg2 reception, i.e., a device assumes CBRA failure if its expecting Msg2 is not received before the reception of either the next Access Trigger message or the subsequent paging message. If we go with Option C by the kth Access Trigger message or the subsequent paging message, a fixed K value is preferred.

Frequency index in Msg2 (Issue 2-5)

Proposal 2:	(2-5) No need to include frequency index in Msg2.

4. References

[1] Draft_A-IoT MAC open issues_v38_Rappv2