3GPP TSG-RAN WG2 Meeting #131		R2-2505130

Bengaluru, India, Aug 25th – 29th, 2025

Source:	vivo

Title:	Discussion on functionality management for RRM measurement event prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN2#130 meeting[1], the following agreements were reached regarding applicability reporting, inference configuration/reporting and performance monitoring for UE-sided model:

Agreements General LCM framework/applicability reporting: - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s), no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Inference configuration: - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. Performance monitoring: 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In this contribution, we will discuss issues related to functionality management for RRM measurement event prediction.

# Discussion

## 2.1 Applicability reporting

At the last RAN2 meeting, it was agreed that the framework for the applicability configuration/reporting of AI-based BM could be reused in AI-based mobility, which supports Option A and Option B-like schemes.

Our understanding is that the framework is common for both RRM measurement prediction and measurement event prediction. Further discussion is needed on how the UE determines the applicability of measurement event prediction functionality.

Figure 2.1-1: Indirect measurement event prediction Figure 2.1-2: Direct measurement event prediction

- For indirect measurement event prediction, measurement event prediction is performed based on RRM measurement prediction. With the RRM measurement prediction, UE can derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Hence, we think the applicability determination of indirect measurement event prediction is the same as RRM measurement prediction, and no need to consider some measurement event-specific parameters, e.g., threshold, hysteresis.

- For direct measurement event prediction, the measurement event occurrence is predicted directly by an AI/ML model. As the measurement event configuration should be aligned between model training and model inference, UE should determine the applicability for specific measurement event types, considering the measurement event-specific parameters, e.g., threshold, hysteresis.

- For RRM measurement event prediction, the UE determines the applicability considering the inference configuration from network:

- For indirect measurement event prediction, the applicability determination is the same as RRM measurement prediction, and no need to consider some measurement event-specific parameters, e.g., threshold, hysteresis.

- For direct measurement event prediction, the applicability determination is based on measurement event-specific parameters, e.g., threshold, hysteresis.

## 2.2 Inference configuration/reporting

RAN2 reached the following agreements about inference configuration:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability; it may contain • PW length (for temporal case A) • Skipping pattern (e.g., SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

Inference configuration

The legacy measurement event configuration contains below information:

- Measurement object, including cell list, timing and frequency locations, etc.

- Report configuration, including event trigger config, e.g., threshold, TTT, etc.

- Measurement identities.

For indirect measurement event prediction, the NW may provide the agreed “inference parameters” to UE as inference configuration, e.g., PW length (for temporal case A).

For direct measurement event prediction, the NW may provide other “inference parameters”, e.g., probability threshold. For direct prediction method, a measurement event could be predicted within an occurrence window starting from current time instance, i.e. t0, and future time instance t1 with a probability as illustrated in Figure 2.2-1.

Figure 2.2-1: occurrence window of direct prediction method

Therefore, we propose:

- For inference configuration of RRM measurement event prediction, the agreed inference parameters (e.g., PW length, skipping pattern) are applicable for indirect measurement event prediction.

- For direct measurement event prediction, additional inference parameters may be needed, e.g., probability threshold.

When to perform inference

At the RAN2#129bis meeting, RAN2 agreed that the UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC, and it is FFS when the UE can perform inference. Regarding the FFS issue for the applicable measurement event prediction, the goal should be to improve mobility performance and UE power saving. Therefore, we think that event-triggered inference should be supported to avoid unnecessary inference.

In our previous simulation [2], it was observed that:

- The inference number of event-triggered inference is much lower than continuous inference.

- The F1 score of event-triggered inference is better than continuous inference.

In addition, during the simulation analysis phase, it is assumed that PW length equals TTT length, i.e., UE triggers inference for measurement event prediction when the entering condition is met. With this approach, the performance gain is significant while the inference effort is acceptable. Therefore, the entering condition being met can be considered as a triggering event for event prediction.

Therefore, we propose:

- Event-triggered inference of measurement event prediction should be supported to reduce inference consumption, e.g., when the entering condition of the event is met.

Inference reporting

The legacy measurement event reporting contains measurement identity and corresponding measurement results.

For indirect measurement event prediction, the UE reports the inference result to the network, and the inference result includes the predicted timing of the measurement event and the corresponding RRM measurement prediction. The network can utilize the predicted timing of the measurement event to perform handover preparation and send the handover command.

For indirect measurement event prediction, the UE provides the inference result to the network, including the probability of event occurrence within the occurrence window.

Therefore, we propose:

- For indirect measurement event prediction, the inference report includes:

- the predicted timing of the measurement event;

- the RRM measurement prediction.

- For direct measurement event prediction, the inference report includes the probability of event occurrence within the occurrence window.

Handover model options

For the system-level simulation, RAN2 discussed two handover model options for measurement event prediction based on intra-frequency temporal domain case A, as below (extracted from [3]):

The handover model is defined to facilitate SLS, where measurement event is predicted based on either intra-frequency temporal domain case A or intra-frequency temporal domain case B. For both cases, network starts with 40ms handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After handover command, 40ms execution duration is assumed. If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event Figure 2.2-2: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time, i.e., t0 measurement event, e.g., A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 2.2-3: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

We understand that the introduction of handover model is only to facilitate the system-level simulation. From the view of specification impacts, the selection of handover model is up to NW implementation. To be specific, for temporal domain prediction case A, the UE needs to report the predicted timing of the measurement event to NW, and it’s up to NW to make HO decision based on either the predicted measurement event or wait for the legacy measurement event report.

- The selection of handover model (e.g., option 1 or 2) of temporal domain case A is up to network implementation, i.e., the network can make HO decision based on either the predicted measurement event or the legacy measurement event.

## 2.3 Performance monitoring

RAN2 reached the following agreements about performance monitoring:

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE-sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

For indirect measurement event prediction, measurement event prediction is performed based on RRM measurement prediction. In the same way, the performance monitoring should be similar to RRM measurement prediction. We may reuse the performance monitoring framework and performance metirc (e.g., RSRP difference) of RRM measurement prediction for indirect measurement event prediction.

- For indirect measurement event prediction, reuse the performance monitoring framework and performance metric (e.g., RSRP difference) of RRM measurement prediction.

For direct measurement event prediction, the inference output is the probability of event occurrence within the occurrence window. We need to consider the performance monitoring method for direct prediction, the potential open issues include:

- What is the performance metric? During the simulation phase, F1 score is the only performance metric to evaluate the accuracy of direct measurement event prediction. However, F1 score is obtained based on statistical results over a long period of time. It’s difficult for the network to perform near-real-time management (e.g., activation/deactivation)

- How to obtain the monitoring data (ground-truth label)? In some cases, the legacy measurement event may be not available if the NW send the handover command in advance, e.g., based on temporal measurement event prediction.

- How to evaluate the probability? Since the inference output is the probability of event occurrence within the occurrence window, it seems not a simple comparation between inference output and monitoring data. .

Above issues are open for both “UE-sided model, NW-sided monitoring” and “For UE-sided model, UE-sided monitoring”. After performance monitoring method is clarified, we can further design the monitoring configuration. Therefore, we propose that:

- For direct measurement event prediction, additional issues on performance monitoring needs to be addressed, e.g.:

- What is the performance metric?

- How to obtain the monitoring data (ground-truth label)?

- How to evaluate the probability in inference output?

## 2.4 Recommendation for normative work

Based on the above discussion, the function management of direct measurement event prediction has a significant difference from indirect measurement event prediction and RRM measurement prediction due to different AI models. Extra effort on LCM is needed for direct measurement event prediction, including applicability reporting, inference configuration/reporting, and performance monitoring. However, most of the LCM for indirect measurement event prediction and RRM measurement prediction are common.

According to our simulation in [4], indirect measurement event prediction shows a better F1 score and advance prediction duration than direct prediction (probability threshold = 80%). In addition, the model of direct measurement prediction may have generalization issue, e.g., only applicable at a specific network configuration.

Taking all of the above factors into consideration, we suggest focusing on the specification of indirect measurement event prediction in Rel-20, and extra impact specific for direct measurement event prediction should be excluded.

- For measurement event prediction, support of indirect measurement event prediction is recommended for normative work, i.e., extra impact specific for direct measurement event prediction is excluded.

# Conclusion

In the contribution, we have the following proposals:

Applicability reporting

- For RRM measurement event prediction, the UE determines the applicability considering the inference configuration from network:

- For indirect measurement event prediction, the applicability determination is the same as RRM measurement prediction, and no need to consider some measurement event-specific parameters, e.g., threshold, hysteresis.

- For direct measurement event prediction, the applicability determination is based on measurement event-specific parameters, e.g., threshold, hysteresis.

Inference configuration/reporting

- For inference configuration of RRM measurement event prediction, the agreed inference parameters (e.g., PW length, skipping pattern) are applicable for indirect measurement event prediction.

- For direct measurement event prediction, additional inference parameters may be needed, e.g., probability threshold.

- Event-triggered inference of measurement event prediction should be supported to reduce inference consumption, e.g., when the entering condition of the event is met.

- For indirect measurement event prediction, the inference report includes:

- the predicted timing of the measurement event;

- the RRM measurement prediction.

- For direct measurement event prediction, the inference report includes the probability of event occurrence within the occurrence window.

- The selection of handover model (e.g., option 1 or 2) of temporal domain case A is up to network implementation, i.e., the network can make HO decision based on either the predicted measurement event or the legacy measurement event.

Performance monitoring

- For indirect measurement event prediction, reuse the performance monitoring framework and performance metric (e.g., RSRP difference) of RRM measurement prediction.

- For direct measurement event prediction, additional issues on performance monitoring needs to be addressed, e.g.:

- What is the performance metric?

- How to obtain the monitoring data (ground-truth label)?

- How to evaluate the probability in inference output?

Recommendation for normative work

- For measurement event prediction, support of indirect measurement event prediction is recommended for normative work, i.e., extra impact specific for direct measurement event prediction is excluded.

# Reference

- RAN2#130_ChairNotes

- R2-2500216 Simulation results of measurement event prediction and SLS, vivo

- R2-2501822 Text proposal of TR 38.744 V0.0.7.

- R2-2501791 Simulation results of measurement event prediction and SLS, vivo