3GPP TSG-RAN WG2 Meeting #131	R2-2505141

# Bangalore, India, 25 – 29 Augugst 2025

Agenda item:		8.11.2

Source:	Xiaomi

Title:	Discussion on RACH in SBFD

Document for:		Discussion and Decision

# Introduction

In RAN2#130 meeting, following was agreed regarding RACH [1]:

- For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion.

- Msg1 repetition number fallback can be supported within SBFD RO.

- Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case.

- For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations.

- The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

- SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

- For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs.

- For PRACH configuration option 1, same featureCombinationPreamblesList configuration is used for both Additional-ROs and Legacy-ROs.

In this contribution, we discuss remaining issues of RACH in SBFD.

# Discussion

## (RRC-1) Network indicates RO types to the UE

In “[Post130][216][SBFD] Running CR for 38.331 (Huawei)” [2], one issue for discussion is how to indicate the RO types to be used. Rapporteur suggests to down-select from the two options below:

Option 1:  Use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type.

Option 2:  Use 1 bit indicating RO type directly (as in the current RRC running CR).

The motivation of Option 1 is for load balancing between SBFD ROs and non-SBFD ROs. However, there are already 3 mechanisms specified for RO type selection in current SBFD running CRs: explicit RO type indication, RSRP based RO type selection (if explicit RO type indication is not provided), and RO type selection based on UE implementation (if neither explicit RO type indication nor RSRP based RO type selection is configured). Therefore load balancing between SBFD ROs and non-SBFD ROs can be achieved with RSRP based RO type selection or UE implementation. There is no need to go with the multiple value option for RO type indication.

Proposal 1: (RRC-1) RO type indication is based on 1 bit as in current RRC running CR.

## (MAC-2) Fallback from CFRA to CBRA

In email discussion “[Post130][217][SBFD] Running CR for 38.321 (Samsung)” [3], open issue MAC-2 is as follows:

Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA?

The related CR text [4] is the condition “3>	determine the next available PRACH occasion from the PRACH occasions of the selected RO type…”. As explained by the rapporteur in the email discussion, RACH resources (e.g. PRACH occasion) is selected in the initialization phase (clause 5.1.1 of TS 38.321) irrespective of CFRA or CBRA is used. The only difference between CFRA and CBRA is the selection of SSB / CSI-RS and associated preamble index, as described in clause 5.1.2 of TS 38.321. Therefore current CR is sufficient to reflect the previous RAN2 agreement.

Proposal 2: (MAC-2) Additional MAC specification changes (compared to the latest version of the MAC running CR) are not needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

## (MAC-4) Parameter initialization after RO type switching

In email discussion “[Post130][217][SBFD] Running CR for 38.321 (Samsung)” [3], open issue MAC-4 is as follows:

[Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale.

[Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale.

Since different RO types have corresponding RACH-ConfigCommon in RACH configuration Option 2, it is straightforward that parameter initialization is needed after RO type switching. For switching from 2-step RA to 4-step RA, parameter initialization is defined in clause 5.1.1a, which can be used as a reference for RO type switching between SBFD RO and non-SBFD RO.

In clause 5.1.1a, parameters PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are initialized, and these parameters can be considered for RO type switching between SBFD RO and non-SBFD RO. In addition, considering the compensation of the power ramping difference for RO type switching (in clause 2.4 below), PREAMBLE_POWER_RAMPING_COUNTER should not be initialized.

Proposal 3: (MAC-4) Parameter initialization is needed after RO type switching in the case of RACH configuration Option 2. At least PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are initialized. PREAMBLE_POWER_RAMPING_COUNTER should be excluded from the initialization.

## (MAC-5) Compensation of the power ramping difference for RO type switching

In email discussion “[Post130][217][SBFD] Running CR for 38.321 (Samsung)” [3], open issue MAC-5 is as follows: “Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.” The following is the related issue description:

In RACH configuration Option 2, power ramping steps for SBFD RO and non-SBFD RO are configured separately by two ‘RACH-ConfigCommon->RACH-ConfigGeneric’, respectively, resulting in possibly different power ramping steps for different RO types. Two companies (OPPO, Apple) suggest to introduce power offset to compensate the power ramping difference between the two RO types, when RO type switching happens, resembling the power offset compensation in 2-step to 4-step RA type fallback. While RAN1 has not achieved consensus on this issue (as shown below), taking into account the potential impact on RAN2 specifications, the Rapporteur deemed it appropriate to initiate a discussion on this topic within RAN2.

RAN1 Chair’s Notes (RAN1#121) … Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference Note: The red part has been added to a previous conclusion from RAN1#120bis.

Power offset to compensate the power ramping difference between the two RO types during RO type switching is necessary, similar to POWER_OFFSET_2STEP_RA to compensate the power ramping difference during the switching from 2-stepRA to 4-stepRA, as from TS 38.321 clause 5.1.1a copied below:

2>	if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure:

3>	set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

Without the power offset compensation, the preamble transmission power after RO type switching cannot be set properly.

Proposal 4: (MAC-5) Power offset is introduced to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching.

# Conclusion

In this contribution, we discuss remaining issues of RACH, and propose the following:

Proposal 1: (RRC-1) RO type indication is based on 1 bit as in current RRC running CR.

Proposal 2: (MAC-2) Additional MAC specification changes (compared to the latest version of the MAC running CR) are not needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

Proposal 3: (MAC-4) Parameter initialization is needed after RO type switching in the case of RACH configuration Option 2. At least PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are initialized. PREAMBLE_POWER_RAMPING_COUNTER should be excluded from the initialization.

Proposal 4: (MAC-5) Power offset is introduced to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching.

# References

[1] R2-2504672, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others"

[2] R2-2505364, Huawei, HiSilicon, "Summary of [Post130][216][SBFD] Running CR for 38.331"

[3] R2-2505560, Samsung, "Summary of the SBFD open issues in MAC"

[4] R2-2505575, Samsung, "Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec"