3GPP TSG-RAN WG2 Meeting #131	R2-2505142

# Bangalore, India, 25 – 29 Augugst 2025

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

# Introduction

In RAN2#130 meeting, following was agreed regarding other aspects of SBFD [1]:

- RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report. No spec change is needed.

- No enhancement is needed for CSI-RS based RLM/BFD/CBD measurements.

- Keep the legacy RLM, BFD and BFR procedure, i.e., no need to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

In this contribution, we discuss RAN2 aspects of SBFD.

# Discussion

In RAN2#129bis meeting, RAN2 sends LS R2-2503191 asking RAN1 whether SBFD and DC can be configured simultaneously. RAN1 reply in R1-2504858/R2-2505015 [2] indicates the following:

- From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

- Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

- It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC.

In “[Post130][216][SBFD] Running CR for 38.331 (Huawei)” [3], for open issue RRC-7 (“whether to support SBFD with NR-DC”), rapporteur recommends the following way forward “RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. Send reply LS to RAN4 (CC RAN1) with this understanding.” We agree with the way forward, and the final decision on whether to support simultaneous configuration of SBFD and DC can be analyzed by RAN4. Meanwhile, RAN2 can analyze potential RAN2 impacts.

A draft LS to RAN4 (Cc RAN1/RAN3) is provided in Annex. The main content is as follows:

RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. RAN2 would like to seek inputs from RAN4 by asking the following question:

Question: Whether RAN4 thinks simultaneous configuration of SBFD and NR-DC can be supported in Rel-19? If yes, what are the impacted RAN4 specifications?

Proposal 1: RAN2 to consider the draft rely LS to RAN4 on simultaneous SBFD and DC configuration in Annex.

As indicated by RAN1, for simultaneous SBFD and NR-DC configuration, SBFD operation is applicable on only one NR TDD carrier. Therefore coordination between MN and SN regarding SBFD configuration is needed. To allow configuration flexibility, it can be assumed that either MN or SN can configure one NR TDD carrier for SBFD operation.

Proposal 2: Up to one TDD carrier can be configured for SBFD operation in NR-DC, and either MN or SN can configure the TDD carrier with SBFD operation.

In RAN3#126 meeting, following was agreed:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

The above agreement has been implemented in RAN3 BL CRs, e.g. R3-253113 to TS 38.423. In legacy NR-DC operation, in messages exchanged between MN and SN on UE configuration (CG-ConfigInfo and CG-Config), the configured MCG cells and SCG cells are provided in servFrequenciesMN-NR and pSCellFrequency / scellFrequenciesSN-NR, respectively. Therefore MN and SN are aware of the SBFD configuration in other node, and further standardization impact is not needed to support the coordination between MN and SN.

Proposal 3: No further RAN2 specification work is needed to support the coordination between MN and SN on SBFD configuration.

# Conclusion

In this contribution, we discuss RAN2 aspects of SBFD, and propose the following:

Proposal 1: RAN2 to consider the draft rely LS to RAN4 on simultaneous SBFD and DC configuration in Annex.

Proposal 2: Up to one TDD carrier can be configured for SBFD operation in NR-DC, and either MN or SN can configure the TDD carrier with SBFD operation.

Proposal 3: No further RAN2 specification work is needed to support the coordination between MN and SN on SBFD configuration.

# References

[1] R2-2504672, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others"

[2] R1-2504858/R2-2505015, RAN1 LS, "Reply LS on simultaneous configuration of SBFD and DC"

[3] R2-2505364, Huawei, HiSilicon, "Summary of [Post130][216][SBFD] Running CR for 38.331"

# Annex Draft reply LS

Title:	[Draft] Reply LS on simultaneous configuration of SBFD and DC

Response to:	R1-2504858 Reply LS on simultaneous configuration of SBFD and DC

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	TBD [to be RAN2]

To:	RAN4

Cc:	RAN1, RAN3

Contact person:	Yujian Zhang

zhangyujian@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# Overall description

RAN2 would like to thank RAN1 for the LS on simultaneous configuration of SBFD and DC in R1-2504858.

RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. RAN2 would like to seek inputs from RAN4 by asking the following question:

Question: Whether RAN4 thinks simultaneous configuration of SBFD and NR-DC can be supported in Rel-19? If yes, what are the impacted RAN4 specifications?

# Actions

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to provide feedback to the above question.

# Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #131bis		13 - 17 Oct 2025					Prague, CZ

TSG RAN WG2 Meeting #132			17 - 21 Nov 2025					Dallas, US