3GPP TSG-RAN WG2 Meeting #131	R2- 2505159

Bengaluru, India, 25th – 29rd Aug 2025

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Remaining issues on event triggered L1 MR

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for event triggered L1 measurement report are discussed.

Discussion

mTRP coexistence

In the [POST130][117] email discussion, whether to allow the coexistence between mTRP and event-triggered L1 MR was discussed.  The key open issue is how the UE determines the “current beam” when evaluating an event. The rapporteur listed four possible rules:

a)	UE should use the best beam of the two “current beams” for event evaluation

b)	It is up to the UE which of the two beams it uses for L1 event evaluation

c)	Network indicates which beam the UE shall use

d)	Specify to use one of the two TCI states (e.g., just use the first indicated TCI state)

Companies provided diverse comments and we also want to discuss why UE cannot use the best beam for event evaluation.

Option a)

As TS 38.133 specified, UE is required to perform measurement and refresh the measurement result for one SSB in one certain measurement period, and UE is not required to measure all serving beam in the same SS-burst set. In an mTRP deployment, different serving beams results may be measured/updated in different SSB periods.

Observation 1: In mTRP scenario, UE is not required to measure all serving beams in the same SSB period.

For option a), it is not clear how does UE define the “best beam of the two current beams” and not clear if it requires UE to re-measure all the measured beam and compared with the new measured beam.

For example, in one typical scenario:

- T=0, UE measures SSB#1

- T=20ms, UE measures SSB#2, Does UE need to re-measure SSB#1 in the same SSB period? Or use the value obtained in T=0 to compare with?

- T=120ms, UE measures SSB#3. Does UE need to re-measure SSB#1 and #2 in the same SSB period? Or use the value obtained in T=0 for SSB#1 and T=20ms for SSB#2 to compare with?

Requiring the UE to measure all serving beams simultaneously is unrealistic, and usually the measured result will be used to compared with the historical measurement result of another serving beam. The fluctuation of beam quality would affect the regularity of mobility.

Also, L1 filtering is up to UE implementation. UE may accumulate or average the power samples obtained in several consecutive SSB periods and provides a general L1 beam quality, rather than always using the fresh measurement result. It is unclear the “best beam” covers L1 filtering or not.

Observation 2: Requiring the UE to measure all serving beams simultaneously is unrealistic, whereas compare fresh results with historical values of other serving beams would affect the regularity of mobility.

Observation 3: It is unclear the “best beam” covers L1 filtering or not.

Therefore, option a) is not preferred in our side if coexistence of mTRP and event-triggered L1 MR is supported.

Option c) and d)

Options c and d both rely on the gNB to pick one of the serving beams and instruct the UE to use that beam for event evaluation; the two options differ only in the format; functionally they are identical.

As noted earlier, the network receives UE beam-quality per measurement period and cannot determine the optimal beam in real time. Moreover, the current signalling design cannot support such dynamic indication, as the TCI configuration is provided via RRC.

In this case, if the network indicates a suboptimal serving beam for comparison, it will lead to more frequent, meaningless handovers and potentially degrade performance (as in fact the serving cell is not that bad). Therefore, although these two options are feasible, their results are suboptimal.

Observation 4: Options c) and d) are feasible but sub-optimal because they can cause unnecessary handovers and performance degradation (if the suboptimal serving beam is selected by network).

Therefore, Option b) should be adopted. This provides the UE with beam selection flexibility without introducing extra measurement complexity or ambiguity.

Proposal 1: Option b) (up to the UE select the serving beams for L1 event evaluation) is adopted for event evaluation for LTM, if the coexistence with mTRP is agreed.

SP CSI-RS measurement for target after LTM cell switch

(Not sure if this belongs to 8.6.2 or 8.6.3)

In the last meeting, RAN2 discussed how to handle the activated semi-persistent CSI-RS resources after LTM cell switch. The following agreements were made:

UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. Postpone target cell case to the next meeting.

Now the discussion point is whether to stop measuring the activated SP-CSI-RS resource of target cell after cell switch.

We see the target cell measurement is very related to the CSI acquisition feature introduced in RAN1.

The motivation of early CSI is to obtain and report target CSI earlier to reduce throughput degradation during mobility. By reporting CSI information earlier, UE throughput can recover quickly after switching.

In RAN1, the following alternatives (cases) are agreed:

Case 1: (Alt-3, as baseline) CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. The report is sent directly to target cell. Case 2: (Alt-2, as optional UE capability) CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. The report is sent directly to target cell.

In case 1 (Alt 3), UE measure the CSI of target cell after handover command. UE sends the CSI report to the new serving cell after UE switches to the new serving cell.

In case 2 (Alt 2), UE could further measure CSI for multiple candidate cell before handover command. UE sends the CSI report (of the new serving cell) to the new serving cell after UE switches to the new serving cell.

In both cases, it’s better for UE to perform a continuous CSI measurement before and after the handover to provide a detailed CSI information. This is consistent with the motivation for introducing early CSI. If the network later want to stop these measurements it can simply send a MAC CE to deactivate the corresponding SP-CSI-RS resources.

Proposal 2: UE continues to measure SP CSI-RS resources of target cell (new serving cell) after the cell switch.

UE capability for the “any beam reporting”

In the email discussion for UE capability [Post130][120][MOB], It was discussed whether to define a per UE capability for the any beam reporting in the MR MAC CE.

It is noted that it is still up to UE implementation to report not-satisfied beam, if network indicate “allowReportAnyBeam”.

In general, there are two options:

Table: Two options on whether to introduce a UE capability for “any beam reporting”

|  | Need a capability? | Network behavior | UE behavior |  |
| --- | --- | --- | --- | --- |
|  |  |  | For not supported UE | For supported UE |
| Option 1 | Yes | Could only indicate allowReportAnyBeam to capable UEs | Not report unsatisfied beam | Up to UE |
| Option 2 | No | Could indicate allowReportAnyBeam to all UEs |  |  |

We can see that the UE’s behavior is consistent in two options, while the only difference is whether network indicates allowReportAnyBeam to UE which is not capable to do that.

Traditionally, networks should not configure a UE with features or parameter values that the UE does not indicate support for, if the UE’s implementation may affect network behavior (e.g., resource allocation or network scheduling.) For example, the network may expect the UE to report more information and therefore reserve more UL resource. Also, extra clarification is needed in the description field to describe the UE behavior to ignore this IE if not supported, which reduce the machine readability.

Therefore, we slightly prefer to add this capability bit.

Proposal 3: A per UE capability is introduced to report the beam not satisfying the event condition in the MR MAC CE.

Conclusion

In this contribution, the following observations and proposals are made:

2.1 mTRP coexistence

Observation 1: In mTRP scenario, UE is not required to measure all serving cell SSBs in the same SSB period.

Observation 2: Requiring the UE to measure all serving beams simultaneously is unrealistic, whereas compare fresh results with historical values of other serving beams would affect the regularity of mobility.

Observation 3: It is unclear the “best beam” covers L1 filtering or not.

Observation 4: Options c) and d) are feasible but sub-optimal because they can cause unnecessary handovers and performance degradation (if the suboptimal serving beam is selected).

Proposal 1: Option b) (up to the UE select the serving beams for L1 event evaluation) is adopted for event evaluation for LTM, if the coexistence with mTRP is agreed.

2.2	SP CSI-RS measurement for target after LTM cell switch

Proposal 2: UE continues to measure SP CSI-RS resources of target cell (new serving cell) after the cell switch.

2.3	UE capability for the any beam reporting

Proposal 3: A per UE capability is introduced to report the beam not satisfying the event condition in the MR MAC CE.

References

- [POST130][120][MOB] (CATT)

- Report from [POST130][117][MOB] (Nokia)

- R2_130_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM