3GPP TSG-RAN WG2 Meeting #131	R2- 2505160

Bengaluru, India, 25th – 29rd Aug 2025

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Remaining issues in conditional LTM

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for conditional LTM are discussed.

Discussion

  - TA handling of LTM CSC when CLTM TAT running

The LTM and CLTM co-existence is agreed to be supported. The following editor’s Note is kept for the LTM and CLTM co-existence case when LTM CSC MAC CE is received while the TAT for the indicated candidate is running (MAC-14):

- 5.18.35 (Enhanced) LTM Cell Switch Command <<omit unrelated part>> The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 1> if the MAC entity receives an Enhanced LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered, the Target Configuration ID and the NCC value included in the Ehanced LTM Cell Switch Command MAC CE; Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received.

This issue can be solved by discussing the following questions：

- Q1: If CTAT for target is running, should network always include TA value in LTM CSC MAC CE?

When the network decides to perform LTM cell switch and determines the target beam, if the TA to which the beam belongs has been acquired during the early UL sync of CLTM and the TA value is still valid (TAT is still running), should the network always include the TA value in the CSC MAC CE?

On the one hand, network also start the TAT when it sends the last TA command, so the TAT between network and UE are synchronized. Network knows the remaining TAT value of UE and whether it has expired or going to expire.

On the other hand, even if no new UL has been received, the network can transmit the same TA value only  to refresh the TAT.  This is valid in legacy to reset the TAT timer for longer validity to dynamically control the TA value, as the TA refresh time may dynamically change based on the scenario (such as UE speed or the scope of the TA range).

Conversely, if the network judges that the remaining timer margin is sufficient, it may omit the TA.

Therefore, it is valid that network include or not include TA value in the LTM CSC MAC CE.

Observation: If TAT is running, it is valid that network indicate or not indicate another TA command to UE.

Proposal 1: If CTAT for target is running, it is up to network to include the TA value (or not) in the (Enhanced) LTM CSC MAC CE.

If proposal 1 is agreed, the UE’s behavior of may be further discussed (assuming the target CTAT is still running):

- Q2: UE’s haviour if an (Enhanced) LTM Cell switch Command MAC CE is received with TA value

If the target CTAT is running, and UE receives an (Enhanced) LTM CSC MAC CE with TA value, the UE's behavior is consistent with the existing spec, i.e., process the newly indicated TA value (no matter this is a new or the same as the previous TA) and refresh the TAT timer.

3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; …… 3> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE or the Enhanced LTM Cell Switch Command MAC CE.

Proposal 2: If the target CTAT is running, and UE receives an (Enhanced) LTM CSC MAC CE with valid TA, UE process the newly indicated TA value and refresh the TAT timer.

- Q3: UE’s haviour if an (Enhanced) LTM Cell switch Command MAC CE is received without TA value.

If the target CTAT is running, and UE receives an (Enhanced) LTM CSC MAC CE without TA value, UE should use the stored TA rather than perform RACH-based LTM. The intention of not including TA in LTM CSC MAC CE is to let UE use the remaining CLTM TAT for the initial value of pTAT. This may be a valid case when remaining timer margin is sufficient for UE but if the TAT is reset then it will be too long.

Proposal 3: If the target CTAT is running, and UE receives an (Enhanced) LTM CSC MAC CE WITHOUT valid TA, UE processes the stored TA value and the remaining CLTM TAT timer for PTAG.

TA handling for beam with valid TA but no associated CG resource.

In the last meeting, the following agreements were made for target beam without associated CG resource:

(MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. (MAC-9) If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

In the latest running CR, the behavior is not updated according to the agreement, and the TA value/TAT timer is still applied depends only on the validity of the TA (running TAT timer)

1> when a conditional LTM cell switch procedure is triggered for a CLTM candidate cell or indicated by upper layer as specified in clause 5.y.3: 2> if the CLTM candidate cell is not configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell is running as specified in clause 5.2x: 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer. 2> if the CLTM candidate cell is configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the CLTM candidate cell for the TAG associated with the selected SSB or selected CSI-RS for CLTM is running as specified in clause 5.2x: 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2.

If the UE performs RACH based CLTM cell switch, the UE does not need to apply the previously stored TA value or CLTM TAT timer.

Proposal: The conditions for applying stored target TA and CLTM TAT should not only rely on TA validity, but also on whether it has associated CG resource. Reflect this change to MAC running CR.

  - Co-existence on CLTM and MIMO 2TA

In RAN2#129 bis meeting, RAN2 reached the following agreement for co-existence on CLTM and MIMO 2TA:

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. - For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

The two agreements made during the last meeting are all for the early UL sync with PDCCH-ordered early RACH case. For UE-based TA, no TAC MAC CE is sent to the UE. In this scenario, the situation is similar to R18 LTM, and co-existence cannot be supported.

Proposal : For CLTM, UE based TA measurement cannot be configured if candidate cell is configured with MIMO 2TA.

MAC-29

MAC-29 describes a scenario where the TA value of an LTM candidate is stored by the UE, and this candidate cell is subsequently removed from the LTM candidate list.

In the current MAC running CR, there is no description regarding TA maintenance operations (such as updating or releasing) for these candidate TAs. Operations are only performed when the UE receives the LTM candidate TA MAC CE, meaning the TA values are retained until timer expiration, even if the MAC is reset.

5.2x Maintenance of UL Synchronization for CLTM candidate cell The MAC entity shall for each CLTM candidate cell: - when an LTM Candidate Timing Advance Command MAC CE described in clause 6.1.3.4x is received: <<omit 2 TA case>> 2> else: 3> store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell as specified in clause 6.1.3.4x; 3> start or restart the ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM candidate cell as specified in clause 6.1.3.4x.

5.12 MAC Reset Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: 1> else if upper layers indicate the reset is triggered by conditional LTM, or triggered by LTM while there is CLTM candidate configuration(s), or triggered by handover while there is CLTM candidate configuration(s): 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimers, and tm-Candidate-TimeAlignmentTimerTAG2, if configured;

Since updating LTM candidates is a static process controlled by RRC, it is unlikely that the network will frequently re-add removed candidates or send additional RRC messages. As a result, the UE may unnecessarily maintain TA values for non-LTM candidates, especially when the TAT timer is long.

Proposal: The stored TA value for an LTM candidate cell should be released, and the corresponding TAT stopped, when the candidate cell is removed from the LTM candidate list.

MAC-30

In the email discussion, MAC-30 is specified as:

For CLTM： If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer? [Rapp]: Since Msg3/A buffer is empty during RACH-less CLTM, the RRCReconfigruaitonComplete message couldn’t be transimitted via Msg3/A buffer. We think RRCReconfigruaitonComplete could be retx after the competition of the RACH procedure for RRCReconfigruaitonComplete initial transmission. But let’s add one open issue in MAC-30 for discussion.

This issue come from the last meeting agreement:

During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

If the initial RRCReconfigurationComplete message is sent but fails, and the TA expires before the UE can retransmit the message, the UE will fall back to RACH-based CLTM and perform a random access procedure to the target cell. The RRCReconfigurationComplete message will then be sent after the random access procedure, which is consistent with legacy behavior. It is up to the UE implementation to either regenerate the RRCReconfigurationComplete message or retrieve it from the HARQ buffer, while nothing needs to be specified.

Proposal: For the scenario where RACH-less CLTM falls back to RACH-based CLTM, no additional specification is needed for UE behavior regarding the transmission of RRCReconfigurationComplete message.

Conclusion

In this contribution, the following observations and proposals are made:

2.1 mTRP coexistence

References

- [POST129bis][117][MOB] (Vivo)

- R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM