3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505169

Bengaluru, India, 25th – 29th Aug，2025

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on remaining issues on OD-SIB1 case 2. The contribution is organized as follows.

In section 2, we discuss the following aspects,

- OD-SIB request on SUL

- Open issue on SI changing

- SIB1 transmission for emergency case

- Cell barring for NES OD-SIB1 Cell

- Unnecessary Paging for legacy UE

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - OD-SIB request on SUL

In last meeting, one FFS issue is whether SIB1 request is supported for SUL or not. The SUL is used to enhance the uplink coverage, the legacy OD-SI request is supported on the supplementary uplink carrier, RA preamble for OD-SI will be transmitted on SUL.

Observation 1: OD-SI procedure has been supported on the supplementary uplink carrier.

R1-2501682 Final Report of 3GPP TSG RAN WG1 #120 v1.0.0: R1-2501380 FL summary 3 for on-demand SIB1 in idle/inactive mode Moderator (MediaTek) From Thursday session Conclusion It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different. Conclusion (amended as shown in red in Friday session) - The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system. - RA-RNTI is not included in the UL-WUS configuration for FDD system. - Cell barring information is not included in the UL-WUS configuration for FDD system. - Cell selection information is not included in the UL-WUS configuration for FDD system. - Accessible UE types is not included in the UL-WUS configuration for FDD system. - CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system. - InitialUplinkBWP is not included in the UL-WUS configuration for FDD system. - TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system. - SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system. - SupplementaryUL is not included in the UL-WUS configuration

For OD-SIB1 request, from the view of RAN2, no obvious issue is identified on SUL carrier, the legacy procedure on OD-SI request could be reused for OD-SIB1 request. Based on above record in RAN1 report, it can be seen that RAN1 once discussed this issue, there is a conclusion that supplementary UL is not included in the UL-WUS configuration based on little support, but which was not finally agreed by RAN1. Considering this issue once discussed in RAN1 and associated to RAN1 scope, it is suggested to leave it to RAN1 to determine whether SUL could be applied for OD-SIB1.

Proposal 1: RAN2 will not support OD-SIB1 request over SUL unless RAN1 determine to support it.

  - SI changing on NES cell

Report of [POST130][107][NES] (Ericsson): 5.2.2.3.1 Acquisition of MIB and SIB1 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED 3> else: while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 4> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]). NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell. [comment] During last meeting, at offline session [AT130][104][NES] (Ericsson), P6 in R2-2504037 about the UE behavior after receiving SI change notifcation was discussed. Rapporteur responded that a condition was added to address the issue: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x; And therefore, at the Comeback session, it was first agreed that: Keep RRC CR as it is for the short message and UE behaviour.. Then, it is commented that the high-lighted part mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, the high-lighted part was removed, and instead the NOTE x is added. However, RAN2#126 agrees that: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.. Therefore, we encourage RAN2 to review the agreement again ‘Keep RRC CR as it is for the short message and UE behaviour.’, whether the NOTE x can cover the above RAN2#126 agreement. The discussion is comprised of the following questions: Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion.

Based on above RRC open issue in Report of [POST130][107][NES], it can been that the SI change procedure needs to be clarified.

RAN1#118 meeting Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

Based on above RAN1 agreements, the UE could acquire the SIB1 based on the time information (e.g, the offset information in RAR) and searchSpaceZero and controlResourceSetZero in OD-SIB1 request configuration. Hence, the UE could also acquire the SIB1 based on other time information, such as the SIB1 period in MP procedure and the searchSpaceZero and controlResourceSetZero in OD-SIB1 request configuration.

For above SI change procedure in NES Cell, regarding legacy procedure, the UE will receive the SI change notification at the first modification period and acquire the SI at the next modification period. Hence, the UE needs to acquire SIB1 in the next modification period. For NES Cell, if SI change notification is transmitted in the first MP, one option is, the network could transmit the SIB1 in next MP based on the CORESET information in OD-SIB request configuration of NES Cell, it is not necessary to switch to CD-SSB mode, the K_Offset in MIB is still set as new value for NES Cell, the NES UE could acquire SIB1 since it has already stored the valid CORESET information included in on OD-SIB1 request configuration.

However, above option is only suitable for the case that the CORESET#0 in the stored OD-SIB1 request configuration is not updated. If the CORESET#0 in OD-SIB1 request configuration is updated, the UE could not acquire SIB1 in the next MP since the system information on CORESET#0 needs to effective immediately in the next period, then the reasonable and unified option is the network needs to switch the to normal mode with legacy K_SSB value. The UE also needs to read MIB and SIB1 in the next period which is also aligned to legacy procedure.

For the issue on whether re-introduce the agreement “Keep RRC CR as it is for the short message and UE behaviour.”, for SI change procedure, according to above analysis, the ssb-SubcarrierOffset is set as legacy one which means that SIB1 is scheduled, which is not in the branch of “2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:”, so the NOTEx is efficient to cover above case, it is not necessary to introduce the agreement again.

Proposal 2: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from network directly without sending OD-SIB1 request in next MP, i.e., the network will switch to normal mode with legacy K_SSB value and transmit the updated SIB1 in next MP.

  - SIB1 transmission for emergency case

According to legacy procedure, if UE is ETWS capable or CMAS capable and receives the etwsAndCmasIndication bit in the Short Message, it will re-acquire the SIB1 immediately if it is provided with searchSpaceSIB1 on the active BWP or the initial BWP, the parameter is included in SIB1 message.

If the UE receives a Short Message, the UE shall: 1> if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceSIB1 and searchSpaceOtherSystemInformation on the active BWP or the initial BWP: 2> immediately re-acquire the SIB1; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6: 3> acquire SIB6, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7: 3> acquire SIB7, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8: 3> acquire SIB8, as specified in clause 5.2.2.3.2, immediately; NOTE: In case SIB1, SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB1, SIB6, SIB7, or SIB8.

Observation 2: In legacy, UE needs to re-acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

For NES Cell, when emergency case occurs, the network could broadcast the SIB1 periodically based on the searchSpaceSIB1 included in prior valid SIB1 and CORESET information in WUS configuration stored by UE, the NES UE could receive the SIB1 immediately without any delay, it is not necessary to trigger the OD-SIB1 request procedure. Then, there is no issue is explored for emergency case, it could be considered as network implementation.

Proposal 3a: In emergency cases, it is up to network implementation to broadcast the SIB1 periodically based on the valid OD-SIB1 request configuration stored by UE, no spec impact will be introduced.

Proposal 3b: Upon receiving the etwsAndCmasIndication in a Short Message on the NES cell, the UE shall immediately re-acquire SIB1 based on the CORESET information contained in the OD-SIB1 request information, without sending an OD-SIB1 request. i.e., the UE will follow the legacy procedure.

  - Cell barring for NES OD-SIB1 Cell

In legacy R18 NES as follows, network introduced a specific barring indication cellBarred in SIB1 for NES UEs, the NES UE will read the cellBarredNES to see whether the UE (with NES DTX/DRX feature) will be barred or not if cellBarred in the acquired MIB is set to barred. Based on this indication, the NES Cell working on CellDTX-DRX feature could allow only NES UEs supporting cell DTX/DRX capability to camp on the NES Cell by broadcasting cellBarredNES.

TS 38.331 5.2.2.4 Actions upon receipt of System Information <skip> 1> if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends; <skip> – SIB1 SIB1-v1800-IEs ::= SEQUENCE { … cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R … } … cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2..

Observation 3: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Agreements on OD-SIB1 There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

For R19 OD-SIB1 enabled Cell, it has been agreed that the legacy UE and R19 UE not supporting OD-SIB capability will be barred by the OD-SIB1 enabled Cell, and k_ssb signaling “no SIB1” indication in MIB will be used. It seems that all the UE not supporting OD-SIB1 capability will be barred by the k_ssb at NES Cell. There is also no good reason for OD-SIB1 enabled Cell to bar the UE supporting OD-SIB1, otherwise, no UE could camp on OD-SIB1 Cell. So it is suggested that RAN2 to clarify the barring mechanism is used or not at OD-SIB1 enabled Cell for UE supporting OD-SIB1. For non-OD-SIB1 enabled Cell, the legacy mechanism could be used to bar all UEs including the UE supporting OD-SIB1, the UE supporting OD-SIB1 could follow the legacy procedure to determine whether the non-OD-SIB1 enabled Cell is barred or not.

Proposal 4: The OD-SIB1 enabled Cell will not introduce additional barring mechanism (e.g., a barring indication in SIB1) to bar UE supporting OD-SIB1.

  - Unnecessary Paging for legacy UE

RAN2#127bis agreement: 9. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. RAN2#128 agreement: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs. RAN2#129bis agreement: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

Although RAN2#129bis agreed that legacy UE could camp on NES UE if it could read the SIB1 message, however, it can be seen that the legacy UE will be barred by ssb-SubcarrierOffset or the new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs, which could make legacy UE not camp on OD-SIB1 Cell, then the paging for legacy UE seems not necessary at NES Cell. It is also network power saving if the paging for legacy UE is not transmitted at NES Cell. Hence, considering the legacy UE is not allowed to camp on NES Cell, also from the view of network power saving, it can be concluded that the NES Cell will not transmit the legacy UE paging message.

Observation 4: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 enabled Cell.

In RAN2#129 meeting, the following content was agreed for R19 paging adaptation:

RAN2#129 agreement: 5.A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Similarly, in order to avoid OD-SIB1 Cell paging legacy UEs, a new UE capability for support of OD-SIB1 can be introduced in UE-RadioPagingInfo. As UE-RadioPagingInfo is included in the CN paging and RAN paging NW nodes, OD-SIB1 Cell can decide whether to initiate paging procedure for a UE based on the new UE capability when CN paging or RAN paging is triggered. For example: When OD-SIB1 Cell receives a PAGING message from AMF and the new UE capability for support of OD-SIB1 is also included, OD-SIB1 Cell initiates paging procedure for the UE via Uu interface. Otherwise, OD-SIB1 Cell doesn’t initiate paging procedure via Uu interface as it identifies the PAGING message is for a UE that doesn’t support OD-SIB1 feature.

Proposal 5: Introduce a new UE capability for the support of OD-SIB1 in UE-RadioPagingInfo.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: OD-SI procedure has been supported on the supplementary uplink carrier.

Observation 2: In legacy, UE needs to re-acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

Observation 3: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Observation 4: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 enabled Cell.

Proposal 1: RAN2 will not support OD-SIB1 request over SUL unless RAN1 determine to support it.

Proposal 2: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from network directly without sending OD-SIB1 request in next MP, i.e., the network will switch to normal mode with legacy K_SSB value and transmit the updated SIB1 in next MP.

Proposal 3a: In emergency cases, it is up to network implementation to broadcast the SIB1 periodically based on the valid OD-SIB1 request configuration stored by UE, no spec impact will be introduced.

Proposal 3b: Upon receiving the etwsAndCmasIndication in a Short Message on the NES cell, the UE shall immediately re-acquire SIB1 based on the CORESET information contained in the OD-SIB1 request information, without sending an OD-SIB1 request. i.e., the UE will follow the legacy procedure.

Proposal 4: The OD-SIB1 enabled Cell will not introduce additional barring mechanism (e.g., a barring indication in SIB1) to bar UE supporting OD-SIB1.

Proposal 5: Introduce a new UE capability for the support of OD-SIB1 in UE-RadioPagingInfo.

Reference

[1] Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES, Samsung, RAN2#130 meeting.