3GPP TSG-RAN WG2 Meeting #131                         R2-2505182

Bangalore, India, Aug 25th–29th, 2025

Agenda Item:	8.2.3

Source: 	Transsion Holdings

Title:  	Discussion on Random Access for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#130 meeting, the A-IoT Random access was discussed, and the following agreements were made[1]:

Agreements

- For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952).  Capture a NOTE that other implementation are allowed.   X, Y will be signalled by paging message

- The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.  R2D trigger message is not sent in CFRA procedure.   Come back if RAN1/4 sees any issues.  Send LS to RAN1/RAN4

- FFS  R2D byte alignment dependent on TBS size discussion

Agreements on RA

- Exclude the option of  MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer)

- A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below.  A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary.

Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

Option A (the boundary being the subsequent paging only) is excluded.

For option C, further discuss in terms of complexity at the device vs reader flexibility.

- Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS Discuss further whether to include it.

Agreements on NACK reception:

- After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access.   FFS how to specify.

Agreements on RN16/AS ID maintainance:

- Confirm a device is not expected to maintain both AS ID and RN16.   After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader.

This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission.  How to capture device behavior is FFS

In this contribution, we provide our views on Random access to proceed the work item

- Discussion

  - MSG2 monitor window

| Issue 2-4: CBRA failure detection | - Further down selection between option B and C for msg2 monitor window in CBRA. - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. | To be discussed by company contributions |
| --- | --- | --- |

In the last meeting how to define MSG2 monitor window in CBRA is discussed, and the options of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window and before next paging are excluded. A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C. The device does not process MSG2 (re)transmission received after the boundary. The difference between Option B and C is whether the boundary is the reception of the next or the Kth R2D trigger message. For option B, it means the reader has to complete the transmission or retransmission of MSG2 before the subsequent R2D trigger, and the Msg2 can only echo X*Y RN16s with the benefit of less latency for device in success case. For optionC, the Msg2 can echo K*X*Y RN16s with the drawback of higher RN collision probability and latency for success cases. From our point of view, although the option C brings reader flexibility, however it may brings device complexity and MSG2 collision probaility, and for success cases it also brings device power consumption as the device may have to wait more time to get its Msg2. To keep things simple, the Option B can be adopted, thus the boundary for msg2 reception is the reception of either the next R2D trigger message or the subsequent paging message.

Proposal 1: For MSG2 reception, the boundary is the reception of either the next R2D trigger message or the subsequent paging message.

  - random ID differentiation in Msg2

| Issue 2-5: random ID differentiation in Msg2 | Whether to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - RAN2#130 agreement: Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - Status in running CR: not captured yet. | To be discussed by company contributions |
| --- | --- | --- |

In last meeting, RAN2 has agreed to support the TDM and FDM of MSG1 resources and multiplexing multiple received random IDs from different A-IoT devices in the same MSG2. As the Msg1 resources in one access occasion increase, the collision probaility of RN16 is increased, to solve this problem, an offline discussion was held and reached an

agreement that including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible, however it is still not a consensus to have it. The main concern is if the frequency index is along with RN16 in MSG2 to , it means the device has to maintain the frequency index together with the RN16, we think this doesn’t bring much device complexity and this can be included.

Proposal 2:  Frequency index can be included along with RN16 in MSG2 to reduce collisions of MSG1 between different devices.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT Random Access and made the following proposals:

Proposal 1: For MSG2 reception, the boundary is the reception of either the next R2D trigger message or the subsequent paging message.

Proposal 2: Frequency index can be included along with RN16 in MSG2 to reduce collisions of MSG1 between different devices.

- 4 References

- “Draft_RAN2_130_Meeting_Report_v3”