3GPP TSG-RAN WG2 Meeting#131	R2-2505193

Bengaluru, India, 25th – 29th Aug, 2025

Source:	vivo, OPPO, CATT, ZTE Corporation, Sanechips, Fraunhofer IIS, Fraunhofer HHI

Title:	Discussion on open issue LPP-18 consistency between training and inference

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

In this contribution, with the guidance illustrated by Stage 3 Rapporteur, we mainly discussed the open issue LPP-18 with potential solutions as follows.

- Discussion

Issue description: At RAN2#130, contribution R2-2503403 "Discussion on consistency between training and inference for AI POS", vivo et al. was discussed which resulted in the following agreement: To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.. Proposed resolution: Companies to provide contributions to the following meeting on details for "the request associated information in on demand prs request".

As the last meeting agreed, considering the accuracy of AI/ML based positioning is extremely area specific, UE should be allowed to request assistance data associated with specific area, i.e., a group of TRPs, in order to ensure the consistency between training and inference. The next step to take is how to implement this agreement into LPP Specification. We identify two issues to resolve:

- How to uniquely identify the specific group of TRPs;

- The requested assistance data type.

Following FFS, the detailed content of request associated information to represent this specific group of TRPs is to be determined first. As a solution, one may come out with a list of TRP IDs that can directly represent the area in which UE is requesting the assistance data. However, as TRP ID in LPP Spec is a logic identifier used locally and momently under an LMF. For example, LMF may provide UE with TRP ID = {1,2,3} at moment T1; while LMF may provide UE with TRP ID = {4,5,6} when at moment T2, where UE may locate at the same area at T1 and T2. The main reason is that NW may recycle and re-allocate TRP IDs to different TRPs with time passing by. In this sense, TRP ID cannot uniquely identify the area at the training and inference stage. However, considering there exists PRS-only transmitting point (TP), A TP which only transmits PRS and is not associated with a cell, TRP ID can be included on condition that TRP is dedicatedly represented without any cell information.

Besides, one may also propose to use associated ID to let NW be aware of the particular area. However, as RAN1 agreed, associated ID is per cell defined, which is valid within one cell and also cannot uniquely identify a specific group of TRPs across multiple cells. For example, different TRPs may be appointed with the same associated ID, when associated ID is used as requested identifier for TRP, it may cause misunderstanding or malfunction.

On the contrary, NCGI does not change, which perfectly suits the need here to uniquely identify an area at any time. Moreover, PCI with ARFCN is also used in the current specification as NCGI’s substitute. Therefore, we think that a list of NCGIs or a list of PCIs with ARFCN can be in a form of a group of TRPs. As mentioned above, TRP ID can be provided as a complementary information for LMF to refer as well.

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

- RAN2 to adopt a list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference.

Another issue is on which level of request to include such information. Here we come two candidates. The first one is to directly include the requested area information in NR-DL-AIML-RequestAssistanceData-r19, which ensures all the requested assistance data is within this area. Another one is to pair the requested area information with the requested on-demand DL-PRS configuration. This aims to allow UE to request on-demand PRS within a specific group of TRPs. Two candidates are illustrated respectively as in Annex.

- RAN2 to discuss how to introduce the specific group of TRPs associated with the requested assistance data:

- to directly acquire assistance data of specific TRPs, i.e., right under NR-DL-AIML-RequestAssistanceData;

- to ask specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request.

- Conclusion

- RAN2 to adopt a list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference.

- RAN2 to discuss how to introduce the specific group of TRPs associated with the requested assistance data:

  - to directly acquire assistance data of specific TRPs, i.e., right under NR-DL-AIML-RequestAssistanceData;

  - to ask specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#130

- R2-2505702	LPP open issues for feature AI/ML for NR air interface	Qualcomm

- Annex

- Candidate 1

NR-DL-AIML-RequestAssistanceData-r19 ::= SEQUENCE {

nr-PhysCellID-r19								NR-PhysCellID-r16					OPTIONAL,

nr-DL-PRS-AssistanceDataReq-r19					ENUMERATED { requested }			OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Req-r19			ENUMERATED { eAoD, eAoA }			OPTIONAL,

nr-on-demand-DL-PRS-Req-r19						NR-On-Demand-DL-PRS-Request-r17		OPTIONAL,

pre-configured-AssistanceDataReq-r19			ENUMERATED { true }					OPTIONAL,

nr-PositionCalculationAssistanceReq-r19			BIT STRING {

trpLoc					(0),

beamInfo				(1),

rtdInfo					(2),

losNlosInfo				(3),

trpTEG-Info				(4),

nr-IntegrityBounds		(5),

pruInfo					(6),

trpImplicitLoc			(7)

} (SIZE (1..8))						OPTIONAL,

nr-IntegrityAssistanceReq-r19					BIT STRING {

serviceParameters		(0),

serviceAlert			(1),

riskParameters			(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5)

} (SIZE (1..8))						OPTIONAL,

nr-TRP-ReqList-r19							NR-TRP-ReqList-r19	OPTIONAL,

...

}

NR-TRP-ReqList-r19 ::= SEQUENCE (SIZE(1..nrmaxFreqLayers-r16)) OF NR-TRP-ReqListPerFreqLayer-r19

NR-TRP-ReqListPerFreqLayer-r19 ::= SEQUENCE (SIZE(1..nrmaxTRPsPerFreq-r16)) OF TRP-ReqInfo-r19

TRP-ReqInfo-r19 ::= SEQUENCE {

dl-PRS-ID-r19					INTEGER (0..255),			OPTIONAL,	-- Need ON

nr-PhysCellID-r19					NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r19					NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r19						ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

}

NR-DL-AIML-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-DL-PRS-AssistanceDataReq This field, if present, indicates that the nr-DL-PRS-AssistanceData are requested.

nr-DL-PRS-ExpectedAoD-or-AoA-Req This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the nr-DL-PRS-AssistanceDataReq is also present.

nr-on-demand-DL-PRS-Req This field indicates the on-demand DL-PRS requested for DL AI/ML positioning.

pre-configured-AssistanceDataReq This field, if present, indicates that the target device requests pre-configured assistance data with area validity.

nr-PositionCalculationAssistanceReq This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 5 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are requested or not; bit 5 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is requested or not; bit 5 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is requested or not; - bit 6 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is requested or not - bit 7 indicates whether the field nr-TRP-ImplicitLocationInfo in IE NR-PositionCalculationAssistance is requested or not.

nr-IntegrityAssistanceReq This field indicates the Integrity Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is requested or not.

nr-TRP-ReqList This field indicates the list of specific TRPs within which the UE request the related assistance data.

- Candidate 2

NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE {

dl-prs-StartTime-and-Duration-r17			DL-PRS-StartTime-and-Duration-r17		OPTIONAL,

nr-on-demand-DL-PRS-Information-r17			NR-On-Demand-DL-PRS-Information-r17		OPTIONAL,

dl-prs-configuration-id-PrefList-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF 
													DL-PRS-Configuration-ID-r17			OPTIONAL,

...,

[[

dl-PRS-AggregationID-PrefList-r18			SEQUENCE (SIZE (1.. maxOD-DL-PRS-Configs-r17)) OF

INTEGER (1.. maxOD-DL-PRS-Configs-r17)

OPTIONAL,

nr-OnDemandDL-PRS-AggregationReqList-r18	SEQUENCE (SIZE (1.. maxOD-DL-PRS-Configs-r17)) OF

NR-OnDemandDL-PRS-AggregationReqElement-r18

OPTIONAL

]],

[[

nr-TRP-ReqList-r19							NR-TRP-ReqList-r19	OPTIONAL

]]

}

DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE {

dl-prs-start-time-r17		INTEGER (1..1024)						OPTIONAL,

dl-prs-duration-r17			SEQUENCE {

seconds-r17		INTEGER (0..59)		OPTIONAL,

minutes-r17		INTEGER (0..59)		OPTIONAL,

hours-r17		INTEGER (0..23)		OPTIONAL,

...

}										OPTIONAL,

...

}

NR-OnDemandDL-PRS-AggregationReqElement-r18 ::= SEQUENCE (SIZE (2..3)) OF

INTEGER (1..nrMaxFreqLayers-r16)

NR-TRP-ReqList-r19 ::= SEQUENCE (SIZE(1..nrmaxFreqLayers-r16)) OF NR-TRP-ReqListPerFreqLayer-r19

NR-TRP-ReqListPerFreqLayer-r19 ::= SEQUENCE (SIZE(1..nrmaxTRPsPerFreq-r16)) OF TRP-ReqInfo-r19

TRP-ReqInfo-r19 ::= SEQUENCE {

dl-PRS-ID-r19					INTEGER (0..255),			OPTIONAL,	-- Need ON

nr-PhysCellID-r19					NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r19					NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r19						ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

}

TRP-ReqInfo field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP that the UE on-demand requests for. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellID.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.