3GPP TSG-RAN WG2 Meeting #131		     R2-2505200

Bengaluru, IN: 25th August – 29th August 2025

Source:	Qualcomm Incorporated

Title:	Open Issues on Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

This contribution paper discusses the open issues on data collection for the network side model training for beam management use case.

# Data Collection for Network Side Models

## 2.1 Dynamic Activation/Deactivation of Measurement Resources

In the RAN2, the following are FFS issues associated with data collection configuration for network-side model training,

- Whether the UE can be configured with multiple RS configurations for network-side model training,

- Whether dynamic activation/deactivation is supported

The training data configuration consists of

- Measurement targets (e.g., SSB and NZP resources for measurements), and

- logging configuration (e.g., periodicity, event triggers for logging).

The configuration/activation of the measurement target for NW-side training should be subject to UE capability. The network should ensure that measurement resources configured/activated (e.g., SSB, NZP resources) across different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring) and different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes) lie within the maximum number of SSB/CSI-RS measurements supported at the UE. This can be achieved by the following

- Network configures one or more resources for data collection configuration, but activates measurements on resources based on the UE capability, or

- Network configuration measurements on resources based on the UE capability.

To avoid the frequent reconfiguration for measurement resource reconfiguration, the network can configure the UE with one or more resources. Thereafter, the network may dynamically (de)activate measurement on different resource sets based on UE mobility and network requirements.

Observation 1: Consistent with the current standard, the network should continue ensuring that the maximum number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

Observation 2: The UE capability can be ensured where

- Network configures one or more resources for data collection configuration, but activates measurements on resources based on the UE capability, or

- Network configuration measurements on resources based on the UE capability.

Proposal 1: To avoid the frequent reconfiguration for measurement resource reconfiguration, the network can configure the UE with one or more resource sets for data collection for network-side training.

Proposal 2: The network may dynamically (de)activate measurement on different resource sets (among the configured resource sets) based on the UE mobility and the network requirements.

Furthermore, to facilitate the network with (de)activation of appropriate RS for data collection, the UE may indicate to the network when the configured RS becomes undetectable, i.e., the UE cannot measure the configured RS.

Proposal 3: To assist the network with (de)activation of the appropriate RS for data collection, the UE may notify the network when the configured RS becomes undetectable, meaning the UE cannot measure configured RS.

## 2.2 Reporting Assistance Information related to Logged Measurements

In RAN2#129bis meeting [1], RAN2 agreed on the following

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Note that the UE may need to send the low-power indication, irrespective of whether the UE has logged measurements or not. Therefore, low power indication should be considered independently from data availability indication. On the other hand, for the data availability indication, indicating the cause should be sufficient. The UE should set the cause or reason for triggering UAI only when the UE has available data. Therefore, for data availability, a single bit indication is used to indicate the reason for the trigger (full buffer, threshold).

Proposal 4: Introduce a one-bit indication for low power indication.

Proposal 5: For data availability, a one-bit indication is used to indicate the reason for the trigger (full buffer, threshold). Data availability is implicitly indicated by the reason for the trigger (full buffer, threshold).

## 2.3 Further Enhancements for Network-Side Data Collection for Different Use Cases

In RAN2#127 [2], RAN2 agreed that AS memory is shared across all of the use cases. In Rel-19, beam management is the only RAN-centric use case. Therefore, there is no need to discuss further enhancements for network-side data collection for different use cases.

Agreements

- UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

Observation 3: In Rel-19, beam management is the only RAN-centric use case requiring network-side data collection.

Proposal 6: No need to discuss further enhancements for network-side data collection for different use cases (at least in Rel-19).

## 2.4 Data Forwarding to the OAM or Source gNB after Handover

RAN3 has existing solutions for forwarding the logged measurement report (to the OAM) and SON reports (e.g., RLF report, SHR report, MHI report, RA report, etc) (to the source cell) from the retrieving gNB to the source gNB / OAM. Traditionally, the inter-node signaling is used to forward the required information to facilitate the UE configuration (during handover, PSCell change, etc.). Therefore, the retrieved logged measurements for network side training from the target to the source / OAM are forwarded using the RAN3 solution, similar to the logged MDT and different SON reports.

Observation 4: RAN3 has existing solutions for forwarding the logged measurement report (to the OAM) and SON reports (e.g., RLF report, SHR report, MHI report, RA report, etc) (to the source cell) from the retrieving gNB to the source gNB / OAM.

Observation 5: Traditionally, the inter-node signaling is used to forward the required information to facilitate the UE configuration (during handover, PSCell change, etc.).

Proposal 7: Forwarding the logged measurement for network-side training from the target gNB to the source gNB / OAM is left to RAN3.

# Conclusion

Data collection and reporting for the beam management use case

Observation 1: Consistent with the current standard, the network should continue ensuring that the maximum number of SSB/CSI-RS configured/activated for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

Observation 2: The UE capability can be ensured where

- Network configures one or more resources for data collection configuration, but activates measurements on resources based on the UE capability, or

- Network configuration measurements on resources based on the UE capability.

Proposal 1: To avoid the frequent reconfiguration for measurement resource reconfiguration, the network can configure the UE with one or more resource sets for data collection for network-side training.

Proposal 2: The network may dynamically (de)activate measurement on different resource sets (among the configured resource sets) based on the UE mobility and the network requirements.

Proposal 3: To assist the network with (de)activation of the appropriate RS for data collection, the UE may notify the network when the configured RS becomes undetectable, meaning the UE cannot measure the configured RS.

Proposal 4: Introduce a one-bit indication for low power indication.

Proposal 5: For data availability, a one-bit indication is used to indicate the reason for the trigger (full buffer, threshold). Data availability is implicitly indicated by the reason for the trigger (full buffer, threshold).

Observation 3: In Rel-19, beam management is the only RAN-centric use case requiring network-side data collection.

Proposal 6: No need to discuss further enhancements for network-side data collection for different use cases (at least in Rel-19).

Observation 4: RAN3 has existing solutions for forwarding the logged measurement report (to the OAM) and SON reports (e.g., RLF report, SHR report, MHI report, RA report, etc) (to the source cell) from the retrieving gNB to the source gNB / OAM.

Observation 5: Traditionally, the inter-node signaling is used to forward the required information to facilitate the UE configuration (during handover, PSCell change, etc.).

Proposal 7: Forwarding the logged measurement for network-side training from the target gNB to the source gNB / OAM is left to RAN3.

# References

- Meeting Report, “RAN2#129bis meeting report”

- Meeting Report, “RAN2#127 meeting report”