3GPP TSG-RAN WG2 Meeting #131	R2-2505225

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on link level enhancement

Document for: 	Discussion and Decision

Introduction

For link level DL CE, RAN1 discussed support of Type0 PDCCH CSS repetition of searchSpaceZero, PDSCH repetition for SIB1 and Msg4 PDSCH repetition [1][2] and send an LS [3] to RAN2 on Msg4 PDSCH repetition. This contribution discusses RAN2 impacts based on latest RAN1 progresses.

Discussion

2.1	On access barring

RAN1’s agreements on PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero and PDSCH repetition of SIB1 are excerpted in the following box. It is observed that:

- PDCCH repetition for Type0 PDCCH CSS is enabled/disabled using reserved bit in PBCH payload. The enabling/disabling of SIB1 PDSCH repetition is implicitly indicated by the enabling/disabling of Type-0 CSS PDCCH repetition. No RAN2 mechanism is required to enable/disable these enhancements.

- A legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot, and this enables the legacy UE to acquire SIB1 and access the cell normally.

- PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1: - Enabling/disabling using reserved bit(s) in PBCH payload - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2. Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the solution agreed for M = 2 is also applied to M = 1 and M = 1/2. - PDSCH repetition of SIB1 Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination. Agreement The enabling/disabling of SIB1 PDSCH repetition is implicitly indicated by the enabling/disabling of Type-0 CSS PDCCH repetition.

Observation 1: Based on the agreements of RAN1, legacy UEs might successfully decode the SIB1 of a cell when the cell applies PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero and PDSCH repetition of SIB1.

On Msg4 PDSCH repetition, three steps are needed, which are: 1) the NW enables PDSCH Msg4 repetition in SIB1; 2) the UE report its capability/request in Msg3 PUSCH; and 3) the NW activates Msg4 repetition for the UE via DCI format 1_0. This implies Msg4 PDSCH repetition is activated per UE only after the UE reported its capability; legacy UEs can still access a cell that supports Msg4 PDSCH repetition by following legacy procedure.

Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled. Send LS to RAN2 informing about the above agreement, asking RAN2 to consider the agreement when designing the report in Msg3 PUSCH.

Observation 2: Based on the agreements of RAN1, Msg4 PDSCH repetition is activated per UE after the UE reported its capability; legacy UEs can still access a cell that supports Msg4 PDSCH repetition by following legacy procedure.

Observation 1 and observation 2 confirm legacy UEs can successful access a cell that supports/applies PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero, PDSCH repetition of SIB1 and Msg4 PDSCH repetition. Consequently, RAN2 need not to introduce new barring bit for the link level enhancement.

Proposal 1: No additional barring bit is introduced for the link level enhancement.

2.2	On Msg4 PDSCH repetition

Per the RAN1 agreement and the LS [3], the generic procedure for Msg4 PDSCH repetition is shown in the figure below. Some detailed issues are discussed in this section.

- Issue#1: How to configure the aggregation factor in SIB1

Per RAN1’s agreement and the consolidated Rel-19 higher layer parameters list [4], the aggregation factor pdsch-AggregationFactor is configured in SIB1 under the parent IE PDSCH-ConfigCommon. The 38.331 running CR [5] is aligned with this design, RAN2 endorses the implementation approach specified therein.

- The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

Proposal 2: The aggregation factor of Msg4 PDSCH repetition is configured in SIB1 under the parent IE PDSCH-ConfigCommon. RAN2 endorses the implementation approach specified in the running CR.

- Issue#2: How the UE reports its Msg4 PDSCH repetition capability/request in Msg3 PUSCH

RAN1 has agreed that the capability/request for Msg4 PDSCH repetition is indicated in Msg3 PUSCH, similar as Msg4 HARQ-ACK repetition in Rel-18. During Rel-18 standardization, extensive discussions were held on how to convey this indication, considering candidate options such as using one R bit in MAC subheader, introducing a zero length MAC CE, or re-purpose a spare bit in the RRC messages, etc. Finally, RAN2 adopted the first R bit to extend the LCID. In Rel-18, this extension added 64 LCIDs and 8 of which have been used [6].

Figure 6.1.2-3: (LX)/R/LCID/ MAC subheader

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

With this is the only one RAN2 meeting remaining, repeat the lengthy Rel-18 discussion is not expected. Instead, we prefer use the Msg4 HARQ-ACK repetition indication mechanism for Msg4 repetition indication, provided it is workable. Feasibility analysis is given as below:

  - Number of the reserved LCIDs: According to the Table 6.2.1-2c in TS 38.321, there is 56 codepoints reserved, which is sufficient for indication of Msg4 PDSCH repetition and possible feature combinations.

  - Applied random access procedures: Refer to the agreed CR from RAN1 [7], only the case “PDSCH scheduled by DCI format 1_0 in PDCCH with CRC scrambled by TC-RNTI” is considered. Therefore, RAN2 should consider Msg4 PDSCH repetition only for CCCH SDU was included in Msg3 [6] (i.e., only for initial access including RRC connection establishment, RRC connection re-establishment or RRC connection resume). This is aligned with the limitation of this MAC subheader [6].

A MAC subheader for a MAC SDU containing UL CCCH consists of the header fields (LX)/R/LCID.

In summarize the reserved codepoints in Table 6.2.1-2c can be used for Msg4 PDSCH repetition.

Proposal 3: Use the reserved codepoints of LCID for UL-SCH when the LX field is set to 1 to indicate Msg4 PDSCH repetition and any relative feature combinations.

All the possible feature combinations are listed hereafter:

|  | Feature combinations |
| --- | --- |
| 1 | CCCH of size 48 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 2 | CCCH of size 64 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 3 | CCCH of size 48 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 4 | CCCH of size 64 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 5 | CCCH of size 48 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 6 | CCCH of size 64 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 7 | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 8 | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 9 | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 10 | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 11 | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 12 | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |

In last meetings, company proposed that feature combination of 7~12 may not be needed: according to [8][9], the Msg4 PDSCH channel is consistently stronger than the PUCCH with HARQ-ACK Msg4 channel. Thus, when an UE requires Msg4 PDSCH repetition, it also requires PUCCH repetition of Msg4 HARQ-ACK, i.e., request Msg4 PDSCH repetition can implicitly indicate request PUCCH repetition of Msg4 HARQ-ACK.

However, the Msg4 HARQ-ACK repetition is a feature that optionally supported by UE in Rel-18. Mandating it as a prerequisite for Msg4 PDSCH repetition would couple the two features and restrict UE capability flexibility. We therefore recommend keeping them independent and allocating 12 codepoints for every feature combinations listed in the table above.

Observation 3: To indicate PDSCH repetition of Msg4, there are 12 kinds of feature combinations. Using a separate codepoint for each combination is benefit for UE capability flexibility.

Proposal 4: Introduce the following codepoints for UL-SCH when the LX field is set to 1. Send an LS to RAN1 on our decision:

- CCCH of size 48 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 of a RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 of a RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 of an eRedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 of an eRedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE.

- Issue#3: whether to specify condition(s) for the UE to report its capability/request

There is an FFS left in RAN1’s agreement:

- FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs.

We think is sufficient to leave it to UE implementation, based on the channel condition, to decide whether to report its capability/request; specified condition(s) is not needed. Schedule Msg4 repetition is then left to the network decision according to the UE’s indication.

Proposal 5: Do not specify any conditions for the UE to report its Msg4 repetition capability/request.

Conclusion

In this contribution, link level enhancement is further discussed with the corresponding observations and proposals listed below:

On access barring

Observation 1: Based on the agreements of RAN1, legacy UEs might successfully decode the SIB1 of a cell when the cell applies PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero and PDSCH repetition of SIB1.

Observation 2: Based on the agreements of RAN1, Msg4 PDSCH repetition is activated per UE after the UE reported its capability; legacy UEs can still access a cell that supports Msg4 PDSCH repetition by following legacy procedure.

Proposal 1: No additional barring bit is introduced for the link level enhancement.

On Msg4 PDSCH repetition

Proposal 2: The aggregation factor of Msg4 PDSCH repetition is configured in SIB1 under the parent IE PDSCH-ConfigCommon. RAN2 endorses the implementation approach specified in the running CR.

Proposal 3: Use the reserved codepoints of LCID for UL-SCH when the LX field is set to 1 to indicate Msg4 PDSCH repetition and any relative feature combinations.

Observation 3: To indicate PDSCH repetition of Msg4, there are 12 kinds of feature combinations. Using a separate codepoint for each combination is benefit for UE capability flexibility.

Proposal 4: Introduce the following codepoints for UL-SCH when the LX field is set to 1. Send an LS to RAN1 on our decision:

- CCCH of size 48 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 of a RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 of a RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 of an eRedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 of an eRedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE;

- CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE.

Proposal 5: Do not specify any conditions for the UE to report its Msg4 repetition capability/request.

Reference

- RAN1#120bis	Chair notes RAN1#120bis.

- RAN1#121	Chair notes RAN1#121.

- R2-2504965	LS on Msg4 PDSCH repetition.

- R1-2503243	LS on Rel-19 higher layers parameters list Post RAN1#121.

- 38331 Running CR - Introduction of NTN enh phase 3.

- 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

- RP-251583	Introduction of Rel-19 Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2501798	Discussion on DL coverage enhancement, Xiaomi.

- 3GPP TR38.830: "Study on NR coverage enhancements ".