3GPP TSG-RAN WG2 Meeting #131	R2-2505238

Bangalore, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In previous RAN1 meetings and RAN2 meetings, great progress was made for LP-WUS in RRC_CONNECTED mode. Based on the agreements, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode. And our proposals are provided.

Discussion

2.1	MAC-X1: The impact to the BWP switching mechanism

The bwp-InactivityTimer is used to control when the UE fallback to the default BWP/initial BWP(e.g., default BWP is not configured). It will be started or restarted when downlink assignment or uplink grant received for/on the active BWP, or MAC PDU is transmitted or received in a CG uplink grant or downlink assignment. Normally, when the MAC entity does not receive a LP-WUS indication from lower layer, it will start to monitoring the PDCCH, and bwp-InactivityTimer can be started/restarted when downlink assignment or uplink grant is received for/on the active BWP. Hence, it is not necessary to start/restart bwp-InactivityTimer when the MAC entity receives a LP-WUS indication from lower layer.

Proposal 1：No impact on bwp-inactivityTimer when the MAC entity receives a LP-WUS indication from lower layer.

2.2	UE operation when LP-WUS is monitored during DTX inactive time

It was captured in RAN1 spec that [1]:

## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED /Skipped/ A UE does not monitor WUS during DTX inactive period for the primary cell. /Skipped/

It can be seen that the UE will not monitor LP-WUS during DTX inactive time.

While the serving cell is in the cell DTX Active time, the UE will monitor PDCCH on the Serving cell. If DRX is configured for the UE, the UE monitors PDCCH based on the DRX configuration.

1> if the Serving Cell is in the cell DTX Active Period: 2> monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

However, when the LP-WUS is not monitored due to cell DTX inactive time, there may be other issues for the first drx-onDurationTimer for Option 1-1 and first lpwus-PDCCHMonitoringTimer for Option 1-2 during the cell DTX active time.

Based on RRC, it is specified that the configured cellDTX-DRX-Cycle is an integer multiple of configured drx-longCycle or vice versa. In the following, we analyse the cases including CellDTX-DRX-Cycle is an integer multiple of drx-longCycle and vice versa for both Option 1-1 and Option 1-2.

Option 1-1

As illustrated in Figure 1 and Figure 2, the LP-WUS which indicates whether to start the first drx-onDurationTimer during Cell DTX active time is not monitored by the UE.

Figure 1: LP-WUS monitoring when cellDTX-DRX-Cycle = M* drx-longCycle, M is an integer

Figure 2: LP-WUS monitoring when drx-longCycle = N* cellDTX-DRX-Cycle, N is an integer

Based on current MAC running CR, the UE will not start drx-onDurationTimer when LP-WUS is not monitored due to cell DTX inactive time, since the none of conditions highlighted in yellow are satisfied in this case.

2> else if LP-WUS monitoring is configured: 3> if lpwus-PDCCH-MonitoringTimer is not configured: 4> if LP-WUS indication associated with the current DRX cycle received from lower layer indicates to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 4> if all LP-WUS monitoring occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4): 5> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

If so, the first drx-onDurationTimer within Cell DTX active period will not be started which may bring impact on the transmission latency. Especially for case of drx-longCycle = N* cellDTX-DRX-Cycle, where all the LP-WUS will not be monitored and all the drx-onDurationTimer will not be started.

One of the solutions is up to the network implementation to configure the LP-WUS only within the DTX Active period. But based on the LP-WUS monitoring procedure specified in 38.213 running CR [1] it can be seen that there is one gap between WUS monitoring occasion and drx-onDurationTimer. There will be the configuration restrction on LP-WUS and DTX/DRX in this solution.

38.213 running CR 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED /Skipped/ The UE starts to monitor WUS in a first WUS monitoring occasion that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1. /Skipped/

The other solution is to follow the collision cases including Active Time, measurement gap, and RAR window monitoring for BFR. I.e., the first drx-onDurationTimer is started. Then the network doesn’t have to wake up the UE in the next DRX cycle which may bring extra latency to the downlink traffic and is not friendly to the system performance. In this solution, all the drx-onDurationTimer may be started, since all the LP-WUS monitoring occasions are overlapped with Cell DTX inactive time. Then, no power gain can be obtained from LP-WUS.

RAN2 needs to discuss which solution is adopted for the issue that LP-WUS is not monitored due to the Cell DTX inactive time.

Proposal 2: Suggest RAN2 discuss the issue caused by collision between LP-WUS monitoring occasion and Cell DTX inactive time. The potential solutions can be:

- Option 1: it is up to the network implementation to configure LP-WUS within the Cell DTX Active time;

- Option 2: the UE starts the corresponding drx-onDurationTimer within Cell DTX active time When the LP-WUS is not monitored due to the Cell DTX inactive time.

Option 1-2

For Option 1-2, the collision issue was discussed in the post email [2] and the following options are provided:

- Option 1: UE starts the lpwus-PDCCHMonitoringTimer when there is collision. (Same operation as Option 1-1)

- Option 2: UE doesnot start the lpwus-PDCCHMonitoringTimer when there is collision.

- Option 3: NW can configure UE whether to start the lpwus-PDCCHMonitoringTimer or not when there is collision.

But the views from companies were divided. The Rapp suggests Option 3, i.e., NW can configure UE whether to start the lpwus-PDCCHMonitoringTimer or not when there is collision, to solve this issue.

Regarding the issue that LP-WUS is not monitored during cell DTX inactive time, the UE operation for LP-WUS can follow the same way.

Proposal 3: NW can configure UE whether to start the corresponding lpwus-PDCCHMonitoringTimer within the Cell DTX active time or not when LP-WUS not monitored due to Cell DTX inactive time.

Conclusion

In this document, we analyze the remaining issues in RRC_CONNECTED for LP-WUS, and the following and proposals are provided:

Proposal 1：No impact on bwp-inactivityTimer when the MAC entity receives a LP-WUS indication from lower layer.

Proposal 2: Suggest RAN2 discuss the issue caused by collision between LP-WUS monitoring occasion and Cell DTX inactive time. The potential solutions can be:

- Option 1: it is up to the network implementation to configure LP-WUS within the Cell DTX Active time;

- Option 2: the UE starts the corresponding drx-onDurationTimer within Cell DTX active time When the LP-WUS is not monitored due to the Cell DTX inactive time.

Proposal 3: NW can configure UE whether to start the corresponding lpwus-PDCCHMonitoringTimer within the Cell DTX active time or not when LP-WUS not monitored due to Cell DTX inactive time.

Reference

- R1-2504971, CR, Introduction of low-power wake-up signal and receiver for NR;

- R2-2505477, discussion, Report of [Post130][213][LPWUS] Running CR for TS 38.321 (Apple).