3GPP TSG-RAN WG2 Meeting #131	R2-2505245

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on paging procedure for Ambient IoT

Document for:	Discussion, Decision

# Introduction

According to the previous RAN2 discussion, the paging message is used for both CFA and CBRA for DO-DTT and DT procedures in Rel-19. According to the WID [1] of Rel-20 A-IoT, the paging message needs to include the configuration for the DO-A UE. In this contribution, we provide our understandings on the remaining issues related to the paging message.

# Discussion

## Security parameters

LS from SA3 in S3-252392 [2]: SA3 is currently working on the authentication procedure. For this procedure, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size.

LS from SA2 in S2-2505793 [3]: In TS 23.369 clause 6.2.2, it is specified that the AIOTF sends the AIoT Identification Information to be included in the paging message to the NG-RAN node, and the AIoT Identification Information can include Filtering Information or a single AIoT Device Identifier. For the Filtering Information, SA2 has agreed the attached pCR (S2-2505850) and asks CT4 to continue the stage 3 work of the Filter Information. Security aspect of the Filter Information will be decided by SA3. For the single AIoT Device Identifier, what identifier(s) to be used is under discussion in SA3. SA2 kindly asks CT4 to provide the maximum length of the Filtering Information, and kindly asks SA3 to provide feedback on security aspect of the Filtering Information and on the single AIoT Device Identifier.

LS from CT4 in C4-252466 [4]: Figure aa.3.1: Structure of Filtering Information

MAC running CR for AIoT [5]: Figure 6.2.1.1-1: MAC PDU of A-IoT Paging message indicating CBRA Figure 6.2.1.1-2: MAC PDU of A-IoT Paging message indicating CFA

As quoted above, SA3 LS in S3-252392 [2] askes RAN2 to add a 128-bit random number in the paging message. SA3 requests RAN2 to confirm the feasibility of including the additional 128-bit random number in the paging message, and the maximum number of bits can be used for the security parameters.

According to the LS from SA2 in S2-2505793 [3], “the AIoT Identification Information can include filtering Information or a single AIoT Device Identifier”.

According to the LS from CT4 in C4-252466 [4], the maximum size of “filtering information” is 256 bits.

According to the discussion on the “single AIoT Device Identifier” in SA3, the “single AIoT Device Identifier” is designed as a temporary identity allocated by the network, which is expected to have a smaller size compared with the “filtering Information”.

According to the discussion on the MAC running CR [5], the paging message size of CBRA is larger than the paging message size of CFRA, and 8 bits of “Paging ID Length” are to be used. 7 bits are to be added to indicate the TBS of the paging message. The maximum paging message size in Rel-19 is 440 bits of byte-alignment (i.e. 47 + TBS indication (7) + maximum paging ID (256) + security parameters (128) = 438 bits). According to RAN1 specification, the maximum TBS for R2D message is 1000 bits. Thus, it is feasible to include additional 128 bits security parameters in paging message.

Regarding the question on the maximum number of bits which can be used for security parameters, we think that we should avoid sending too many bits of security parameters in the paging message, as more bits in paging message will reduce the coverage of the paging message. On the other hand, as the paging message is to be extended to include future extensions (i.e. “Fill Bits” field), the paging message should reserve some bits for the future extension. Thus, we think that RAN2 should inform SA3 not to include too many bits of security parameters in the paging message, so as to allow future extension and ensure the transmission coverage of the paging message.

Proposal 1: Rely to SA3 that:

- It is feasible to include additional 128 bits security parameters in paging message.

- The maximum size of paging message is 1000 bits.

- It is difficult to calculate the maximum bits which can be used for security parameters, as the paging message is used to include both the Rel-19 paging fields and future extension fields. We should avoid sending too many bits of security parameters according to the current paging message size.

## Transaction ID for CFRA

RAN2#130 meeting agreements [6]: - For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case.

RAN2#130 meeting agreements [6]: - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification.

Rel-20 A-IoT WID [1]: RAN2-led - DO-A specific procedure design. - Paging and random access for active device(s).

According to the RAN2#130 meeting agreement quoted above, Rel-19 devices are not expected to receive parallel service requests. The command message is sent via the “R2D Upper Layer Data Transfer message” (which does not include Transaction ID) following the CFA Msg1. In Rel-19, as the assumption for CFA procedure is that each paging indicating CFRA is only for one device, the transaction ID is not needed in the paging message for CFA. According to the Rel-20 AIoT WID scope [1], the DO-A is triggered by the device, according to the DO-A configuration provided via the paging message. As the DO-A configuration is not restricted to a specific service procedure (i.e. transaction), the transaction ID is also not needed in the Rel-20 paging message for forward-compatibility.

Proposal 2: The transaction ID is not needed for CFA paging.

## R bit

RAN2#130 meeting agreements: - To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required.

Text proposal for Rel-19 MAC running CR for AIoT [5]: #### 6.2.1.1 A-IoT Paging message … - R: This field is a future extension indication. The device shall ignore the A-IoT Paging message when the value of this field is set to 1.

Example of Rel-20 MAC specification: #### 6.2.1.1 A-IoT Paging message … - R: When the value of this field is set to 1, the fields in this message only include R2D Message Type, D2R Scheduling Info and Number of Access Occasions.

According to the MAC running CR, only one bit of R field is used for the future extension. The intention is to allow the Rel-19 device to ignore the whole paging message, when only the Rel-20 features (e.g. DO-A configuration) are included in the paging message. For example, the Rel-20 paging message including only DO-A configuration may include only the following fields:

- R2D Message Type

- D2R Scheduling Info

- Number of Access Occasions

Proposal 3: To clarify that if the R bit is set to “1”, the Rel-19 device skips the paging message.

# Conclusion

In this contribution, we focus on paging procedure and have the following proposals:

Proposal 1: Rely to SA3 that:

- It is feasible to include additional 128 bits security parameters in paging message.

- The maximum size of paging message is 1000 bits.

- It is difficult to calculate the maximum bits which can be used for security parameters, as the paging message is used to include both the Rel-19 paging fields and future extension fields. We should avoid sending too many bits of security parameters according to the current paging message size.

Proposal 2: The transaction ID is not needed for CFA paging.

Proposal 3: To clarify that if the R bit is set to “1”, the Rel-19 device skips the paging message.

# Reference

[1] RP-251885, “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2”

[2] S3-252392, “LS on security parameter in paging request message”

[3] S2-2505793, “Reply LS on paging ID length”

[4] C4-252466, “Reply LS on paging ID length”

[5] [POST130][027][AIoT] MAC Running CR (Huawei)

[6] RAN2#130 meeting minutes