3GPP TSG RAN WG2 Meeting #131                                                            R2-2505264

# Bangalore, India, August 25th - 29th, 2025

Agenda item:	8.2.3

Source:	Ofinno

Title:	Remaining issues on A-IoT random access

Document for:	Discussion and decision

# Introduction

This document discusses unclear parts and suggests corresponding TPs on the selection of access occasions (AOs) for Msg1 and the failure determination for the CBRA considering the running CR reviewed as part of the email discussion [POST130][027][AIoT] [1][2].

# Discussion

## Selection of access occasions (AOs) for Msg1 in CBRA

For the selection of an access occasion (AO) for D2R transmission of Access Random ID message, the following points seem unclear/ambiguous on how the device behaves (mainly in §5.3.1.1 of A-IoT MAC running CR [1]):

- Device’s behaviour associated to the Access Trigger message during CBRA (e.g., how long it monitors it for, when to start/finish monitoring it and what to do upon detecting the applicable Access Trigger message).

- Device’s determination of the random number (RN) ‘i’ (e.g., when the RN may be calculated within a paging cycle, i.e. between two consecutive A-IoT Paging messages).

- Device’s choice to perform the selection of AOs via a counter-based approach.

The device’s behaviour seems unclear when selecting access occasion (AO) for D2R transmission of Access Random ID message for CBRA (e.g., monitoring and actions associated to the Access Trigger message, determination of random number (RN) during a paging cycle and choice of the counter-based approach).

The following suggested changes are proposed with the corresponding text proposal (TP) shown below:

- [TP#1] Device only applies D2R Scheduling Info and generates of RN upon initiating CBRA procedure according to clause 5.2 due to a reception of A-IoT paging message [TP#1 highlighted in green].

- [TP#2] Clarify device behaviour on how the counter-based approach works, highlighting:

  - [TP#2.1] When ACCESS_OCCASION_COUNTER >= m, the device needs to keep monitoring for next Access Trigger message or A-IoT Paging message until Access Random ID message is transmitted or next A-IoT Paging message is received [TP #2.1 highlighted in blue].

  - [TP#2.2] When ACCESS_OCCASION_COUNTER < m, the device only selects an access occasion if the Access Random ID message, for the A-IoT Paging message selecting the device, has not been transmitted [TP #2.2 highlighted in yellow].

  - [TP#2.3] Explicitly capture where the counter-based approach starts and remove of the note [TP #2.3 highlighted in grey].

#### 5.3.1.1 Selection of access occasion for D2R transmission of Access Random ID message If Contention-Based Random Access procedure is initiated according to clause 5.2, the device selects access occasion for D2R transmission of Access Random ID message based on A-IoT Paging message or Access Trigger message. If needed, the device monitors for Access Trigger message until it has received a A-IoT Paging message. Upon initiating contention-Based Random Access procedure according to clause 5.2 due to a reception of A-IoT Paging message, the A-IoT MAC entity shall: 1> apply the D2R Scheduling Info received from the A-IoT Paging message; 1> generate a random number 'i' in the range: 0 ≤ i ≤ n-1, where n is the number of access occasions configured in A-IoT Paging message; For a counter-based behaviour, the access occasions are counted starting with the A-IoT Paging message selecting the device and in each subsequent Access trigger message (until Access Random ID message is transmitted or next A-IoT Paging message is received). For this, the A-IoT MAC entity should: 1> set the ACCESS_OCCASION_COUNTER to 'i'; 1> if the Access Random ID message, for the A-IoT Paging message selecting the device, has not been transmitted and if ACCESS_OCCASION_COUNTER < m, where m equals to X*NSFS (where X and NSFS are defined in clause 6.2.1.6): 2> select the (ACCESS_OCCASION_COUNTER+1)th access occasion from the m access occasion(s) triggered by the A-IoT Paging message; 2> initiate the transmission of Access Random ID message, as specified in clause 5.3.1.2; 1> else (i.e. ACCESS_OCCASION_COUNTER >= m): 2> perform the following procedure upon reception of each Access Trigger message until Access Random ID message is transmitted or next A-IoT Paging message is received if Access Random ID message has not been transmitted: 3> decrement ACCESS_OCCASION_COUNTER by m; 3> if ACCESS_OCCASION_COUNTER < m: 4> select the (ACCESS_OCCASION_COUNTER+1)th access occasion from the m access occasion(s) triggered by this Access Trigger message; 4> initiate the transmission of Access Random ID message, as specified in clause 5.3.1.2. 3> else (i.e., if ACCESS_OCCASION_COUNTER >= m): 4> keep monitoring for next Access Trigger message or A-IoT Paging message; NOTE: The count-down behaviour defined above does not preclude other device implementation alternatives of random selection of access occasion.

- Selection of access occasion for D2R transmission of Access Random ID message (MAC running CR §5.3.1.1) is updated to capture the following updates (with example TPs shown within this document):

  - [TP#1] Device only applies D2R Scheduling Info and generates of RN upon initiating CBRA procedure according to clause 5.2 due to a reception of A-IoT Paging message.

  - Clarify device behaviour on how the counter-based approach works, highlighting:

    - [TP#2.1] When ACCESS_OCCASION_COUNTER >= m, the device needs to keep monitoring for next Access Trigger message or A-IoT Paging message until Access Random ID message is transmitted or next A-IoT Paging message is received.

    - [TP#2.2] When ACCESS_OCCASION_COUNTER < m, the device only selects an access occasion if the Access Random ID message, for the A-IoT Paging message selecting the device, has not been transmitted.

    - [TP#2.3] Explicitly capture the start of counter-based approach and remove the note.

## Termination of CBRA (Success or failure)

It seems unclear/ambiguous when/how the failure of CBRA is triggered (and correspondingly successful).

When/how the failure of CBRA is triggered seems unclear/ambiguous.

The suggestion is to capture when the termination of CBRA is considered as failed in §5.3.1.3 of the A-IoT MAC running CR [1]. For this, two approaches are shown:

- The termination of the CBRA procedure triggering the failure procedure is captured with a general/final statement (i.e. after reception of the [FFS 1st or kth] Access Trigger message).

- The termination of the CBRA procedure triggering the failure procedure is captured repeating the failure conditions (i.e., device does not received a random ID response message including the Echoed Random ID identical to the value of the Random ID field in the transmitted Access Random ID message after transmission of the Access Random ID message and before the [FFS 1st or kth] Access Trigger message).

For alignment, it is also added when the CBRA procedure is considered as successful (§5.3.1.3 in [1]) and the failure detection section is updated accordingly (§5.5 in [1]). Example TPs of these changes are shown below:

#### 5.3.1.3 Reception of Random ID Response message Once the Access Random ID message is transmitted, the device shall monitor for Random ID Response message until it has received [FFS one or k] Access Trigger message or one A-IoT Paging message (i.e., the device does not process the Random ID Response message after that). Upon reception of Random ID Response message, the A-IoT MAC entity shall: 1> if the device has no stored AS ID (i.e., initial reception of Random ID Response message): 2> for each ID entry in Random ID Response message: 3> if the value indicated by Echoed Random ID field is identical to the value of the Random ID field in the transmitted Access Random ID message: 4> consider this CBRA procedure is successful; 4> if the Assigned AS ID field corresponding to the Echoed Random ID field is included (i.e., AS ID Present Indication field is set to 1): 5> set AS ID to the value indicated by the Assigned AS ID field and store the AS ID; 4> else: 5> set AS ID to the value indicated by the Random ID field and store the AS ID; 4> consider this Contention-Based Random Access procedure as successful; 4> initiate the D2R message transmission as specified in clause 5.4.1, upon which the procedure of processing this Random ID Response message ends; 1> else 2> for each ID entry in the Random ID Response message: 3> if the Assigned AS ID field corresponding to the Echoed Random ID field is included, and the value indicated by Assigned AS ID field is identical to the stored AS ID; or 3> if the Assigned AS ID field corresponding to the Echoed Random ID field is not included, and the value indicated by Echoed Random ID field is identical to the stored AS ID: 4> consider this Contention-Based Random Access procedure as successful; 4> initiate the D2R message transmission as specified in clause 5.4.1, upon which the procedure of processing this Random ID Response message ends. <<< TP continues in next page >>>

<<< TP continues from previous page >>> // Approach (a) Termination of the CBRA procedure triggering the failure procedure with general/final statement After transmission of the Access Random ID message and after the [FFS 1st or kth] Access Trigger message, this Contention-Based Random Access procedure shall be considered as failed, as specified in clause 5.5, upon which the procedure of processing this Random ID Response message ends; //Approach (b) Termination of the CBRA procedure triggering the failure procedure repeating the conditions After transmission of the Access Random ID message and before the [FFS 1st or kth] Access Trigger message, if the device has not received a random ID response message including the Echoed Random ID identical to the value of the Random ID field in the transmitted Access Random ID message, the MAC entity shall: 1> consider this Contention-Based Random Access procedure as failed as specified in clause 5.5: …. ## 5.5 Failure detection Once the device transmitted the first D2R Upper Layer Data Transfer message after CBRA procedure, the A-IoT MAC entity shall monitor for NACK Feedback message until the device receives a A-IoT Paging message or R2D Upper Layer Data Transfer message addressed to the device (i.e., the device does not process NACK Feedback message after that). Upon reception of NACK Feedback message, the A-IoT MAC entity shall: 1> for each AS ID entry in the NACK Feedback message: 2> if the value indicated by the AS ID field is identical to the stored AS ID: 3> consider that the current procedure associated with the stored Transaction ID failed, upon which this procedure of processing NACK Feedback message ends. Upon Contention-Based Random Access CBRA procedure is considered as failed as specified in clause 5.3.1.3, the A-IoT MAC entity shall: 1> if CBRA procedure has not been considered as successful as specified in clause 5.3.1.3: 2> consider that the current procedure associated with the stored Transaction ID failed.

- MAC running CR captures when the CBRA procedure is considered a failure or success as part of §5.3.1.3 (with example TPs shown within this document).

  - The termination of the CBRA proc. triggering the failure proc. is captured in §5.3.1.3 via either the TP showing approach (a) or approach (b):

    - [TP approach (a)] A general/final statement (i.e. after reception of the [FFS 1st or kth] Access Trigger message).

    - [TP approach (b)] Repeating the failure conditions (i.e., device does not received a random ID response message including the Echoed Random ID identical to the value of the Random ID field in the transmitted Access Random ID message after transmission of the Access Random ID message and before the [FFS 1st or kth] Access Trigger message).

  - The successful termination of the CBRA proc. is also captured just before initiating the D2R message transmission in §5.3.1.3.

- If Proposal 2 is agreed, to also update failure procedure (§5.5 in MAC running CR) to refer to the Contention-Based Random Access procedure being considered as failed (with example TPs shown within this document).

# Conclusion

The observations captured are the following:

Observation 1.	The device’s behaviour seems unclear when selecting access occasion (AO) for D2R transmission of Access Random ID message for CBRA (e.g., monitoring and actions associated to the Access Trigger message, determination of random number (RN) during a paging cycle and choice of the counter-based approach).

Observation 2.	When/how the failure of CBRA is triggered seems unclear/ambiguous.

The proposals captured are the following:

Proposal 1.	Selection of access occasion for D2R transmission of Access Random ID message (MAC running CR §5.3.1.1) is updated to capture the following updates (with example TPs shown within this document):

Proposal 1.1.	[TP#1] Device only applies D2R Scheduling Info and generates of RN upon initiating CBRA procedure according to clause 5.2 due to a reception of A-IoT Paging message.

Proposal 1.2.	Clarify device behaviour on how the counter-based approach works, highlighting:

Proposal 1.2.1.	[TP#2.1] When ACCESS_OCCASION_COUNTER >= m, the device needs to keep monitoring for next Access Trigger message or A-IoT Paging message until Access Random ID message is transmitted or next A-IoT Paging message is received.

Proposal 1.2.2.	[TP#2.2] When ACCESS_OCCASION_COUNTER < m, the device only selects an access occasion if the Access Random ID message, for the A-IoT Paging message selecting the device, has not been transmitted.

Proposal 1.2.3.	[TP#2.3] Explicitly capture the start of counter-based approach and remove the note.

Proposal 2.	MAC running CR captures when the CBRA procedure is considered a failure or success as part of §5.3.1.3 (with example TPs shown within this document).

Proposal 2.1.	The termination of the CBRA proc. triggering the failure proc. is captured in §5.3.1.3 via either the TP showing approach (a) or approach (b):

Proposal 2.1.1.	[TP approach (a)] A general/final statement (i.e. after reception of the [FFS 1st or kth] Access Trigger message).

Proposal 2.1.2.	[TP approach (b)] Repeating the failure conditions (i.e., device does not received a random ID response message including the Echoed Random ID identical to the value of the Random ID field in the transmitted Access Random ID message after transmission of the Access Random ID message and before the [FFS 1st or kth] Access Trigger message).

Proposal 2.2.	The successful termination of the CBRA proc. is also captured just before initiating the D2R message transmission in §5.3.1.3.

Proposal 3.	If Proposal 2 is agreed, to also update failure procedure (§5.5 in MAC running CR) to refer to the Contention-Based Random Access procedure being considered as failed (with example TPs shown within this document).

# Reference

- [POST130][027][AIoT] MAC Running CR (Huawei), MAC running CR, TS 38.391 V0.0.3 (2025-08).

- [POST130][027][AIoT] MAC Running CR (Huawei), Remaining A-IoT MAC open issues,.