3GPP TSG RAN WG2 Meeting#131                                                         R2-2505268

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.12.3

Source:	Ofinno

Title:	Enhancements for UE-initiated Beam Reporting

Document for:	Discussion and Decision

# Introduction

RAN2 has made the following agreements for UE-initiated Beam Reporting

RAN2#129bis Agreements In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH. Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP. If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP. RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec. RAN2#130 Agreements UL skipping is not applicable to mode-B type-1 CG event-triggered beam report (i.e., MAC PDU is not generated). For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated), there is no MAC impact. The existing rule is applied to handle the overlapping/prioritization between the PUSCH of mode-A UE-initiated report and SR/other PUSCH in MAC. No MAC impact is expected. For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Exact MAC spec can be discussed further When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG. As a baseline, the events defined for UEI BM should be captured as a new reportConfigType which can also include parameters applicable only for one event. This can be captured once more parameters for this objective are defined by RAN1.

In this contribution, we present our views on the remaining issues of UE-initiated beam reporting.

# Open issues

## UL skipping for UE-initiated beam reporting

In RAN2#130, the following agreement has been made regarding Rel-15 UL skipping.

- RAN2#130 Agreement For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Exact MAC spec can be discussed further.

In the MAC running CR [1], the rapporteur has specified a change to reflect the above agreement but added an Editor’s Note for FFS since a company prefers not to specify it in Rel-15, instead relies on NW implementation to configure Rel-16 UL skipping.

MAC running CR [1] The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if there is no mode-A UE-initiated CSI reporting triggered for this PUSCH transmission as specified in TS 38.214 [7]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. Editor’s Note: FFS if the above change is needed for the agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

In Rel-15, for aperiodic CSI report, the uplink grant cannot be skipped even if there is no data in the MAC layer. This is because the aperiodic CSI report is scheduled by a DCI on PUSCH. Based on the TS 38.214, if the MAC does not generate a MAC PDU/TB for a PUSCH scheduled by a DCI, the PHY will not transmit this PUSCH.

As for periodic and semi-periodic CSI reporting, the UL grant is skipped when there is no MAC data available. In that case, CSI report is transmitted on PUCCH. Conversely, if there is some MAC data, CSI report is sent on PUSCH. However, if PUSCH and PUCCH are overlapping, the network cannot distinguish between these two scenarios, i.e., the network does not know the periodic and semi-periodic CSI report is transmitted on PUCCH or PUSCH since it does not know whether the UE has UL MAC data or not, which forces it to attempt decoding CSI report on both PUSCH and PUCCH, leading to unnecessary complexity. To resolve this issue, Rel-16 introduced an enhancement where the UL grant cannot be skipped if any CSI/UCI is expected to be multiplexed on the PUSCH, even when there is no MAC data. This ensures that when PUSCH and PUCCH are overlapping, the network can reliably expect the CSI report on PUSCH.

We believe that the Rel-16 enhancement goes beyond merely addressing the issue present in Rel-15. If the sole objective was to rectify the Rel-15 issue, the most efficient approach would have been to revise the existing Rel-15 feature directly, rather than introduce a new parameter that allows the network to determine whether the Rel-16 UL skipping behavior should be configured for a given UE. This implies that even if a UE supports Rel-16 UL skipping, the network retains the flexibility to configure only Rel-15 UL skipping based on its own considerations. Moreover, although the network can configure Rel-16 UCI multiplexing behavior instead of the legacy Rel-15 behavior, this should also depend on UE’s capability. Rel-16 UL skipping is an optional feature and thus may not be supported by all the UEs.

If we do not explicitly specify the Rel-15 UL skipping behavior for mode-A UE-initiated CSI reporting, there is a risk that triggered mode-A UE-initiated CSI reports cannot be transmitted in scenarios where the UE either does not support Rel-16 UL skipping feature or the network chooses not to configure Rel-16 UL skipping for that UE, as the UE may then skip the UL grant. Therefore, we consider it’s necessary to explicitly specify the Rel-15 UL skipping behavior for the mode-A UE-initiated CSI reporting to eliminate any potential issues.

- Explicitly specify the RAN2 agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

## TAT expiry for sTAG in UE-initiated beam reporting

In the current procedure, when the TAT for an SCell expires, all PUCCH resources and configured uplink grants associated with that SCell are released by the RRC or cleared at the MAC layer, respectively. This can also be applied to UE-initiated CSI reporting because the PUCCH/PUSCH resources cannot be used on a SCell when the TAT of the SCell expires. In particular, if a UE-initiated report configuration uses a PUCCH resource linked to the expired TAT on the SCell, that resource is released by RRC. Similarly, if a Type-1 CG for a UE-initiated report configuration is associated with the expired TAT on that SCell, it is cleared by MAC. The benefit of these behaviors is that the network can reassign the released/cleared resources to other UEs for better resource efficiency and system capacity.

For mode-A, we believe there is no issue to follow the legacy behavior, however, for mode-B, we consider there are two potential cases, as illustrated in Figure 1, worth highlighting for further discussion given the fact that the PUCCH resource and the Type 1 CG may be configured on different serving cells associated with different TAGs. Please note that it’s essential to consider the interdependency between the PUCCH notification and the corresponding CSI report on PUSCH: the PUCCH notification is rendered useless if the corresponding CSI report is not transmitted on the PUSCH, and reciprocally, the CSI report on PUSCH cannot be transmitted without the previous PUCCH notification.

- Case 1: The TAT of the TAG associated with the PUCCH resource (e.g., TAG 1) expires, while the TAT of the TAG associated with the Type 1 CG (e.g., TAG 2) remains running. Following the legacy behavior, the UE releases the PUCCH resource but retains the Type 1 CG. However, since the UE cannot perform the PUCCH transmission for the UEI report, the retained Type 1 CG cannot be utilized.

  - If the UE instead clears the Type 1 CG in such a case, the network could reassign those resources to other UEs, improving overall resource efficiency and system capacity.

- Case 2: Conversely, if the TAT of the TAG (e.g., TAG 2) associated with the Type 1 CG expires while the TAT for the PUCCH-associated TAG (e.g., TAG 1) is still running, following the legacy behavior, the UE clears the Type 1 CG resources. In this case, since the UE cannot use the Type 1 CG, there is also no need to perform the PUCCH transmission for the UEI report.

  - If the UE instead releases the PUCCH resources in such a case, the network could reassign those resources to other UEs, improving overall resource efficiency and system capacity.

- : TAT expiry cases for different Cells with different TAGs

- For the TAT expiry of a sTAG, RAN2 discusses the following two cases when the PUCCH and the Type 1 CG are configured on different Cells with different TAGs:

- Case 1: When PUCCH configured on a SCell is released due to the TAT of the sTAG of the SCell expiry, whether to clear the type-1 CG configured on another Cell.

- Case 2: When type-1 CG configured on a SCell is cleared due to the TAT of the sTAG of the SCell expiry, whether to release the PUCCH configured on another Cell.

## RACH initiation for UE-initiated report under expired TAT

In the MAC running CR email discussion [1], the following has been proposed by the rapporteur for discussion:

MAC running CR email discussion [1] Proposal 2: Regarding whether to initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3).

In legacy systems, once the TAT expires, periodic/semi-persistent CSI reports are not transmitted because the PUCCH/PUSCH resources are released/cleared. Also, the UE does not initiate RACH since the triggering of periodic/semi-persistent CSI reports are under control by network, if the network still wants to have periodic/semi-persistent CSI reports, the network can reconfigure it.

However, there are some differences in UEIBR. UEIBR is an event-driven procedure by the UE. PHY specified some event evaluation mechanisms for the UE side to determine whether to trigger the UEIBR. For Option 1, if the UE does not do anything, some potential issues may arise. For example:

- If the UEIBR has not been triggered during the TAT expiry, whether the UE still needs to perform the event evaluation? This may waste UE’s power if the report cannot be transmitted.

- If the UEIBR has been triggered but cannot be transmitted during the TAT expiry, the triggered UEIBR remains pending, and it is unclear whether the associated counters and timers used for event evaluation are reset or maintained.

- After the UL is resynchronized and the PUCCH/Type 1 CG resources are reconfigured, the UE can start transmitting for the pending UEIBR triggered during the TAT expiry. However, the network may not distinguish whether the UEIBR has been triggered during the TAT expiry or after the UL is resynchronized, and thus though the current serving beam has a high quality, the network may still update the current serving beam.

To resolve these issues, Option 1 can work with the addition of a cross-layer indication to the PHY layer upon TAT expiry. This would allow the PHY layer to cancel a pending UEIBR or stop event evaluation if the UEIBR has not yet been triggered.

For Option 3, we believe initiating RACH can be beneficial when the network is aware of the cause for the RACH procedure, e.g., by sending the cause in Msg3. It is possible to take long time to recover from TAT expiry to UL resynchronized. With this cause indication, the network can reconfigure new resources for UEIBR as early as possible (e.g., in Msg4). However, if there is no cause indication in RACH, the network does not know the UE has triggered the UEIBR during TAT expiry, then reconfiguring the new resources for the UEIBR can be delayed. The delay may result in an outdated report which degrades the beam management performance. An indication in Msg3 is helpful to inform the cause of the RACH procedure. Introducing a new MAC CE to indicate the cause in Msg 3 is feasible since RAR UL grant size may be 48/64bits and the C-RNTI MAC CE only has two octets, the remaining size can include this new MAC CE. RAN2 can discuss how to indicate the cause by this MAC CE, e.g., whether any field is needed or simply using a MAC subheader with (e)LCID.

- Regarding whether to initiate RACH when UEI beam report is triggered but TAT is expired, select Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH.

- The cause of RACH is indicated by a MAC CE via Msg3.

## Cell DTX operation

In the legacy systems, for a serving cell configured with cell DTX, after transmission of an SR on PUCCH, the UE monitors PDCCH on the serving cell regardless of whether the serving cell is in the cell DTX active period or not (see the text copied below from 38.321 Section 5.34.2).

TS 38.321 [2] ## 5.34.2 Cell Discontinuous Transmission For each Serving Cell configured with cell DTX, the MAC entity shall: … 1> if the Serving Cell is in the cell DTX Active Period: 2> monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification. 1> if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or 1> if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or 1> if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or 1> if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a): 2> monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification. 1> if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell: 2> monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

In RAN1#120, the following agreement is made regarding the priority between LRR, normal SR, and first PUCCH carrying UE-initiated report indication.

RAN1#120 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule

According to the RAN1#120 agreement, the first PUCCH carrying a UE-initiated report indication has higher priority than a normal SR. The PUCCH used for Mode A UE-initiated CSI reporting serves a function analogous to that of SR and is even assigned a higher priority. This prioritization signifies that the PUCCH associated with Mode A UE-initiated CSI reporting is deemed more critical than the SR. Therefore, the UE should follow the same behavior as SR to monitor the PDCCH if the PUCCH has been transmitted for mode-A UE-initiated CSI reporting regardless of cell DTX Active Period.

In RAN2#129b, the following agreement regarding DRX active time of Mode A of UE-initiated CSI reporting has been reached.

RAN2#129bis Agreement - In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

If cell DTX is configured, the UE should monitor PDCCH after the first PUCCH transmission to receive the DCI, even if the serving cell is not in the cell DTX active period. This behavior and motivation are consistent with the RAN2#129b agreement on DRX active time of Mode A of UE-initiated CSI reporting copied above.

Moreover, the UE-initiated CSI report is important even outside the current DTX active period, as it prepares the network with timely channel state information for the upcoming DTX active interval. Otherwise, the UE may need to perform the UE-initiated CSI reporting at the beginning of the next active period, leading to a delay in data transmission. Additionally, postponing the UE-initiated CSI reporting for Event 1 until the DTX active period may increase the risk of beam failure due to outdated channel information.

- Regardless of cell DTX Active Period, the UE monitors PDCCH if PDCCH scheduling mode-A PUSCH has not been received after transmitting PUCCH for mode-A UEI beam report.

# Remaining issues

## FR2 UL Gap handling

The UE may perform FR2 transmission power management by using a FR2 UL gap during which regular uplink data transmissions are paused. The uplink slots used for FR2 UL gaps are used for body proximity sensing (BPS). In the legacy systems, in order not to degrade the system performance, the UE can still perform some critical uplink transmissions during FR2 UL gap such as Msg1/Msg3/MsgA, SR, LRR, UL-SCH for configured grant, and CSI/L1-RSRP report during SCell activation (see the text copied below from 38.321 Section 5.30).

TS 38.321 [2] ## 5.30 Handling of FR2 UL gap During the FR2 UL gap configured by ul-GapFR2-Config as specified in TS 38.331 [5], the MAC entity shall, on the Serving Cell(s) of FR2 single CC and intra-band CA, or on the Serving Cell(s) of FR2 inter-band CA where UE does not support tx-Support-UL-GapFR2: 1> only perform transmission of: 2> PRACH preamble as specified in clause 5.1.2 and 5.1.2a; 2> UL-SCH for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 2> UL-SCH for configured grant; 2> the valid CSI report during SCell activation procedure where the valid CSI report is valid CQI with non-zero CQI index defined in TS 38.214 [7], clause 5.2.2.1, when the time period between UL gap colliding with CSI report of non-zero CQI and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the valid L1 RSRP report during SCell activation procedure, where the valid L1 RSRP report is non lowest L1 RSRP defined in TS 38.133 [11], clause 10.1.6, when the time period between UL gap colliding with L1 RSRP reporting and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the PUCCH transmission for SR, and link recovery request (LRR) defined in TS 38.133 [11], clause 8.5.

In RAN1#120, the following agreement is made regarding the priority between LRR, normal SR, and first PUCCH carrying UE-initiated report indicator.

RAN1#120 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule

According to the RAN1#120 agreement, the first PUCCH carrying a UE-initiated report indication has higher priority than a normal SR. The PUCCH used for Mode A UE-initiated CSI reporting serves a function analogous to that of SR and is even assigned a higher priority. This prioritization signifies that the PUCCH associated with UE-initiated CSI reporting is deemed more critical than the SR. Since the normal SR is transmitted during FR2 UL gaps, the UE behavior should also be specified for cases where the higher-priority UE-initiated report indicator in PUCCH overlaps with an FR2 uplink gap. Given that UE-initiated CSI reporting is an important mechanism for maintaining the radio connection, dropping the first PUCCH during an FR2 uplink gap could negatively affect system performance. In our view, the first PUCCH transmission with UE-initiated report indication should be transmitted during FR2 UL gaps.

- During FR2 UL gap, the UE performs PUCCH transmission for UE Initiated Report Indication.

In the case of Mode A, the second PUSCH is dynamically scheduled and can be transmitted in special slots or uplink slots not overlapping with FR2 UL gaps. As such, in our view, handling of this overlap does not require special consideration. However, for Mode B, the Type 1 configured grant PUSCH may fall within the FR2 UL gap. In legacy, the normal CG is transmitted during the FR2 UL gaps. Given that the Type 1 CG for UE-initiated CSI reporting is more important and is a key mechanism for radio connection maintenance, dropping the configured grant PUSCH during the FR2 UL gap could negatively impact system performance. Therefore, the CG type 1 for mode-B UE-initiated CSI reporting should also be transmitted during FR2 UL gaps.

- During FR2 UL gap, the UE performs transmission of CG Type 1 for mode-B UE-initiated CSI reporting.

A text proposal is shown below:

Text Proposal for TS 38.321 ## 5.30 Handling of FR2 UL gap During the FR2 UL gap configured by ul-GapFR2-Config as specified in TS 38.331 [5], the MAC entity shall, on the Serving Cell(s) of FR2 single CC and intra-band CA, or on the Serving Cell(s) of FR2 inter-band CA where UE does not support tx-Support-UL-GapFR2: 1> only perform transmission of: 2> PRACH preamble as specified in clause 5.1.2 and 5.1.2a; 2> UL-SCH for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 2> UL-SCH for configured grant; 2> Configured grant Type 1 for mode-B UE-initiated CSI reporting; 2> the valid CSI report during SCell activation procedure where the valid CSI report is valid CQI with non-zero CQI index defined in TS 38.214 [7], clause 5.2.2.1, when the time period between UL gap colliding with CSI report of non-zero CQI and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the valid L1 RSRP report during SCell activation procedure, where the valid L1 RSRP report is non lowest L1 RSRP defined in TS 38.133 [11], clause 10.1.6, when the time period between UL gap colliding with L1 RSRP reporting and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the PUCCH transmission for SR, UE Initiated Report Indication, and link recovery request (LRR) defined in TS 38.133 [11], clause 8.5.

## Invalid PUCCH resources and Type 1 CG resources for BWP

The UE behavior when a UE-initiated CSI reporting is triggered but no valid PUCCH or PUSCH resource is available for its transmission due to TAT expiry is currently being discussed in RAN2. However, TAT expiry is just one case where valid resources may be unavailable. Another example is when a UE-initiated CSI reporting is triggered, but the uplink BWP configured with the PUCCH resource and/or Type 1 CG PUSCH resource for the UE-initiated CSI reporting is not active.

In legacy, if PUCCH resource for a pending SR is not configured in the active uplink BWP, the UE initiates a RACH procedure. Note that the network has the flexibility of configuring PUCCH resources in each UL BWP. However, according to the current RRC running CR [3], the PUCCH resource and Type 1 CG PUSCH resource for UE-initiated CSI reporting are configured only on a single uplink BWP (i.e., based on a single ul-BWP-Id for pucch-Resource and a single ul-BWP-Id for pusch-ResourceOfModeB-r19). Since the network does not know when the UE will trigger the UE-initiated CSI reporting, the network needs to keep these uplink BWP(s) configured with the PUCCH resource and Type 1 CG PUSCH resource always active. Otherwise, the UE needs to wait until both of these uplink BWP(s) are activated to perform UE-initiated CSI reporting, which delays the CSI reporting.

For example, in Figure 2 below, the PUCCH resource for the UE-initiated CSI reporting is configured on Uplink BWP 2. It is possible that, when the UE triggers the UE-initiated CSI reporting, Uplink BWP 3 is the active BWP for the UE. In this case, the UE cannot transmit PUCCH for the UE-initiated CSI reporting. If the UE must wait for switching back to Uplink BWP 2, this can lead to increased reporting delay or increased signaling overhead (e.g., if it waits for the network to switch BWP). One possible solution is to allow the UE to directly switch to Uplink BWP 2 (which contains the PUCCH resource for the UE-initiated CSI report) when UE-initiated CSI reporting is triggered and the current active BWP is not configured with PUCCH resource for the UE-initiated CSI report. Another approach is to configure multiple PUCCH resources, one on each uplink BWP, so that the UE can immediately transmit the PUCCH transmission for the UE-initiated CSI report without switching BWPs or waiting for the appropriate BWP (e.g., Uplink BWP 2) to be activated.

- : BWP switching for PUCCH transmission of UEIBR

Similarly, in Figure 3 below, the Type 1 CG PUSCH resource is configured on Uplink BWP 2 of CC 0 and the PUCCH resource is configured on CC 1. It is possible that, when the UE transmits the PUCCH for the UE-initiated CSI reporting on CC 1, Uplink BWP 3 on CC 0 is the active BWP for the UE. If the UE must wait for switching back to Uplink BWP 2 on CC 0, this can lead to increased reporting delays or increased signaling overhead (e.g., if it waits for the network to switch BWP). One possible solution is to allow the UE to directly switch to Uplink BWP 2 on CC 0 (which contains the Type 1 CG for the UE-initiated CSI report) when UE-initiated CSI reporting is triggered and the current active BWP is not configured with Type 1 CG for the UE-initiated CSI report. Another approach is to configure multiple Type 1 CG configurations, one on each uplink BWP, so that the UE can immediately transmit the UE-initiated CSI report without switching BWPs or waiting for the appropriate BWP (e.g., Uplink BWP 2) to be activated.

- : BWP switching for type 1 CG transmission in Mode-B of UEIBR

- Support one of the following two options to ensure that the UE can transmit the PUCCH and/or Type 1 CG for UE-initiated CSI reporting without waiting for BWP switching by network when UE-initiated CSI reporting is triggered:

- Option 1: If the PUCCH resources and/or the Type 1 CG resources for UE-initiated CSI reporting are not configured in the activated UL BWP, the UE performs BWP switching to an UL BWP where the PUCCH resource and/or the Type 1 CG PUSCH resource are configured.

- Option 2: The CSI report configuration for UE-initiated CSI reporting configures multiple PUCCH resources and Type 1 CG resources on each UL BWP of a Cell.

## Redundant servCellIndex in the pusch-ResourceOfModeB-r19

In RAN1#118, the following agreement has been reached regarding the CC to transmit the beam report via type 1 CG in Mode-B. In particular, the type 1 CG of a CSI report configuration should be configured on the CC where the CSI report configuration is configured.

RAN1#118 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC.

In the running CR for 38.331 [3], the parameter pusch-ResourceOfModeB-r19 comprises servCellIndex parameter indicating the CC where the configured grant configuration for the type 1 CG carrying the beam report is located. However, the servCellIndex parameter is not needed, as per RAN1#118 agreement, the CC where the configured grant configuration is located is already implicitly determined based on the CC where the CSI report configuration is configured. Since the servCellIndex in the pusch-ResourceOfModeB-r19 should always be configured as the same as servCellIndex in the CSI-ReportConfig, it is considered as redundant.

- Remove servCellIndex-r19 from pusch-resourceOfModeB-r19.

# Conclusion

In this contribution, we make the following proposals.

- Explicitly specify the RAN2 agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

- For the TAT expiry of a sTAG, RAN2 discusses the following two cases when the PUCCH and the Type 1 CG are configured on different Cells with different TAGs:

- Case 1: When PUCCH configured on a SCell is released due to the TAT of the sTAG of the SCell expiry, whether to clear the type-1 CG configured on another Cell.

- Case 2: When type-1 CG configured on a SCell is cleared due to the TAT of the sTAG of the SCell expiry, whether to release the PUCCH configured on another Cell.

- Regarding whether to initiate RACH when UEI beam report is triggered but TAT is expired, select Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH.

- The cause of RACH is indicated by a MAC CE via Msg3.

- Regardless of cell DTX Active Period, the UE monitors PDCCH if PDCCH scheduling mode-A PUSCH has not been received after transmitting PUCCH for mode-A UEI beam report.

During FR2 UL gap, the UE performs PUCCH transmission for UE Initiated Report Indication.

- During FR2 UL gap, the UE performs transmission of CG Type 1 for mode-B UE-initiated CSI reporting.

- Support one of the following two options to ensure that the UE can transmit the PUCCH and/or Type 1 CG for UE-initiated CSI reporting without waiting for BWP switching by network when UE-initiated CSI reporting is triggered:

- Option 1: If the PUCCH resources and/or the Type 1 CG resources for UE-initiated CSI reporting are not configured in the activated UL BWP, the UE performs BWP switching to an UL BWP where the PUCCH resource and/or the Type 1 CG PUSCH resource are configured.

- Option 2: The CSI report configuration for UE-initiated CSI reporting configures PUCCH resources and Type 1 CG resources on each UL BWP of a Cell.

- Remove servCellIndex-r19 from pusch-resourceOfModeB-r19.

# References

- [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung).

- TS 38.321 V18.6.0; Medium Access Control (MAC) protocol specification.

- [Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson).