3GPP TSG-RAN WG2 #131	R2-2505293

Bengaluru, India, August 25th - 29th, 2025

Source:                    	DENSO CORPORATION

Title:  	Remaining consideration on NR NTN downlink coverage enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.8.2		Downlink coverage enhancements

- Introduction

According to the chair notes for last 2 meetings RAN2#129bis[1] / #130[2], the following agreements are made for downlink coverage enhancement:

RAN2 #129bis:

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

RAN2 #130:

Agreements:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

This paper attempts to investigate about remaining consideration on downlink coverage enhancement based on the above agreements.

- Discussion

In the last meeting (RAN2#130), it was marked as working assumption to introduce a mechanism to assist the network to configure the SMTCs in connected mode. We think this working assumption should be confirmed because the assistance information is beneficial to configure suitable SMTCs for UE in the large coverage of NTN operation. As discussed in the last meeting, it would be the one of solution to enhance the existing D1/D2 event for the assistance mechanism. However, using enhanced D1/D2 event may cause bigger signalling overhead since multiple ReportConfig (each config has 2 reference locations and 2 thresholds) need to be configured, and specification might be complicated for handling special case. Therefore, we think the working assumption should be confirmed with adopting the options Alt 1 or Alt 2 as listed in the chair’s note. For the options, we slightly prefer to adopt Alt 1, since it seems to be simple and straightforward.

Proposal 1: RAN2 to confirm the working assumption to introduce a mechanism to assist the network to configure the SMTCs in connected mode with adopting Alt 1 (UE provide the closest N reference locations/neighbor cells to network)

Furthermore, concrete message and sending conditions were not explicitly mentioned in the working assumption. As some companies already discussed, UEAssistanceInformation can be used for the sending message, as the location related information is just assistance information. Additionally, we think OtherConfig in RRCReconfiguration can be used to allow UE to send UEAssistanceInformation and configure the sending conditions such as reference locations, threshold distance for update, etc.

Proposal 2: RAN2 to adopt UEAssistanceInformation to send assistance information for SMTCs configuration

Proposal 3: Network can use OtherConfig in RRCReconfiguration to allow UE to send assistance information for SMTCs configuration, and configure the sending conditions (reference locations, threshold distance for update, etc.)

Besides the assistance mechanism, whether to support SSB-index based SMTC selection was marked as FFS. Since RAN2 has already agreed to adopt location based SMTC selection, this would be redundant and requires more specification effort. Therefore, we would like to propose:

Proposal 4: SSB-index based SMTC selection will not be supported in Rel-19

- Summary and proposal

This paper discussed about consideration on cell barring for downlink coverage enhancement and proposed the following:

Proposal 1: RAN2 to confirm the working assumption to introduce a mechanism to assist the network to configure the SMTCs in connected mode with adopting Alt 1 (UE provide the closest N reference locations/neighbor cells to network)

Proposal 2: RAN2 to adopt UEAssistanceInformation to send assistance information for SMTCs configuration

Proposal 3: Network can use OtherConfig in RRCReconfiguration to allow UE to send assistance information for SMTCs configuration, and configure the sending conditions (reference locations, threshold distance for update, etc.)

Proposal 4: SSB-index based SMTC selection will not be supported in Rel-19

- References

[1] R2-2502983, “Report from Break-out session on NR-NTN and IoT-NTN”

[2] R2-2504673, “Report from Break-out session on NR-NTN and IoT-NTN”