3GPP TSG-RAN WG2 Meeting #131	R2-2505365

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on MAC open issues for random access in SBFD

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#130 meeting, RAN2 achieved the following agreements about the impacts on random access operation by the evolution of duplex operation:

Agreements on MAC open issues - For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion. - Msg1 repetition number fallback can be supported within SBFD RO. - Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. - For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. - The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available. Other agreements - SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA. - For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. - For PRACH configuration option 1, same featureCombinationPreamblesList configuration is used for both Additional-ROs and Legacy-ROs.

In this contribution, we will further analyse the remaining RAN2 impacts to support random access in SBFD symbols based on the MAC open issue list.

# 2	Discussion

During the discussion of open issue in MAC, there are some issues for companies to further discuss via contributions:

[Issues for further discussion via contributions] [Open Issue MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA? [Open Issue MAC-2-2] If so, please clarify what the changes should be. [Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale. [Open Issue MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to: - Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE). [Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale. [Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale. [Open Issue MAC-5] Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.

For [Open Issue MAC-2-1] and [Open Issue MAC-2-2]:

For the case CFRA fallback to CBRA, we agree with MAC spec rapporteur that the RACH resource set is reused in the sense that the RACH resource of same RO type in the same RACH resource set is always available. There is no reason that the RACH resources can only be used for CFRA and not for CBRA. In other word, we think the condition of “when the RACH resources for the same RO type is provided for CBRA” is always satisfied hence not needed.

[Proposal for MAC-2-1 and MAC-2-2]: The condition of “when the RACH resources for the same RO type is provided for CBRA” is always satisfied, hence no need to make any additional MAC specification changes (from current MAC running CR).

For [Open Issue MAC-3-1] and [Open Issue MAC-3-2]:

We assume SBFD configuration for CBRA in a cell would be a quite static configuration and UE would be able to read the SBFD configuration in case it is configured to do CFRA. It would be a strange implementation gNB chooses not to configure SBFD for CBRA meanwhile configuring a UE to do CFRA with SBFD resource. Thus, we don’t think the additional specification changes (in MAC and/or RRC) are needed, we can assume the gNB can configure the proper CFRA configuration to UE. In other words, the reasonable network implementation would only indicate RO type as SBFD RO for CFRA if an SBFD RACH configuration is configured for CBRA and there is no point to explicitly mandate such network behavior in the specs.

[Proposal for MAC-3-1 and MAC-3-2]: There is no issue regarding how to derive the SBFD RO locations for CFRA, and no need for any additional specification changes accordingly.

For [Open Issue MAC-4-1] and [Open Issue MAC-4-2]:

When the RO type falls back, the UE shall be able to initial the parameters configured in RACH-ConfigCommon, considering that the RACH-configCommon are configured separately for RACH configuration 2, and all parameters may be different for SBFD RO and legacy RO. Thus, we think it is better to initiate all parameters in RACH-ConfigCommon if the sbfd-RACH-DualConfig is configured after the RO type fallback. Regarding whether to do partial initialization of parameters (plus partial reusing of existing parameters), there might be the need for additional discussion on which parameters to reuse as well as the need of involving RAN1. Also the benefits of partial initialization for UE implementation seems to be not clear, we propose not to consider it.

[Proposal for MAC-4-1]: Support to initiate the parameters after RO type switching in the case of RACH configuration Option 2.

[Proposal for MAC-4-2]: Initiate all parameters in RACH-ConfigCommon in RACH configuration Option 2.

For [Open Issue MAC-5]:

For this issue, we believe that RAN1 has excluded such solution to introduce the power ramping offset, and the power gap of two type of RO is considered as acceptable. As this issue is mainly within RAN1 domain and they have already had the agreement (to not consider a compensation), it is appropriate for RAN2 not to further discuss this issue.

[Proposal for MAC-5]: RAN2 doesn't need to discuss the issue of power ramping difference compensation during RO type fallback.

# 3	Conclusion

Based on the above discussion, we have the following proposals for MAC open issues:

[Proposal for MAC-2-1 and MAC-2-2]: The condition of “when the RACH resources for the same RO type is provided for CBRA” is always satisfied, hence no need to make any additional MAC specification changes (from the current MAC running CR).

[Proposal for MAC-3-1 and MAC-3-2]: There is no issue regarding how to derive the SBFD RO locations for CFRA, and no need for any additional specification changes accordingly.

[Proposal for MAC-4-1]: Support to initiate the parameters after RO type switching in the case of RACH configuration Option 2.

[Proposal for MAC-4-2]: Initiate all parameters in RACH-ConfigCommon in RACH configuration Option 2.

[Proposal for MAC-5]: RAN2 doesn't need to discuss the issue of power ramping difference compensation during RO type fallback.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)