3GPP TSG-RAN WG2 Meeting #131											R2-2505375

Bengaluru, India, Aug 25th – 29th, 2025

Source: 		ZTE Corporation, Sanechips

Title:                 Open issues for paging

Agenda item:   8.2.2

Document for: Discussion and Decision

# Introduction

In this contribution we take a look at the remaining open issues for paging procedure.

# Paging message contents

# Transaction ID format and length

RAN2 agreed that the transaction ID is generated by the reader based on the CN correlation ID. However, the exact mechanism to generate the transaction ID was left as FFS. It should be noted that the transaction ID is used to indicate the following:

- Correlation ID (i.e. service related)

- Reader ID (different readers may use different transaction IDs)

- Delta paging (paging and reaccess will be differentiated by using the transaction ID)

1 and 2 above are related to network deployment and how the CN information is processed in the network. So, these issues are not related to RAN2. For 3), all we need to specify is device behaviour for the same and different transaction ID and this is already taken care of in the MAC procedure. Based on this, we think there is no need to specify any network behaviour for how transaction ID is derived.

Proposal 1 (Issue 1-2): The exact reader behaviour for how transaction ID is generated need not be specified

Then, since the transaction ID needs to cover all the above issues (correlation ID, differentiate the readers, delta paging), considering something like 2 bits for each of these, we propose that a transaction ID of 6 bits is adopted.

Proposal 2 (Issue 1-2): The size of the transaction ID should be 6 bits

# Transaction ID for CFRA

In case of CFRA as a base line it was agreed that transaction ID is absent. However, with this approach, the question is how to release the ASID. Unless a new explicit message is defined for releasing the ASID (which is not preferable), the device will have to release the ASID upon receiving any paging message. This means that the reader is not allowed to send any subsequent paging until the CFRA procedure is complete. It should be noted that in case of command, there is a significant CN round trip delay for the command execution. During this time, the reader is not allowed to initiate any other paging. Furthermore, if CT1 concludes that the write procedure needs to wait until the write command is successfully executed on the device side, then the overall command procedure for write will go even longer. Keeping the reader occupied for such long time for CFRA essentially makes CFRA unusable for command case.

Observation 1: If there is no transaction ID in CFRA paging message, then the reader cannot send any other paging until the CFRA procedure for a single device is completed

Observation 2: If the procedure for CFRA takes a long time (e.g. for command and/or command with write), then keeping the reader dedicated for this entire time for a single device is not good

For the above scenarios, one easy way is to also include transaction ID in CFRA case. In this case, the reader can initiate other procedures (either CFRA or CBRA), using the same transaction ID and serve other devices at the radio level. The ASID release procedure then for CFRA and CBRA would be very similar and this causes no additional complexity. If this is not acceptable, the CFRA procedure is essentially useless for procedures that take long time (like command) and the reader should then use CBRA instead (even with a single device).

Proposal 3 (Issue 1-5): RAN2 to discuss the following options for the CFRA procedure

Option 1: Include transaction ID also in CFRA case and the device behaviour for ASID release is same as CBRA

Option 2: No transaction ID is included in CFRA, but the reader uses CBRA for single device (e.g. in case of command)

# Security parameters in paging

An LS on security parameters in paging message was received from SA3 in [1]. According to the LS, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. And SA3 requests RAN2 to confirm the feasibility of including additional security parameters and their maximum size.

S3-252392 SA3 is currently working on the authentication procedure. For this procedure, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size.

According to TR 38.769, in both R2D and D2R directions, a maximum TB size of around 1000 bits can be supported. Furthermore, in R2D direction there is no possibility to support segmentation. So, it can be assumed that the maximum size of paging message is around 1000bits. So RAN2 should inform that including additional security parameter (e.g. a 128-bit random number) in the paging message is feasible, but it should be noted that there is an overall limit on the total size of any R2D message in general (1000 bits) and hence any additional security related parameters should fit within this overall size limit and this information should be conveyed to SA3.

Furthermore, it is suggested that RAN2 waits for SA3 further progress before finalizing the security parameters in paging message considering that the intention of the incoming LS is to check the feasibility of including security parameters in the paging message. The final security framework in SA3 is not stable yet and including the 128-bit random number in the paging message is only mentioned for the authentication procedure. On the other side, there is still ongoing discussion regarding the ID privacy protection and the final solution for overall security framework may impact also the authentication procedure. So, RAN2 should keep the design flexible and only finalize the overall paging message format once SA3 design is finalize for this aspect.

Proposal 4 (Issue 1-7): RAN2 replies to SA3 that it is feasible to include 128 bits security parameter in paging message, and the maximum size of paging message is around 1000bits and hence it is preferable, where possible, to minimize the overhead in the paging message.

Proposal 5 (Issue 1-7): RAN2 should keep the paging message design flexible to incorporate the final SA3 framework for security.

# Conclusion

Proposal 1 (Issue 1-2): The exact reader behaviour for how transaction ID is generated need not be specified

Proposal 2 (Issue 1-2): The size of the transaction ID should be 6 bits

Observation 1: If there is no transaction ID in CFRA paging message, then the reader cannot send any other paging until the CFRA procedure for a single device is completed

Observation 2: If the procedure for CFRA takes a long time (e.g. for command and/or command with write), then keeping the reader dedicated for this entire time for a single device is not good

Proposal 3 (Issue 1-5): RAN2 to discuss the following options for the CFRA procedure

- Option 1: Include transaction ID also in CFRA case and the device behaviour for ASID release is same as CBRA

- Option 2: No transaction ID is included in CFRA, but the reader uses CBRA for single device (e.g. in case of command)

Proposal 4 (Issue 1-7): RAN2 replies to SA3 that it is feasible to include 128 bits security parameter in paging message, and the maximum size of paging message is around 1000bits and hence it is preferable, where possible, to minimize the overhead in the paging message.

Proposal 5 (Issue 1-7): RAN2 should keep the paging message design flexible to incorporate the final SA3 framework for security.

- References

- R2-2505064	LS on security parameter in paging request message (S3-252392)