3GPP TSG-RAN WG2 Meeting #131		R2-2505396

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on LP-WUS/WUR in RRC_CONNECTED for RAN2 as below listed from [1] and [2].

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-5 | whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2 Editor’s NOTE: There is no conclusion on whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

Editor’s NOTE:	FFS in Option 1-2 whether the UE should start the lpwus_PDCCHMonitoringTimer (as if LP-WUS was detected) when the UE is not able to monitor the LP-WUS occasion(s).

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-X1 | The impact to the BWP switching mechanism | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| MAC-X2 | Whether to consider the multiple LP-WUS cycles? | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

| MAC-X3 | UAI of the LP-WUS preference | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

- Discussion

  - UE operation under collision

Regarding the following MAC open issue:

Editor’s NOTE: The working assumption for UE operation under collision for Option 1-1 needs to be confirmed. Editor’s NOTE: FFS in Option 1-2 whether the UE should start the lpwus_PDCCHMonitoringTimer (as if LP-WUS was detected) when the UE is not able to monitor the LP-WUS occasion(s).

Companies discussed the collision for Option 1-1 and Option 1-2 in [2], most companies agree to confirm the working assumption for Option 1-1:

- Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s), the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

While companies’ views are diverse for UE operation in case the potential collision for Option 1-2 with the following options are considered:

- Option 1: UE starts the lpwus-PDCCHMonitoringTimer when there is collision. (Same operation as Option 1-1)

- Option 2: UE does not start the lpwus-PDCCHMonitoringTimer when there is collision.

- Option 3: NW can configure UE whether to start the lpwus-PDCCHMonitoringTimer or not when there is collision.

Companies supporting Option 1 have concerns on UE missing potential scheduling and want to reduce the data transmission latency. However, in our view, for option 1-2, for the collision cases/scenarios when the UE is not able to monitor the LP-WUS occasion(s), it is more reasonable that the UE does NOT start the lpwus_PDCCHMonitoringTimer. Even RAN1 doesn’t have explicit agreements on Option 1-2 periodicity, it is typical that Option 1-2 LP-WUS has shorter periodicity and more frequent LP-WUS monitoring occasion configuration. Network has sufficient flexibility and opportunities to send LP-WUS to timely wake up MR. If we follow the same solution as for option 1-1, considering the dense LP-WUS monitoring occasions and short periodicity which results in more frequent collisions, the MR will always be woken up unnecessarily and the power consumption will be increased significantly. Therefore, a more reasonable solution should be that for Option 1-2, the UE only wakes up upon detecting a LP-WUS.

Proposal 1: In Option 1-2, when the UE is not able to monitor the LP-WUS occasion(s), the UE does not start the lpwus-PDCCHMonitoringTimer.

  - (RRC-5) Empty UAI on offset for option 1-1 and option 1-2

RAN2 agreed in RAN2#129bis meeting that UE could provide the preferred time offset via UAI signalling[4].

If configured, the UE can signal a preferred time offset via UAI signalling. RAN2 assumes the design of UAI reporting for preferred time offset is same as the legacy, e.g. including the configuration, procedure, as well as prohibit timer, etc.

Regarding the issue whether it is allowed to report an empty UAI offset for LP-WUS monitoring for both option 1-1 and option 1-2 as specified in RRC-5, RAN2 discussed it in RAN2#130 meeting. But there is no conclusion during the online discussion, as companies need more time to check it.

In our understanding, there is no need to restrict on UE couldn’t report an empty UAI offset, similar as legacy UAI report for power saving. In this case, it means UE has no preference for the time offset between LP-WUS and PDCCH monitoring.

Proposal 2: (RRC-5) It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2, which means UE has no preference on the time offset between LP-WUS and PDCCH monitoring.

  - (MAC-X1) LP-WUS impact on bwp-InactivityTimer

Regarding the following MAC open issue:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-X1 | The impact to the BWP switching mechanism | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

Since in current MAC spec, it specifies that UE should restart the bwp-InactivityTimer when UE receives the PDCCH indicating downlink assignment or uplink grant on the active BWP, which is as follows:

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer: 1> if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or - if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or omit 2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or 2> if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or 2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or 2> if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or 2> if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast: 3> if there is no ongoing Random Access procedure associated with this Serving Cell; or 3> if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5): 4> start or restart the bwp-InactivityTimer associated with the active DL BWP.

Observation 1: bwp-InactivityTimer will be started or restarted when PDCCH is received.

If LP-WUS is used and when there is data transmission for UE, the network will send the LP-WUS firstly. Correspondingly, the UE will firstly receive the LP-WUS indicating to start the drx-OnDurationTimer (for option 1-1) or lpwus-PDCCHMonitoringTimer (for option 1-2) for the reception of the above PDCCH(s) which is highlighted in green. Thus it is straightforward for UE to start or restart the bwp-InactivityTimer associated with the active DL BWP upon UE receives the LP-WUS.

Otherwise, in case UE receives the LP-WUS while the bwp-InactivityTimer associated with the active DL BWP expires before PDCCH reception via MR on active BWP, i.e., BWP1, as shown in the following figure, the UE will switch to the default BWP or initial BWP. But the network may not be aware of this, so the PDCCH is still scheduled on the active BWP. The UE may miss the PDCCH, which will impact the data transmission/latency.

Fig.1: time for UE to wake up -> time offset for UE to wake up.

Observation 2: Data transmission and latency will be impacted in case UE receives the LP-WUS with wake up indication while the bwp-InactivityTimer associated with the active DL BWP expires before PDCCH reception.

Based on the LS from RAN1 [3], the time offset between LP-WUS and PDCCH reception will be up to 37ms, which means it is not a corner case for the scenario in Observation 2.

UE time offset preference Agreement: For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types - V1=5ms - V2=13ms - V3=37ms Agreement: For the UAI of preferred time offset for LP-WUS monitoring in RRC connected mode, - The candidate values are same as those for the UE capability of minimum time gap, i.e., {V1=5, V2=13, V3=37}ms - The reported UAI value is equal to or longer than the minimum time gap reported by UE capability for a UE Note: There is no change to the RAN1 agreed definition for UAI

In our understanding, in order to avoid such problem, the most straightforward solution is to start or restart the bwp-InactivityTimer when receiving LP-WUS. Typically, when receiving LP-WUS, which means there would be PDCCH scheduling subsequently, it is reasonable for UE to keep on the active BWP.

Proposal 3：(MAC-X1) Start or restart the bwp-InactivityTimer associated with the active DL BWP in case UE receives the LP-WUS which indicates to start the drx-OnDurationTimer or lpwus-PDCCHMonitoringTimer.

  - (MAC-X2) Multiple LP-WUS cycles for Option 1-2

As we specified in section 2.1, it is typical that Option 1-2 LP-WUS has shorter periodicity. However, RAN1 only has the following agreement on the periodicity for Option 1-2, which means it is possible that the LP-WUS cycle could be configured very different:

- Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

For C-DRX, we introduced the long C-DRX cycle and short C-DRX cycle to match different data transmission in different periodicities. For example, when UE has sporadic or infrequent transmissions, the long C-DRX cycle will be used; on the other hand, when UE has frequent transmissions, the short C-DRX cycle will be applied. This mechanism balances UE power saving and data transmission delay efficiently and flexibly.

Since when Option 1-2 LP-WUS is used, the drx-onDurationTimer is not started, which means dynamic data transmission depends totally on LP-WUS periodicity, which is similar to the legacy C-DRX control.

Observation 3: Two sets of DRX cycles is configured to match different data transmissions in different periodicities.

Thus, we think the same mechanism should be introduced for LP-WUS Option 1-2, which means long LP-WUS cycle and short LP-WUS cycle will be introduced, and using MAC CE or timer to control the switching between long LP-WUS cycle and short LP-WUS cycle, similar to the C-DRX.

Proposal 4: (MAC-X2) For Option 1-2, two LP-WUS cycles will be configured. RAN2 assumes the design of switching between long LP-WUS cycle and short LP-WUS cycle is same as the C-DRX mechanism, including the MAC CE control, time control.

  - (MAC-X3) Enable and disable for LP-WUS monitoring

We added an open issue in[2], which is as follows:

| MAC-X3 | UAI of the LP-WUS preference | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In our view, it is similar as the RAN1 issue that whether support UAI for LP-WUS enable and disable. RAN1 has also agreed that RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality, as below.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID, current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary, which is aligned with current MAC specification.

Proposal 5: (MAC-X3) In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, we want to clarify the following terms firstly:

- Enable for LP-WUS monitoring: Which is to enable the LP-WUS functionality while UE could use LP-WUS only if the LP-WUS functionality is enabled and is activated if there is some activation mechanisms for LP-WUS;

- Activation for LP-WUS monitoring: When LP-WUS function is enabled, it is possible for network to activate the LP-WUS monitoring, and in this case UE could monitor LP-WUS; otherwise if the LP-WUS is not enabled, the activation procedure for LP-WUS is not expected to happed;

- Deactivation for LP-WUS monitoring: After LP-WUS monitoring is activated, upon the corresponding deactivation mechanism is performed, UE stops using the LP-WUS functionality.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, and the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Besides, according to the following RAN1 agreement, it could be observed LP-WUS could be used if it is enabled, thus there is no other additional activation/deactivation mechanism for LP-WUS. Which is aligned with current MAC specification.

Conclusion From RAN1 perspective, when LP-WUS monitoring is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers MR PDCCH monitoring according to the legacy UE behaviour - Above applies at least for Option 1-1

Proposal 6: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

  - CSI report behaviour within the Active time triggered by LP-WUS

For LP-WUS Option 1-2, in RAN2#128 meeting, we agreed that the drx-ondurationTimer is not started if Option 1-2 LP-WUS is configured and in this case, during the non-Active time configured by drx-onDurationTimer UE doesn’t report the semi-periodic CSI and report the periodic CSI/L1-RSRP based on network configuration [5], for example:

- If lpwus-TransmitOtherPeriodicCSI is configured with value true, UE reports periodic CSI;

- If lpwus-TransmitPeriodicL1-RSRP is configured with value true, UE reports periodic L1-RSRP.

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Observation 4: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

However, we have not discussed the UE behaviour on CSI report during the Active Time triggered by LP-WUS Option 1-2. According to the current MAC running CR [6], UE behaviors on CSI report is the same as other Active Time when UE in the C-DRX Active time, i.e., lpwus-PDCCHMonitoringTimer triggered by LP-WUS is running.

The current CSI report behavior within the drx-onDurationTimer and other C-DRX Active Time is as follows:

- In the Active time triggered by drx-onDurationTimer: UE report the periodic CSI on PUCCH, semi-periodic CSI on PUCCH and PUSCH;

- In the Active time triggered not but others than drx-onDurationTimer: UE report semi-periodic CSI on PUCSCH, and:

  - if CSI-mask is set to true, UE doesn’t report periodic CSI and semi-periodic CSI on PUCCH;

  - if CSI-mask isn’t set to true, UE report periodic CSI and semi-periodic CSI on PUCCH.

Since when LP-WUS Option 1-2 is configured, the drx-onDurationTimer is not started, then the Active Time triggered by LP-WUS is most frequently compared with Active Time caused by other triggers, thus we think the LP-WUS triggered C-DRX Active Time is more similar to the legacy drx-onduraionTimer rather than C-DRX Active Time caused by other triggers. And the only difference of CSI report between the drx-onduraionTimer and other C-DRX Active Time is in case CSI-mask is set to true.   Accordingly, the CSI/L1-RSRP reporting behaviour in LP-WUS triggered C-DRX Active Time should be same as the legacy drx-ondurationTimer. Specifically, UE reports the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer triggered by LP-WUS is running. Otherwise, insufficient CSI reporting may cause the failure of data transmission during Active Time when CSI-mask is set to true.

Proposal 7: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

- Conclusion

In this contribution, we discuss the remaining issues on LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: bwp-InactivityTimer will be started or restarted when PDCCH is received.

Observation 2: Data transmission and latency will be impacted in case UE receives the LP-WUS with wake up indication while the bwp-InactivityTimer associated with the active DL BWP expires before PDCCH reception.

Observation 3: Two sets of DRX cycles is configured to match different data transmissions in different periodicities.

Observation 4: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Proposals:

Proposal 1: In Option 1-2, when the UE is not able to monitor the LP-WUS occasion(s), the UE does not start the lpwus-PDCCHMonitoringTimer.

Proposal 2: (RRC-5) It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2, which means UE has no preference on the time offset between LP-WUS and PDCCH monitoring.

Proposal 3：(MAC-X1) Start or restart the bwp-InactivityTimer associated with the active DL BWP in case UE receives the LP-WUS which indicates to start the drx-OnDurationTimer or lpwus-PDCCHMonitoringTimer.

Proposal 4: (MAC-X2) For Option 1-2, two LP-WUS cycles will be configured. RAN2 assumes the design of switching between long LP-WUS cycle and short LP-WUS cycle is same as the C-DRX mechanism, including the MAC CE control, time control.

Proposal 5: (MAC-X3) In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 6: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is configured, no additional indication/condition is needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

Proposal 7: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

- References

- R2-2505393, Discussion summary and list of RRC open issue for LP-WUS WUR(vivo)

- R2-2505477, Report of [Post130][213][LPWUS] Running CR for TS 38.321 (Apple)

- R2-2504961, Reply LS on LP-WUS in RRC_CONNECTED

- RAN2#129bis meeting minutes

- RAN2#128 meeting minutes

- R2-2503807	 Running MAC CR for LP-WUS Apple