3GPP TSG-RAN WG2 Meeting #131	R2-2505421

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.8.2

Source:	Samsung

Title:	Open issues on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the following open issues for DL coverage enhancement.

- SMTC enhancement of extending number of periodicities and offsets, based on RAN4 LS reply.

- FFS location-based SMTC selection for connected mode.

- UE capability report of Msg4 PDSCH repetition in Msg3 PUSCH, based on RAN1 LS.

# Discussion

## Extending the number of SMTC periodicities and offsets

For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, RAN4 in the LS reply (see Appendixt) has confirmed the technical feasibility but expressed concern on the workload in Rel-19 timeframe. RAN2 need to discuss whether to confirm SMTC enhancement on two periodicities to be supported in Rel-19.

Observation 1: For the SMTC enhancement on configuring two SMTC periodicities per frequency, RAN4 LS indicates concern on the workload in Rel-19 timeframe.

If to support up to 2 SMTC periodicities per frequency, it can be either a frequency same as the serving cell’s frequency (intra-frequency case) or a frequency different from the serving cell’s frequency (inter-frequency case). For intra-frequency case, neighbor cells can have a different SSB periodicity than the serving cell’s SSB periodicity. For inter-frequency case, neighbor cells on this same frequency can have different SSB periodicities, which can be different from the serving cell’s periodicity. So it we agree to support up to 2 SMTC periodicity, it is needed for each frequency.

Observation 2: If support up to 2 SMTC periodicities per frequency, new SMTC signaling is needed in SIB2, SIB4, and measurement object configuration.

For the SMTC enhancement on configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, RAN4 in the LS reply has confirmed the technical feasibility and reminded the current UE behavior supports up to parallelSMTC-r17 (i.e., 4) SMTCs per frequency. This is aligned with RAN2 agreement that UE will not support more than 4 SMTCs per frequency and it is up to UE implementation to select from all the configured SMTCs.

One open issue is the stage-3 detail of how to signal up to 6 SMTCs considering the existing smtc4list which supports up to 4 SMTCs (including smtc1). We have agreed in the last meeting to introduce a new list.

To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

In our understanding of the above agreement, a new SMTC list can be introduced, i.e., smtc5list, which is a list including up to 5 elements of SSB-MTC5, and each element SSB-MTC5 contains a list of PCI, an offset, and a periodicity (if agreed 2 periodicities). In order to save signaling overhead, if a parameter (PCI list, or periodicity or offset) of an element SSB-MTC5 in smtc5list is absent, the corresponding parameter of the element in the same position in smtc4list is applied.

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

smtc5list-r19                       SSB-MTC5List-r19                                OPTIONAL         -- Need R

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..5)) OF SSB-MTC5-r17           OPTIONAL         -- Need R

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

periodicity-r19(TBD)         ENUMERATED {sf10, sf20, sf40, sf80, sf160}

offset-r19                   INTEGER (0..159)

}

Proposal 1: Introduce smtc5list, which is a list including up to 5 elements of SSB-MTC5, and each element SSB-MTC5 contains a list of PCI, an offset, and a periodicity (if agreed 2 SMTC periodicities). If a parameter (PCI list, or periodicity or offset) of an element SSB-MTC5 in smtc5list is absent, the corresponding parameter of the element in the same position in smtc4list is applied.

Proposal 2: If both smtc4list and smtc5list are configured, smtc5list is applied if supported and smtc4list is ignored.

## SMTC in RRC connected

For RRC_CONNECTED, RRM measurement objects and the associated SMTCs are UE dedicated configuration controlled by NW. NW should configure SMTC(s) subject to the UE capability of supported SMTC number (i.e., at most 4). We have agreed that at any time UE does not consider more SMTCs than previous release. Hence, at most 4 SMTCs per frequency layer are configured for RRC_CONNECTED.

Proposal 3: For RRC_CONNECTED, the number of SMTCs configured per frequency layer in measurement object is at most 4 as legacy.

To configure SMTC(s) for neighbor cells that are close to UE, NW may need to know an approximate UE location.  From NW perspective, UE location can be obtained by coarse location report, subject to user consent.

- NW configures coarseLocationRequest in UEInformationRequest message, and UE reports location if available in UEInformationResponse message.

- NW configures coarseLocationRequest and/or includeCommonLocationInfo in the measurement report configuration, and UE reports location in periodical or event-triggered measurement report.

UE location can also be estimated by the NW based on the eventD1/D2 in event-triggered measurement reporting.

- NW configures eventD1/D2 for event-triggered measurement reporting to track UE distance to reference locations.

Observation 3: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location estimated from event-triggered measurement reporting with eventD1/D2, and can update SMTC based on the latest report.

In the last meeting, the following working assumption has been reached for SMTC enhancement for RRC connected mode.

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

The discussion point is whether any additional mechanism is needed considering the existing eventD1/D2.

One of the common issues for Alt1 and Alt2 is that the closest reference locations or the selected SMTCs may change as UE moves or serving/neighbor cell moves. To solve this issue, event-triggered reporting is needed and moving reference location for earth-moving cell also needs to be considered. This will end up with similar design as the existing eventD1/D2. For such duplicated functionality, we think it is not necessary.

Observation 4: Closest reference location(s) or selected SMTCs may change as UE or serving/neighbor cell moves. This leads to event-triggered reporting and moving reference location for earth-moving cells, thus, duplicating functionality as eventD1/D2.

One concern raised for eventD1/D2 is that it takes time for UE to trigger report for eventD1/D2 which may delay NW configuration. However, reporting UE assistance information for Alt1 or Alt2 also depends on the UE’s availability of the information and up to UE implementation when to send. In our view, if NW has no location reporting/assistance information at all, NW should anyway have an initial configuration for neighbor cell measurement (if needed for RRM).

For event-triggered reporting with eventD1/D2, NW can configure eventD1/D2 with reference locations associated to a certain neighbor cell or associated to a certain area instead of specific neighbor cells. If the event for a certain report configuration is not received by the NW, it implies UE is not close enough to the neighbor cell for the current time.

Another concern raised for eventD1/D2 is that the distance threshold is from 1 (representing 50m) not 0, so that UEs within 50m of a serving cell reference location cannot trigger reporting. This should not a big issue as NTN cell is large and UE within (50m + a small hysteresis) of cell center does not need to perform neighbor cell RRM measurement for mobility.

For inter-frequency neighbor cell co-located with the serving cell, some have commented that eventD1/D2 may not be applicable for location-based SMTC configuration/selection. However, for neighbor cell co-located with the serving cell, NW can configure SMTC directly for the measurement (if needed) without location-based SMTC configuration/selection, because the handover to the co-located cell (if needed) in this case is not due to UE at cell edge of the serving cell, but usually for load balancing.

Proposal 4: No need of location-based SMTC configuration/selection enhancement for RRC connected mode.

## UE capability reporting of Msg4 PDSCH repetition in Msg3

Regarding Msg4 PDSCH repetition, RAN1 has send a LS asking the UE capability (i.e., request) reporting of Msg4 PDSCH repetition in Msg3 (see Appendix). Note in Rel-18, the similar functionality has been designed for UE capability reporting in Msg3 for PUCCH repetition for Msg4 HARQ-ACK. New LCIDs for CCCH/CCCH1 are introduced as highlighted below.

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

In addition, according to the RAN1 RRC parameter list, the repetition number for Msg4 PDSCH repetition (pdsch-AggregationFactor) is configured in SIB1 PDSCH-ConfigCommon.

Assuming to apply the existing design of PUCCH repetition for Msg4 HARQ-ACK, UE indicates capability/request of Msg4 PDSCH using LCIDs in Msg3 when certain condition is met, and a RSRP threshold can be used for the condition. Reusing the condition for PUCCH repetition for Msg4 HARQ-ACK, if the repetition number is configured and the RSRP threshold is not configured, or if both are configured and the RSRP of DL PL RS is less than the RSRP threshold, UE indicates capability/request of Msg4 PDSCH repetition in Msg3 by LCID.

If RSRP threshold is not used, UE indicates capability/request of Msg4 PDSCH repetition if the repetition number (pdsch-AggregationFactor) is configured.

Proposal 5: Use LCID(s) in Msg3 to indicate capability/request of Msg4 PDSCH repetition.

Proposal 6: Discuss whether RSRP threshold is used for the condition of indicating capability/request of Msg4 PDSCH repetition. If to be used, whether to introduce a new RSRP threshold or reuse the existing rsrp-ThresholdMsg4HARQ-ACK.

Considering the existing design for PUCCH repetition of Msg4 HARQ-ACK, we can consider two options for Msg4 PDSCH repetition.

Option 1: reuse the existing RSRP threshold (rsrp-ThresholdMsg4HARQ-ACK) and the existing LCIDs (with updated LCID values) introduced for PUCCH repetition for Msg4 HARQ-ACK to indicate the capability/request for Msg4 PDSCH repetition.

Option 2: introduce a new RSRP threshold (rsrp-ThresholdMsg4PDSCH) and new LCIDs to indicate the capability/request for Msg4 PDSCH repetition.

By Option 1, if numberOfMsg4HARQ-ACK-Repetitions or pdsch-AggregationFactor is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK, UE uses the existing LCIDs (with updated LCID values) to indicate capability/request for either PUCCH repetition of Msg4 HARQ-ACK or Msg4 PDSCH repetition.

By Option 2, if pdsch-AggregationFactor is configured and rsrp-ThresholdMsg4PDSCH is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4PDSCH, UE uses new LCIDs to indicate capability/request for Msg4 PDSCH repetition. A set of new LCIDs are needed for UE indicating Msg4 PDSCH repetition only and another set of new LCIDs are needed for UE indicating both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK. Assuming Msg4 PDSCH repetition can be supported for (e)RedCap UEs, totally 12 new LCIDs are needed.

Proposal 7: Discuss whether Msg4 PDSCH repetition is supported for (e)RedCap UEs.

Proposal 8: Discuss whether to reuse the existing LCIDs (with updated LCID values) introduced for PUCCH repetition for Msg4 HARQ-ACK or introduce new LCIDs to indicate the capability/request for Msg4 PDSCH repetition.

# Conclusion

In this contribution we have the following proposals.

Observation 1: For the SMTC enhancement on configuring two SMTC periodicities per frequency, RAN4 LS indicates concern on the workload in Rel-19 timeframe.

Observation 2: If support up to 2 SMTC periodicities per frequency, new SMTC signaling is needed in SIB2, SIB4, and measurement object configuration.

Proposal 1: Introduce smtc5list, which is a list including up to 5 elements of SSB-MTC5, and each element SSB-MTC5 contains a list of PCI, an offset, and a periodicity (if agreed 2 SMTC periodicities). If a parameter (PCI list, or periodicity or offset) of an element SSB-MTC5 in smtc5list is absent, the corresponding parameter of the element in the same position in smtc4list is applied.

Proposal 2: If both smtc4list and smtc5list are configured, smtc5list is applied if supported and smtc4list is ignored.

Proposal 3: For RRC_CONNECTED, the number of SMTCs configured per frequency layer in measurement object is at most 4 as legacy.

Observation 3: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location estimated from event-triggered measurement reporting with eventD1/D2, and can update SMTC based on the latest report.

Observation 4: Closest reference location(s) or selected SMTCs may change as UE or serving/neighbor cell moves. This leads to event-triggered reporting and moving reference location for earth-moving cells, thus, duplicating functionality as eventD1/D2.

Proposal 4: No need of location-based SMTC configuration/selection enhancement for RRC connected mode.

Proposal 5: Use LCID(s) in Msg3 to indicate capability/request of Msg4 PDSCH repetition.

Proposal 6: Discuss whether RSRP threshold is used for the condition of indicating capability/request of Msg4 PDSCH repetition. If to be used, whether to introduce a new RSRP threshold or reuse the existing rsrp-ThresholdMsg4HARQ-ACK.

Proposal 7: Discuss whether Msg4 PDSCH repetition is supported for (e)RedCap UEs.

Proposal 8: Discuss whether to reuse the existing LCIDs (with updated LCID values) introduced for PUCCH repetition for Msg4 HARQ-ACK or introduce new LCIDs to indicate the capability/request for Msg4 PDSCH repetition.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

# Appendix

## RAN4 LS reply R4-2508433

RAN4 thanks RAN2 for the information in LS R2-2503068. RAN4 discussed the RRM requirements impact of the SMTC enhancement, and would like to provide following conclusions achieved in RAN4#115:

- For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements:

From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but that requires some clarifications/modifications to the existing RRM requirements.

- RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.

Note: RAN4 has not estimated the full impact during RAN4#115 meeting. There is a concern on the workload on to support those agreements in Rel-19 timeframe from RAN4 requirements perspective.

- For the SMTC enhancement on configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs:

From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. However, the current requirements in TS 38.133 for the scenarios when the number of configured SMTCs exceed the parallelSMTC-r17 for idle/inactive UEs are captured below for reference:

For Measurements of intra-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected. For Measurements of inter-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the value indicated by parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

## RAN1 LS R2-2504965

As part of the WI on NR NTN enhancement phase 3, RAN1 has agreed to support for Msg4 PDSCH repetition. The following agreement has been made regarding the activation/deactivation of Msg4 PDSCH repetition.

Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

RAN1 kindly requests RAN2 to consider the above agreement when designing the report included in Msg3 PUSCH.