3GPP TSG-RAN WG2 Meeting #131	R2-2505455

Bangalore, India Aug 25th – 29th, 2025

Agenda item:	8.6.2

Source:	Samsung

Title:	Remaining issues of Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In this contribution, we will further address remaining essential open issues including handling of target cell’s SP CSI-RS resources after cell switch, stage-3 identified open issues.

Discussion

Semi-persistent CSI-RS resources after cell switch

During the post email discussion [POST130][120][MOB], below new MAC-20 issue was discussed.

Open issue MAC-20 (essential): How to design the MAC CE to activate/deactivate the SP CSI-RS resource for LTM CSI acquisition for candidate cells based on RAN1 LS

Based on the majority companies view, same SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both RRM measurement and early CSI acquisition purpose and this MAC CE also includes the CSI-IM resource sets for early CSI acquisition purpose. We think the separate MAC CE for each feature is cleaner way but we tend to agree that there are many common operation between RRM and earl CSI acquisition in this case. Therefore, we are also fine to introduce a new SP CSI-RS Resource Set Activation/Deactivation MAC CE for both RRM and early CSI purpose.

The next question is how the target cell’s SP CSI-RS resources is handled after cell switch is indicated by LTM cell switch MAC CE. In RAN2#130 meeting, RAN2 made following agreements on the issue of handling the SP CSI-RS resource of candidate cells (other than the target cell) after cell switch.

Agreements on L1 measurement triggered MR

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

It seems majority companies think the common SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both LTM RRM and early CSI purpose, UE could keep measuring the activated SP CSI-RS resources  after LTM cell switch if the early CSI acquisition reporting is configured. If the early CSI acquisition reporting is not configured in the target cell UE deactivates SP CSI-RS resource of target cell after cell switch.

Based on the RAN1 agreements, the source cell send SP CSI-RS Resource Set Activation/Deactivation MAC CE for early CSI acquisition to the UE, and UE continues to measure the activated resources after cell switch. In addition, a single CSI report configuration is configured for early CSI acquisition so UE report the early CSI measurement in the first UL MAC PDU. Then we think UE could stop to measure the activated SP CSI-RS resources after UE report the early measured CSI to the target cell. It means UE just follow the report configuration for the early CSI acquisition.

Proposal 1: UE could keep measuring the activated SP CSI-RS resources after LTM cell switch if the early CSI acquisition reporting is configured. If the early CSI acquisition reporting is not configured in the target cell, UE deactivates SP CSI-RS resource of target cell after cell switch.

Proposal 2: For early CSI acquisition, UE stop to measure the activated SP CSI-RS resources after UE report the early measured CSI to the target cell.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: UE could keep measuring the activated SP CSI-RS resources after LTM cell switch if the early CSI acquisition reporting is configured. If the early CSI acquisition reporting is not configured in the target cell, UE deactivates SP CSI-RS resource of target cell after cell switch.

Proposal 2: For early CSI acquisition, UE stop to measure the activated SP CSI-RS resources after UE report the early measured CSI to the target cell.

References

[1]	R2-250xxxx		Report of [POST130][120][MOB]: Discussion summary and list of MAC open issue for Mob Ph4		Vivo.