3GPP TSG-RAN WG2 Meeting #131	R2-2505473

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.2.1

Source:	Mediatek Inc.

Title:	Discussion on Functionality Management for RRM Measurement Prediction

Document for:	Discussion, Decision

Introduction

In the last RAN2 #130 meeting, RAN2 reached an agreement on the general applicability procedures, covering both Option A and Option B. During the discussion on the inference procedure, inference parameters for Option B were further identified. However, the specifics of the full inference configuration and the timing for performing inference were not addressed. Regarding performance monitoring, RAN2 agreed to explore both UE-sided and NW-sided monitoring procedures for the UE-sided model. In this contribution, we aim to address the remaining issues. We will examine the granularity of functionality related to the applicability report, the specifics of the full inference configuration, and the timing for performing inference. For monitoring, we will focus on the configuration of monitoring (and reporting) and the monitoring metrics for each use case.

Discussion

Functionality granularity

In the last meeting, RAN2 agree to use the general applicability procedure in AI-BM as the baseline, as given below

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced.

Figure 1: Illustration of general applicability report procedure 3.1-1[1]

The applicability procedure allows the UE to report whether the inference configuration provided by the NW is applicable. One remaining issue is the granularity. In the first two steps, the UE will report UE capability to the NW. The granularity of the UE capability for AI mobility could be left to the UE capability discussion in the normative work and is outside the scope of this contribution. Once the inference configuration (which can be a full inference configuration or inference parameters) is received by the UE in Step 3, the UE needs to report whether the AI inference indicated by the inference configuration is applicable. Thus, we think it is natural to consider that the granularity of the applicable functionality is actually an inference configuration or a set of inference parameters. This means that when the UE receives one or multiple full inference configurations or sets of inference parameters, what the UE reports for applicability is just the IDs of those inference configurations or sets of inference parameters. Additionally, we recommend that RAN2 consider whether both Option A and Option B can be configured simultaneously. The UE is capable of sending reports based on the type of configuration it receives. If only one option is configured, the UE will report accordingly. However, if both options are configured, the UE will include the applicability for both in the same report.

While the benefits of allowing both options to coexist are not clearly defined, it is proposed that the network should handle any conflicting results that may arise from the UE's applicability reports for Option A and Option B.

Proposal 1: The functionality corresponds to a full configuration or a set of configuration parameters. The applicable reports indicate the applicable full inference configuration or the set of inference parameters.

Proposal 2: RAN2 discuss whether both Option A and Option B can be configured simultaneously for AI mobility. NW should handle any conflicting results that may arise from the UE’s applicability reports if both options are configured simultaneously.

Inference Procedure

In the last RAN2 #130 meeting, the content of inference parameters in Option B has been discussed as follows. In this section, we will focus on the content of full inference configuration.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Full Inference Configuration

RAN2 has agreed to reuse the legacy RRM measurement configuration framework as the baseline for AI mobility inference configuration. The legacy RRM measurement configuration contains the information for measurement objects and measurement reports. Here we list the related components for the AI mobility inference configuration as follows.

- Measurement object (MO)

- Report configuration

- Measurement identities

The information of measurement targets is carried by MO, which includes the timing and frequency locations, cells list, etc. The report configuration contains the report triggering condition, e.g., event-triggered, periodical, and the report quality, e.g., RSRP, SINR, RSSI (via ReportQuantityCell). A measurement/report task is identified by a measurement ID which contain one MO ID and one report configuration ID. This framework can also be used for AI mobility inference configuration.

However, it is clear that in AI mobility inference configuration, in addition to the MO, we need to identify the information of the inference target, i.e., the predicted object. It should contain the timing and frequency location of the predicted target. It should also include additional information for different sub-use cases, for example, the prediction window (PW) length for Temporal domain case A, and the skipping pattern and MRRT for Temporal domain case B.

Observation 1: For UE-sided model RRM prediction, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, MRRT, etc.

Similar to the legacy measurement, the report configuration provides the measurement report triggering condition and the report quantity, however, for AI mobility use cases, the measurement report can be generated based on not only real measurements but also predicted results. Also, similar to the measurement ID, for AI mobility inference, we can introduce “inference ID” to figure out the mapping of an MO, a predicted object, a report configuration, and an inference report configuration. The overall configuration structure can be shown in the following figure where the inference configuration contains the following components:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report. The measurement report can be generated based on both real measurement and inference results.

- Predicted object: Describes the information of inference/predicted target.

- Inference identity: Refers to a measurement Identity (MeasId) and provides the mapping of a measurement object, a predicted object, and a report configuration.

Figure 2: Overall structure of inference configuration.

Proposal 3: RAN2 consider the inference configuration which contains

- Measurement object: Describes the information of measurement/observed target.

- Predicted object: Describes the information of inference/predicted target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report.

- Inference identity: Refers to a measurement Identity (MeasId) and provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 4: The predicted object contains

- Timing and frequency information, e.g., smtc, ssbfrequency, subcarrier, etc.

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

The details of the content can be FFS.

With the inference configuration, UE can figure out the AI mobility use case, e.g., Temporal domain case A, Temporal domain case B, Spatial domain, etc., from the information carried by measurement object, predicted object, and report configuration directly, without additional explicit description.

Observation 2: Based on the measurement object, predicted object, and report configuration, UE can realize the task of AI inference (the sub-use case of AI mobility) without explicit description.

When to perform the inference

The other issue is about when to perform inference. For RRM prediction use cases with 1st goal, i.e., Temporal case B, spatial domain prediction, and frequency domain prediction, the only requirement related to the timing of inference is the latency requirement of the measure report. For those cases, when to perform inference should be UE implementation, as long as UE can meet all the report requirements defined by RAN4. Whether the report requirement should be relaxed or modified for AI use cases can be FFS in RAN4.

Proposal 5: For RRM Temporal domain case B, Spatial domain prediction, and frequency domain prediction, when to perform inference is up to UE implementation. UE needs to meet the report requirements determined by RAN4.

Proposal 6: Whether to relax or modify report requirement for AI use cases can be FFS in RAN4.

For RRM Temporal domain case A, the AI is applied to predict the future measurement results. The AI outcome is more related to indirect measurement event prediction, rather than providing a measurement report earlier. The related requirement and the timing of inference for this use case are addressed in another document [2] related to measurement event prediction.

Observation 3: For RRM Temporal domain case A, the AI result is expected to be applied for indirect measurement event prediction. The timing of inference is left for measurement event prediction discussion.

Performance Monitoring

In the last meeting, RAN2 agreed to consider both UE-monitoring and NW-sided monitoring for UE-sided model. Furthermore, for UE-monitoring procedure, there can be either NW-decision and UE-decision procedures. Where the use case of later one is FFS, as shown in the following agreement.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In our view, the UE-decision procedure should be considered, at least in cases where the UE notices that the AI model is no longer suitable. Compared to the NW-decision (where the UE needs to wait for signaling exchange), a quick fallback—i.e., reverting to a non-AI approach—triggered by the UE can help prevent severe performance loss due to an outdated model. However, UE-decision may not be applied to every use case. An exception is when the NW needs to reconfigure before the UE can fallback. For example, in RRM Temporal Domain Case B, if the NW has reduced the transmission of RS, the UE-decision cannot be applied. However, if the NW does not reduce the transmission and the AI prediction is only used to reduce measurement overhead on the UE side, the UE-decision can facilitate a quick fallback to a non-AI approach when the AI model becomes outdated. Since the UE can decide on its own whether to use AI or not, the UE-decision fallback has a greater impact on whether the UE can, before notifying the NW and waiting for the new configuration, choose not to follow the existing reporting requirements and monitoring requirements (if any).

Proposal 7: For RRM prediction use cases where the NW does not reduce RS transmission due to AI prediction, NW can configure UE to make the management decision with UE-sided monitoring.

Monitoring Configuration

To support the performance monitoring for AI mobility, NW must configure both the inference configuration and monitoring configuration to UE. RAN2 has agreed to reuse the legacy RRM measurement configuration framework as the baseline for AI mobility inference configuration. In previous section, we have already proposed an detail structure of inference configuration as shown in Figure 2. In the following, we add the monitoring component under the same structure to complete the related configuration, which can be shown as follows.

Figure 3: Overall structure of monitoring configuration.

The legacy measurement object can be reused to carry the information about the measurement target (to measure ground-truth data). Several new items are added to provide information about monitoring criteria, monitoring reports, and the corresponding inference task. In our view, a monitoring configuration should contain the following components:

- Measurement object: Describes the information about the measurement target (to measure ground-truth/monitoring data).

- Monitoring report configuration: Describes the report triggering condition, report quantity, and monitoring criteria.

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity.

With the monitoring configuration, UE can measure the ground-truth/monitoring data according to the measurement object. For NW-sided monitoring, UE directly reports the raw monitoring data to the NW. The necessary information for reporting the monitoring data is configured in the monitoring reporting configuration, including the report type (e.g., event-triggered, periodical), and the report quantity (e.g., L1/L3 RSRP, SINR, RSSI). For UE-sided monitoring, UE reports the monitoring results (the comparison result) instead of raw monitoring data (e.g., ground-truth). The monitoring criteria (e.g., average L3 RSRP difference, F1 score, predefined threshold) and the report triggering condition are all provided in the monitoring report configuration. It should be noted that, for the UE-sided monitoring, RAN4 needs to be informed and involved to define the requirement to determine whether the AI prediction is accurate or not.

Proposal 8: The monitoring configuration contains

- Measurement object: Describes the information about the measurement target

- Monitor report configuration: Describes the report triggering condition, report quantity, and monitoring criteria/metric

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity

Proposal 9: The monitoring criteria/metric for RRM prediction can be the L3 RSRP difference.

Proposal 10: For UE-sided monitoring, UE compares the monitoring data and inference output based on the monitoring criteria/metric in the monitoring configuration. RAN4 needs to be informed and involved to define the corresponding requirement to determine whether the AI prediction is accurate or not.

Mapping of monitoring and inference sample

To perform performance monitoring, the device must know which monitoring result and inference output should be compared. The information about the corresponding inference identity is therefore provided in the monitoring identity. It should be noted that the timing (and periodicity) of generating inference outputs may not align with the timing (and periodicity) of measuring monitoring data as shown in the following figures.

For NW-sided monitoring, which monitoring data and inference output are used for comparison is up to NW implementation. For UE-sided monitoring, the monitoring result should be generated by comparing the closest inference output with the monitoring data.

Figure 4: For UE-sided monitoring, monitoring result is generated by comparing the closest inference output and monitoring data.

Proposal 11: If the timing and/or period of inference output and monitoring data are not the same,

- For NW-sided monitoring, which inference output and monitoring data are used for comparison is up to NW implementation.

- For UE-sided monitoring, the monitor result is generated by comparing the closest inference output with the monitoring data.

Conclusion

Applicability Report Procedure:

Proposal 1: The functionality corresponds to a full configuration or a set of configuration parameters. The applicable reports indicate the applicable full inference configuration or the set of inference parameters.

Proposal 2: RAN2 discuss whether both Option A and Option B can be configured simultaneously for AI mobility. NW should handle any conflicting results that may arise from the UE’s applicability reports if both options are configured simultaneously.

Inference Procedure:

Full inference configuration:

Observation 1: For UE-sided model RRM prediction, NW needs to configure information about the predicted target to UE. This may include the timing and frequency location of the predicted target and the related information to perform inference such as PW length, skipping pattern, MRRT, etc.

Proposal 3: RAN2 consider the inference configuration which contains

- Measurement object: Describes the information of measurement/observed target.

- Predicted object: Describes the information of inference/predicted target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report.

- Inference identity: Refers to a measurement Identity (MeasId) and provides the mapping of a measurement object, a predicted object, and a report configuration.

Proposal 4: The predicted object contains

- Timing and frequency information, e.g., smtc, ssbfrequency, subcarrier, etc.

- PW length (for temporal case A)

- Skipping pattern, MRRT (for temporal case B)

- Measured and predicted beam/cell pattern, MRRS (for spatial domain prediction)

The details of the content can be FFS.

When to perform inference:

Proposal 5: For RRM Temporal domain case B, Spatial domain prediction, and frequency domain prediction, when to perform inference is up to UE implementation. UE needs to meet the report requirements determined by RAN4.

Proposal 6: Whether to relax or modify report requirement for AI use cases can be FFS in RAN4.

Observation 3: For RRM Temporal domain case A, the AI result is expected to be applied for indirect measurement event prediction. The timing of inference is left for measurement event prediction discussion.

Performance monitoring:

Proposal 7: For RRM prediction use cases where the NW does not reduce RS transmission due to AI prediction, NW can configure UE to make the management decision with UE-sided monitoring.

Proposal 8: The monitoring configuration contains

- Measurement object: Describes the information about the measurement target

- Monitor report configuration: Describes the report triggering condition, report quantity, and monitoring criteria/metric

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity

Proposal 9: The monitoring criteria/metric for RRM prediction can be the L3 RSRP difference.

Proposal 10: For UE-sided monitoring, UE compares the monitoring data and inference output based on the monitoring criteria/metric in the monitoring configuration. RAN4 needs to be informed and involved to define the corresponding requirement to determine whether the AI prediction is accurate or not.

Proposal 11: If the timing and/or period of inference output and monitoring data are not the same,

- For NW-sided monitoring, which inference output and monitoring data are used for comparison is up to NW implementation.

- For UE-sided monitoring, the monitor result is generated by comparing the closest inference output with the monitoring data.

Reference

[1] 	R2-2407848 LS on applicable functionality reporting.docx, RAN2

[2] 	R2-2505474 Discussion on Functionality Management for RRM measurement event prediction, Mediatek