# 3GPP TSG RAN WG2 Meeting #130      	                        R2-2505504
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.1.3

Source:	Apple

Title:	Remaining issues on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

In RAN2#130 [2], most of essential issues on NW-side data collection have been closed. However, there are some leftover issues. In post-meeting email discussion on RRC [3] and 38.306 running CR [4], multiple open issues are identified. In this contribution, we discuss the following remaining issues:

- MAC impact on NW-sided data collection

- Key RRC open issues on NW-sided data collection

  - Open issue RRC-21: Time related content of logged data

  - Open issue RRC-52: NW control on retaining logged data at CHO/LTM

  - Open issue RRC-19: Reporting assistance information related to logged measurements

  - Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

  - Open issue RRC-31: The release of logged AIML data in UE’s buffer

  - Open issue RRC-39: The naming of IEs related to NW-side data collection

- Key UE capability open issue of NW-sided data collection

  - Open issue UECap-1: whether UE can support other memory sizes and indicate to network

  - Open issue UE Cap-2: How to define UE logged measurement of data collection

# 2 Discussion

## 2.1 MAC impacts on NW-side data collection

In existing TS 38.321 [5], the L1 CSI reporting is stopped in the following cases:

- The SCell is deactivated (in Section 5.9	)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

The reason for stopping L1 CSI reporting behind these specifications is because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG.

Observation 1: In existing TS 38.321, L1 CSI reporting is stopped in the following cases because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG:

- The SCell is deactivated (in Section 5.9)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

Following the same logic, we think that it is necessary to specify the UE stops L1 CSI measurement logging for the L1 resources which are configured in deactivated SCell/BWP (i.e. above Case 1-3). According to the following RAN2#129b agreement [6], NW-sided data collection can’t be configured in SCG. Thus, the above Case 4 (i.e. deactivated SCG) is not applied to L1 measurement logging.

For NR-DC, the configuration and reporting of NW sided data collection can only be via MN

This UE behaviour should be captured in MAC spec. As CR Rapporteur, we draft the running 38.321 CR in [7] to capture these impacts. We suggest RAN2 to agree the UE behaviour.

Proposal 1: RAN2 agree that the UE stops L1 measurement logging in the following cases, and capture them in MAC running CR:

- The SCell is deactivated

- BWP is deactivated

- BWP is dormant BWP

## 2.2 Key RRC open issues on NW-side data collection

In this section, we share our view on some key open issues suggested by RRC CR Rapporteur in [3].

### 2.2.1 RRC-21: Time related content of logged data

The issue can be described as follows:

###### Open issue RRC-21: Time related content of logged data Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity). This issue refers to the RAN2#129bis agreement: - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit RAN2#130 agreement: Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.. The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3 and 6.2.2. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

According to RAN2#130 agreement [2], the need of time information was confirmed but how to add timestamp was not converged. During online discussion, the main concern is the signalling overhead of adding timestamp for each logging sample. That is why the highlighted part of the agreement implies that some timestamps can be skipped if its time interval to previous sample is same as the configured periodicity because NW can identify these gaps.

Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

We use Figure.1 to illustrate our understanding. In the figure, timestamp for logging sample 1 and 4 are needed because NW can’t identify their logging time if without timestamp. However, timestamp of the logging sample 2 and 3 can be skipped because their interval to previous logging sample is same as configured logging periodicity.

Figure. 1 Illustration of how to add timestamp for logged measurements

Observation 2: According to RAN2#130 agreement, the need of time information was confirmed, and some timestamp can be skipped if its time interval to previous sample is same as the configured periodicity because NW can identify these gaps. This is helpful to reduce signaling overhead.

One followed issue is whether the first logged sample needs to add timestamp because it doesn’t have the previous logging sample. We prefer to make it configurable whether to add absolute timestamp. Thus, we propose:

Proposal 2 (open issue RRC-21): For one logging sample, the UE adds timestamp in the following way:

- For the first logging sample, whether to add absolute timestamp is configurable by NW.

- Other than the first logging sample, add the time interval between its previous logging sample only if it is larger than the configured logging periodicity (i.e., only add gap larger than logging periodicity).

### 2.2.2 RRC-52: NW control on retaining logged data at CHO/LTM

The issue can be described as follows:

###### Open issue RRC-52: NW control on retaining logged data at CHO/LTM Issue description: It is unclear whether the following RAN2#130 agreement is applicable only to regular HO, or also to CHO and LTM. 1 (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3. One argument raised by companies is that the agreement is not applicable to CHO and LTM because the target cell configuration of CHO and LTM can only be provided to the UE before HO execution. One solution proposed by companies is: Source cell makes decision, and it includes a list of 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration to include LTM configuration or CHO condition. In the rapporteur’s view, there is no need for further specification changes and the RAN2 agreement above applies also for CHO and LTM. There are RRCReconfiguration with sync messages applied by the UE at time of execution in both cases, and there is a HO preparation procedure before. Proposed resolution: It is suggested that companies address this issue in their contributions.

We think the solution agreed in RAN2#130 is not applicable to CHO and LTM because the target cell configuration of CHO and LTM can only be provided to the UE before HO execution. Both LTM and CHO do not send RRCReconfiguration with Sync message to UE. According to Section 5.3.5.5.2 of 38.331 [9], the UE shall immediately perform DL sync and execute HO towards target cell upon reception of RRCReconfiguration with Sync (i.e. it is only applied to regular HO). In LTM, MAC-CE triggers HO execution while UE autonomously trigger HO execution when condition is met. As RAN2#130 solution relies on RRCReconfiguration with Sync message, we understand it is NOT applicable to LTM and CHO.

Observation 3: Both LTM and CHO do not send RRCReconfiguration with Sync message to UE: MAC-CE triggers HO execution in LTM while UE autonomously trigger HO execution when condition is met. As RAN2#130 solution relies on RRCReconfiguration with Sync message, it is NOT applicable to LTM and CHO.

We think the solution is simple: source cell makes decision and includes a list of 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration to include LTM configuration or CHO condition. Thus, we propose:

Proposal 3 (open issue RRC-52): RAN2 confirm that the solution agreed in RAN2#130 is only applicable to regular HO. To enable NW control on retaining logged data at CHO/LTM, source cell makes decision and includes a list of 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration with LTM configuration or CHO condition.

### 2.2.3 RRC-19: Reporting assistance information related to logged measurements

The issue can be described as follows:

###### Open issue RRC-19: Reporting assistance information related to logged measurements Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated? This issue is related to the RAN2#129bis agreement: Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined. The issue is captured as an editor’s notes in the running CR, in clause 5.3.5.9, 5.7.4.3, 6.2.2, and 6.3.4. Based on the comments in the RRC running CR, some companies have a different view on the solution: - Each of the three indications (full buffer being reached, buffer threshold being reached, and low power) is configured separately; or - Full buffer reporting is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (no additional configuration bit is needed) and only buffer threshold being reached and low power indications are configured each separately. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

On the following two options listed by CR Rapporteur:

- Option 1: Each of the three indications (full buffer being reached, buffer threshold being reached, and low power) is configured separately; or

- Option 2: Full buffer reporting is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (no additional configuration bit is needed) and only buffer threshold being reached and low power indications are configured each separately.

We prefer Option 2 because the UE autonomously stops logging measurement upon reporting full buffer indication. It is important to align understanding between UE and gNB on whether the UE has stopped data collection. To avoid misalignment between NW and UE on whether the UE still performs NW-side data collection, we think full buffer indication is essential and thereby should be enabled by default.

Observation 4: As the UE autonomously stops logging measurement upon reporting full buffer indication, disabling it may lead to misalignment between NW and UE on whether the UE still performs data collection.

Thus, we propose:

Proposal 4 (open issue RRC-19): Full buffer indication is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (i.e. no additional configuration bit is needed).

### 2.2.4 RRC-25: Dynamic activation/deactivation of data collection configurations for logging

The issue can be described as follows:

###### Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging Issue description: RAN2#127 made the following agreement regarding NW-side data collection: 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.. The second FFS on dynamic activation/deactivation has not yet been addressed, as also pointed out in the stage-2 running CR draft. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In RAN2#128 [8], it was agreed not to support measurements on aperiodic CSI resources. However, it was not concluded whether to support measurements on Semi-Persistent (SP) CSI resource.

Agreements on NW side data collection - Measurements on aperiodic CSI resources are not reported for NW sided data collection.

We think it is not reasonable to support NW-side data collection on SP CSI resources. Enhanced MDT is intended to support data collection for model training which doesn’t have latency requirement. If NW needs the UE to perform data collection only for certain time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Observation 5: If NW needs the UE to perform data collection only for certain time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via dynamically activation / deactivation SP CSI resources.

Thus, we propose not to support dynamic activation / deactivation of logged L1 measurements on SP CSI resources because it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources.

Proposal 5 (open issue RRC-25/RRC-30): As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

### 2.2.5 RRC-31: The release of logged AIML data in UE’s buffer

The issue can be described as follows:

###### Open issue RRC-31: The release of logged AIML data in UE’s buffer Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue.

We think it is sufficient to only specify 48 hour storage validity duration which is legacy solution of logged MDT. The other two conditions are optimization without clear benefit because it is not urgent to release logged data. Thus, we propose:

Proposal 6 (open issue RRC-31): Same as logged MDT, only specify 48 hour storage validity duration for logged L1 measurements.

### 2.2.6 RRC-39: The naming of IEs related to NW-side data collection

The issue can be described as follows:

###### Open issue RRC-39: The naming of IEs related to NW-side data collection Issue description: The naming of IEs related to NW-side data collection can be revisited. We will have more CSI-based and other use cases, each with different measurement logs and we should minimize the risk of having to replicate the NW-side DC procedures and ASN.1 for every new use case. One solution direction is to make the naming of procedures and ASN.1 related to NW-side DC generic and enclose use case specific IEs / data logs within so that our framework can support future use cases. An alternative is to name the IEs based on the use case, which would help to write the procedural text in a simpler way, while being clear. Proposed resolution: Adopt the proposals below. - (RRC-39) The UE stores logged data for BM in a variable specific to L1 CSI related measurements. (RRC-39) RAN2 to discuss whether the name of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW are use case specific.

We don’t have strong view on how to capture variable on logged data (i.e. we are fine with Proposal 18). But for Proposal 19, we have concern to introduce request indication for specific use case or specific configuration ID. In our understanding, one single UE should not be configured with multiple parallel tasks of NW-side data collection. Due to power consumption concern, it is expected that NW will distribute NW-side data collection of different use cases to different UEs.

Observation 6: Due to power consumption concern, it is expected that NW will distribute NW-side data collection of different use cases to different UEs. In other word, one single UE should not be configured with multiple parallel tasks of NW-side data collection.

Thus, we prefer one single field to request all the data in the buffer.

Proposal 7 (open issue RRC-39): Only support single field to request all the data in the buffer, i.e. don’t introduce request indication for one specific use case or one specific configuration ID.

### 2.2.7 RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

The issue can be described as follows:

###### Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc Issue description: Handling of configuration for UE assistance information, to report data availability and low power state (LoggedDataCollectionAssistanceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified. In the rapporteur’s view, handling the UAI configuration for reporting data availability and low power state should follow the way that the logging configurations are handled, which depends on the solution that RAN2 will adopt based on the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE). Proposed resolution: Adopt the proposal below. - (RRC-28) RAN2 to discuss whether logging configurations for NW-side data collection are discarded upon IDLE/INACTIVE/RLF. - (RRC-28) RAN2 to discuss whether handling the configuration for UE assistance information to report data availability and low power state upon IDLE/INACTIVE/RLF follows the UE behaviour for handling the logging configurations.

First, we think the UAI configuration for NW-side data collection should be released upon entering IDLE/INACTIVE/RLF. However, the exact timing to release the UAI configuration in OtherConfig depends on different cases according to TS 38.331 [9]

- When entering IDLE state: the UAI configuration is released when the UE receives RRC release message.

- When entering INACTIVE state: the UAI configuration is released when the UE initiates the RRC resume procedure

- When detection of RLF: the UAI configuration is released when the UE initiates the RRC Re-establishment procedure.

We think it is sufficient to follow legacy UE behaviour of releasing UAI configuration. Thus, we propose:

Proposal 8 (open issue RRC-28): Follow legacy RRC procedure when to release the UAI configuration in OtherConfig for NW-side data collection upon entering IDLE/INACTIVE state or detection of RLF.

## Key UE capability open issue of NW-sided data collection

In this section, we share our view on some key open issues suggested by 38.306 CR Rapporteur in [4].

### 2.3.1 Open issue UECap-1: whether UE can support other memory sizes and indicate to network

We think 3GPP can only specify minimum buffer size for NW-side data collection. It is up to UE implementation whether to allocate additional memory. Thus, we propose:

Proposal 9 (open issue UECap-1): Only specify 64KB minimum buffer size for NW-side data collection. It is up to UE implementation whether to allocate additional memory.

### 2.3.2 Open issue UECap-2: How to define UE logged measurement of data collection

Summary report [4] made the following proposal:

Proposal 3: Introduce an optional UE capability to indicate supporting logged measurements of data collection for NW-side model, which includes the minimum AS layer memory size. FFS whether the following components are part of this capability or separate capabilities, and how: - periodic logging - event-based logging - measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc) - provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

On the FFS, our preference is:

- The capability of periodic logging is integrated into general capability of NW-side data collection (i.e. it is mandatory to support periodic logging if the UE reports supporting of NW-side data collection).

- The capability of providing UAI for NW-side data collection is integrated into general capability of NW-side data collection (i.e. it is mandatory to support UAI if the UE reports supporting of NW-side data collection).

- Introduce a separate capability of event-based logging.

- For consideration of future-proof, introduce a separate capability of logging measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc).

Thus, we propose:

Proposal 10 (open issue UECap-2): RAN2 agree the following details of UE capability of NW-side data collection:

- The capability of periodic logging and reporting assistance information for NW-side data collection are integrated into general capability of NW-side data collection (i.e. it is mandatory to support periodic logging and UAI if the UE reports supporting of NW-side data collection).

- Introduce a separate capability of event-based logging.

- Introduce a separate capability of logging measurement quantities and resources for AI/ML based BM (Set A/B, maximum resources of Set A/B, etc).

# 3 Conclusion

In this contribution, we discuss remaining issues of NW-sided data collection. Our Observation are:

Observation 1: In existing TS 38.321, L1 CSI reporting is stopped in the following cases because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG:

- The SCell is deactivated (in Section 5.9)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

Observation 2: According to RAN2#130 agreement, the need of time information was confirmed, and some timestamp can be skipped if its time interval to previous sample is same as the configured periodicity because NW can identify these gaps. This is helpful to reduce signaling overhead.

Observation 3: Both LTM and CHO do not send RRCReconfiguration with Sync message to UE: MAC-CE triggers HO execution in LTM while UE autonomously trigger HO execution when condition is met. As RAN2#130 solution relies on RRCReconfiguration with Sync message, it is NOT applicable to LTM and CHO.

Observation 4: As the UE autonomously stops logging measurement upon reporting full buffer indication, disabling it may lead to misalignment between NW and UE on whether the UE still performs data collection.

Observation 5: If NW needs the UE to perform data collection only for certain time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via dynamically activation / deactivation SP CSI resources.

Observation 6: Due to power consumption concern, it is expected that NW will distribute NW-side data collection of different use cases to different UEs. In other word, one single UE should not be configured with multiple parallel tasks of NW-side data collection.

Based on observations, our proposals can be found below:

MAC impacts on NW-side data collection

Proposal 1: RAN2 agree that the UE stops L1 measurement logging in the following cases, and capture them in MAC running CR:

- The SCell is deactivated

- BWP is deactivated

- BWP is dormant BWP

Key RRC open issues on NW-side data collection

Proposal 2 (open issue RRC-21): For one logging sample, the UE adds timestamp in the following way:

- For the first logging sample, whether to add absolute timestamp is configurable by NW.

- Other than the first logging sample, add the time interval between its previous logging sample only if it is larger than the configured logging periodicity (i.e., only add gap larger than logging periodicity).

Proposal 3 (open issue RRC-52): RAN2 confirm that the solution agreed in RAN2#130 is only applicable to regular HO. To enable NW control on retaining logged data at CHO/LTM, source cell makes decision and includes a list of 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration with LTM configuration or CHO condition.

Proposal 4 (open issue RRC-19): Full buffer indication is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (i.e. no additional configuration bit is needed).

Proposal 5 (open issue RRC-25/RRC-30): As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

Proposal 6 (open issue RRC-31): Same as logged MDT, only specify 48 hour storage validity duration for logged L1 measurements.

Proposal 7 (open issue RRC-39): Only support single field to request all the data in the buffer, i.e. don’t introduce request indication for one specific use case or one specific configuration ID.

Proposal 8 (open issue RRC-28): Follow legacy RRC procedure when to release the UAI configuration in OtherConfig for NW-side data collection upon entering IDLE/INACTIVE state or detection of RLF.

Key UE capability open issues of NW-side data collection

Proposal 9 (open issue UECap-1): Only specify 64KB minimum buffer size for NW-side data collection. It is up to UE implementation whether to allocate additional memory.

Proposal 10 (open issue UECap-2): RAN2 agree the following details of UE capability of NW-side data collection:

- The capability of periodic logging and reporting assistance information for NW-side data collection are integrated into general capability of NW-side data collection (i.e. it is mandatory to support periodic logging and UAI if the UE reports supporting of NW-side data collection).

- Introduce a separate capability of event-based logging.

- Introduce a separate capability of logging measurement quantities and resources for AI/ML based BM (Set A/B, maximum resources of Set A/B, etc).

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#130, Chair Notes.

[3] R2-2505778, Remaining RRC open issues in feature AIML PHY, Ericsson.

[4] R2-250xxxx, Report of [POST130][038][AI PHY] UE capabilities (Xiaomi), Xiaomi.

[5] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[6] RAN2#129b, Chair Notes.

[7] R2-2505501, Running MAC CR for AI/ML for Air Interface, Apple (Rapporteur).

[8] RAN2#128, Chair Notes.

[9] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.