3GPP TSG-RAN WG2#131	R2-2505507

Bengaluru, India, Aug 25th – 29th 2025

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Remaining issues on on-demand SIB1 procedure

Document for:	Discussion and decision

# Background

In RAN2#130 meeting, RAN2 made some agreements as follows [1].

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

In this contribution, we would like to discuss and provide our views on the remaining open issues for finalization of on-demand SIB1.

# Discussion

## SIB1 request procedure

### SIB1 request configuration

(RRC/OD-SIB1 issue1) Stored SIB1 in connected mode UE

The following remaining issue is still open:

- “Q12: Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?”.

To clarify the UE behaviours in connected mode, it is reasonable to add some text in the field description. In addition, because it is applied only in a case of OD-SIB1 cell, it should be indicated by “if SIB1 is being transmitted on-demand in this cell”.

Proposal 1:	(RRC/OD-SIB1 issue1) Add “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1, if SIB1 is being transmitted on-demand in this cell” to the field description of si-BroadcastStatus.

Maximum value of PCIs sharing the same OD-SIB1 configuration

The value for maxNrofODSIB1-r19 and maxPCI-r19 is defined as 64 and 8, respectively, in the RRC running CR [2]. The IE maxNrofODSIB1-r19 defines the maximum number of OD-SIB1 configurations for the target NES cells. The IE maxPCI-r19 defines the maximum number of PCIs sharing the same OD-SIB1 configuration per frequency. The value of maxNrofODSIB1-r19 (i.e., 64) is decided by 8 OD-SIB cells x 8 supported frequencies.

However, for IE maxPCI-r19, it depends on the network deployments and it might be a bit difficult to expect the number of NES cells in practical network. So, the easiest way is just reusing the existing cell list configuration. It means the physCellIdList-r19 should be indicated by ‘PCI range’ instead of PCIs. The proposed changes for SIBxx are summarised in TP in Annex A.

Proposal 2:	Instead of phyCellId, PCI-Range is used for the physCellIdList-r19.

### SIB1 request transmission

SIB1 request for Supplementary UL

In RAN2#130 meeting, it was discussed whether to support SUL for SIB1 request. It was postponed for checking the RAN1 discussion about SUL. In RAN1#120 meeting, it was discussed to include into the SIB1 request configuration and RAN1 has reached “Supplementary UL is not included in the UL-WUS configuration” as there were few supports [3].

Observation 1:	RAN1 agreed that SUL is not included in the SIB1 request configuration.

Because SUL is used for UL coverage enhancements, but we think OD-SIB1 cell is typically small cell therefore there is no UL coverage shortage. In addition, if OD-SIB1 cell support SUL, the additional p-max and the frequencyShift7p5khz should also be introduced in SIB1 request configuration for selected SUL carrier but it requires additional discussion in RAN1. Then it seems too late to support SUL for OD-SIB1 request.

Proposal 3:	SUL is not supported for OD-SIB1 request.

Proposal 4:	Remove the EN ”FFS: if there is need to emphasize it is normal uplink” in section 5.2.2.3.3x in RRC CR and keep the current procedural text.

Correction for SIB1 request triggering condition

Based on the comment in the e-mail discussion [2], the conditions about the current RRC state were removed as below. But we think it is technically not correct. Because the conditions are used to check the RRC state where the SIB1 request is allowed. Especially, the UE met the yellow highlighted conditions can also need SIB1 acquisition, if these conditions are missing, the UE in connected mode is allowed to transmit SIB1 request even if T311 is not running.

##### 5.2.2.3.1 Acquisition of MIB and SIB1 <<skip unchanged part>> 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 45> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5.

Thus, the condition removed above should be reverted to control a SIB1 request triggering.

Proposal 5:	Revert the removed conditions for SIB1 request triggering in section 5.2.2.3.1 in RRC CR as follows:

3>	if the UE has a stored valid version of od-SIB1-Config for this cell:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5>	perform the actions as specified in clause 5.2.2.3.3x;

### On-demand SIB1 reception

(RRC/OD-SIB1 issue2) SI change after camping in NES cell

Two following questions were raised in the e-mail discussion about RRC running CR [2]:

Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update?

Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this?

In our understanding, the answer of Q1 is NO, it depends on NW implementation whether to change its NCD-SSB to CD-SSB during the updated SIB1 is sent. Next, for Q2, the UE can acquire the updated SIB1 as the same as OD-SIB1 request transmission if OD-SIB1 request configuration for the NES cell is not changed. If the UE cannot acquire the updated SIB1 e.g., due to updating CORESET#0 of the NES cell, the NES cell should stop energy saving mode and will start legacy operation with periodical SIB1 transmission. Thus, because the NW starts to transmit SIB1 periodically at the boundary of SI modification period in case of SI change, the existing UE behaviour can be reused to reacquire the updated SIB1 neither transmitting SIB1 request nor returning to the Cell A. Considering such a NW behaviour, no further clarification is required.

Observation 2:	For SI change in NES cell, it is up to NW implementation to transmit the updated SIB1 periodically or on-demand.

Observation 3:	For SI change in NES cell, the existing UE behaviour can be reused to identify the updated SIB1 is transmitted periodically or on-demand.

Proposal 6:	(RRC/OD-SIB1 issue2) In case of SI change in NES cell, no new UE behaviour is required for updated SIB1 acquisition.

# Conclusion

In this contribution, the following proposals and observations were made:

Proposal 1:	(RRC/OD-SIB1 issue1) Add “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1, if SIB1 is being transmitted on-demand in this cell” to the field description of si-BroadcastStatus.

Proposal 2:	Instead of phyCellId, PCI-Range is used for the physCellIdList-r19.

Proposal 3:	SUL is not supported for OD-SIB1 request.

Proposal 4:	Remove the EN ”FFS: if there is need to emphasize it is normal uplink” in section 5.2.2.3.3x in RRC CR and keep the current procedural text.

Proposal 5:	Revert the removed conditions for SIB1 request triggering in section 5.2.2.3.1 in RRC CR as follows:

3>	if the UE has a stored valid version of od-SIB1-Config for this cell:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5>	perform the actions as specified in clause 5.2.2.3.3x;

Proposal 6:	(RRC/OD-SIB1 issue2) In case of SI change in NES cell, no new UE behaviour is required for updated SIB1 acquisition.

# References

- R2-2504671, Report from session on R18 SL, R18/19 MOB, and R19 NES

- R2-2505708, Running RRC CR for enhancements for network energy efficiency

- R1-2501380

# Annex A: TP for SIBxx

### 6.3.1	System information blocks

### –	SIBxx

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-ConfigList-r19         SEQUENCE (SIZE(1..maxNrofODSIB1-r19))  OF OD-SIB1-Config-r19

lateNonCriticalExtension       OCTET STRING                           OPTIONAL,

...

}

OD-SIB1-Config-r19  ::=        SEQUENCE {

carrierFreq-r19                    ARFCN-ValueNR,

physCellIdList-r19                 SEQUENCE (SIZE(1..maxPCI-r19)) OF PCI-Range,

sib1-rsrp-ThresholdSSB-r19         RSRP-Range                       OPTIONAL, -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                                 INTEGER (0..837),

l139                                 INTEGER (0..137)

}                                                                    OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}       OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}            OPTIONAL, -- Need R

offsetToCarrier-r19                      INTEGER (0..2199),

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks)                                      OPTIONAL, -- Need R

locationAndBandwidth-r19                 INTEGER (0..37949          OPTIONAL, -- Need R

absoluteFrequencyPointA-r19              ARFCN-ValueNR              OPTIONAL, -- Cond FDD

offsetToPointA-r19                       INTEGER (0..2199)          OPTIONAL, -- Cond TDD

frequencyBandList-r19ultiFrequencyBandListNR-SIB                        OPTIONAL, -- Need R

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB    OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480-v1700, kHz960-v1700} OPTIONAL, -- Need R

p-Max-r19                                P-Max                      OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                   INTEGER (-60..50)          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                 SEQUENCE {

inOneGroup                            BIT STRING (SIZE (8)),

groupPresence                         BIT STRING (SIZE (8))      OPTIONAL  -- Cond FR2-Only

},

ssb-Periodicity-r19                       ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2}           OPTIONAL, -- Need R

od-sib1-WindowDuration-r19                ENUMERATED {ms20, ms40, ms80, ms160, ms320, spare1, spare2, spare3}   OPTIONAL, -- Need R

od-sib1-windowStartOffset-r19             INTEGER (1..FFS),

k-ssb-r19                                 INTEGER (1..23),

sib1-RequestConfig-r19                    SIB1-RequestConfig-r19

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19             INTEGER (1..63),

rach-OccasionsSIB1-r19                    SEQUENCE {

ssb-perRACH-Occasion-r19                   ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}           OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SIB1-RequestResources-r19,

sib1-pdcch-RestrictionToPRACH-r19         ENUMERATED {true, false},

rach-ConfigSIB1-r19                       RACH-ConfigGeneric

controlResourceSetZero-r19                ControlResourceSetZero,

searchSpaceZero-r19                       SearchSpaceZero,

ra-SearchSpace-r19                        SearchSpaceId                OPTIONAL,   -- Need S

n-TimingAdvanceOffset-r19                 ENUMERATED { n0, n25600, n39936, spare1 }                               OPTIONAL    -- Need R

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)           OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)           OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

<<skip unchanged part>>