3GPP TSG RAN WG2 #131                             	R2-2505509

Bengaluru, India, August 25th– 29th, 2025

Agenda item:	8.5.3

Source: 	Sharp

Title: 	Remaining open issues on OD-SIB1 request

Document for:	Discussion and Decision

Introduction

In this paper, we would discuss the following potentional open issues.

- Whether SUL carrier should be included in OD-SIB1 request.

- OD-SIB1 update with SIB1-RequestConfig modification on a NES cell.

Discussions

Supplementary Uplink

In RAN2#130 meeting [1], RAN2 WG decided to revisit the issue whether On-demand SIB1 request should be supported on both SUL and NUL carrier.

However, please note, RAN1 WG already concluded not to include supplementary uplink in the UL-WUS configuration in R1#120 meeting [2].

- RAN1 WG already reached a conclusion that SUL is not included in the UL-WUS     configuration.

In addition, adding additional RACH configuration on SUL carrier would also influence the network energy saving gain since the NES cell need to monitor the RACH occasions on both NUL carrier and SUL carrier periodically. In addition, since the RACH configuration for UL-WUS transmission is configured independently, serving RAN can control the OD-SIB1 configuration to facilitate UEs on the SUL coverage to initiate a OD-SIB1 request procedure. Therefore, we would suggest not to support supplementary uplink in the UL WUS configuration.

- Supplementary uplink should be exempt from UL-WUS configuration for the purpose of network energy saving.

OD-SIB1 update with CORESET#0 modification

- In the [POST130][107][NES]e-mail discussion, one potential open issue for OD-SIB1 was raised for OD-SIB1 update with CORESET#0 modification on a NES cell. Firstly, a NES cell is broadcasting NCD-SSB without broadcasting SIB1. Then, the NES cell decides to update OD-SIB1 and so the NES cell would start to broadcast the SI change notification short message to UEs. In addition, the NES cell may stick on NCD-SSB broadcasting (i.e., without broadcasting CD-SSB) for the OD-SIB1 update.

- NES cell may stick on NCD-SSB broadcasting (i.e., no CD-SSB broadcasting) even during the OD-SIB1 update.

- Furthermore, it is possible that NES cell also wants to change the parameters in the SIB1-RequestConfig along with the OD-SIB1 update (e.g., a new ControlResourceSetZero is also updated during this OD-SIB1 update), or a more likely condition: the modification of SIB1-RequestConfig (which has been broadcasted by the NES cell as part of other SI) is the root cause of this OD-SIB1 update.

- To the UE, whom has stored SIB1 and SIB1-RequestConfig configuration, it would try to monitor the updated OD-SIB1 after receiving the SI change notification message. However, some companies doubted it is impossible for the UE to receive the updated SIB1 successfully. It is because the NES cell would broadcast new SIB1 based on the new ControlResourceSetZero but the UE can only monitor the SIB1 based on the original ControlResourceSetZero of the stored SIB1-RequestConfig configuration.

- To this issue, we can also refer to the agreements in the RAN2#130 meeting [1]. It was already agreed that UE would follow the legacy validity principle of stored SIB before the UE receives the update SIB. Based on the agreement, the UE would consider the stored SIB1 and SIB1-RequestConfig configuration are still valid until the UE receives the updated SIB1 and SIB1-RequestConfig configuration. The legacy validity rule should also apply to the NES cell when it starts broadcasting the updated OD-SIB1 and the updated SIBxx. Therefore, the NES cell should broadcast the updated OD-SIB1 based on the original ControlResourceSetZero, rather than the updated ControlResourceSetZero. In addition, since ControlResourceSetZero is going to be updated, the NES cell should also broadcast the updated SIBxx accordingly during the SI change procedure. RAN2 WG is suggested to confirm that the procedure above is already supported and no additional spec impact to be considered.

- For OD-SIB1 update with SIB1-RequestConfig modification on a NES cell, RAN2 WG is suggested to confirm that the NES cell should broadcast the updated OD-SIB1 based on original SIB1-RequestConfig provided by the NES cell. Then, SIB1-RequestConfig modification would be implemented jointly as part of the SI update procedure.

Conclusions

In this paper, we proposed that RAN2 WG to not include SUL carrier for OD-SIB1 request.

- RAN1 WG already reached a conclusion that SUL is not included in the UL-WUS     configuration.

- Supplementary uplink should be exempt from UL-WUS configuration for the purpose of network energy saving.

- Then, to OD-SIB1 update with SIB1-RequestConfig modification on a NES cell, it is observed that a NES cell may stick on NCD-SSB broadcasting (i.e., no CD-SSB broadcasting) even during the OD-SIB1 update. Even so, the NES cell should broadcast the updated OD-SIB1 based on original SIB1-RequestConfig provided by the NES cell. Then, SIB1-RequestConfig modification would be implemented jointly as part of the SI update procedure. The procedure is already supported based on RAN2 agreements.

- NES cell may stick on NCD-SSB broadcasting (i.e., no CD-SSB broadcasting) even during the OD-SIB1 update.

- For OD-SIB1 update with SIB1-RequestConfig modification on a NES cell, RAN2 WG is suggested to confirm that the NES cell should broadcast the updated OD-SIB1 based on original SIB1-RequestConfig provided by the NES cell. Then, SIB1-RequestConfig modification would be implemented jointly as part of the SI update procedure.

References

R2-2504671 Report from session on R18 SL, R18/19 MOB and R19 NES

Chair’s notes RAN1#120, Feb. 17th – 21st, 2025