3GPP TSG RAN WG2#131	R2-2505552

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item:	8.9.4

Source:	OPPO

Title:	Discussion on PWS for NB-IoT

Document for:	Discussion, Agreement

Introduction

In last RAN2 meetings, RAN2 discussed the support of PWS for NB-IoT, and the following agreements were made so far.

Agreements in RAN2#127bis meeting:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

Agreements in RAN2#128 meeting:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

Agreements in RAN2#129 meeting:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

- Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

- We don’t introduce a new SIB for carrying geographical information for emergency areas

Agreements in RAN2#129bis meeting:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

Agreements in RAN2#130 meeting:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

2.	Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In this contribution, we further discuss open issues on support of PWS for NB-IoT NTN and present our view.

Discussion

In LTE, a cell type called acceptable cell is defined, which enables UE to camp on a cell for limited service including PWS and emergency calls, without requiring that the cell is a part of the UE’s LPMN. Since both PWS and emergency calls are not supported for NB-IoT before, the concept of acceptable cell is not supported for NB-IoT. In Rel-19, with the introduction of support of PWS in NB-IoT NTN, whether to support acceptable cell for PWS reception was discussed in RAN2130 meeting, with the following agreement made.

Agreements:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

Regarding how to reflect the above agreement in the spec, there may be two options：

- Option 1: Introduce an acceptable cell category for NB-IoT

- Option 2: Only simply specify that “a PWS capable NB-IoT UE may camp on an NB-IoT cell for PWS reception, without requiring the cell to be a part of UEs PLMNs”

In our view, option 1 is more straightforward, we can just extend the current acceptable cell category to NB-IoT. For option 2, it is unclear how/when UE camps on a cell only for PWS reception, so we may need more discussion on UE behaviour. Considering that there is only one meeting left in Rel-19, we prefer option 1.

Introduce an acceptable cell category for NB-IoT (at least for NTN).

- Conclusion

Based on the discussion we propose the following:

- Introduce an acceptable cell category for NB-IoT (at least for NTN).