3GPP TSG-RAN WG2 Meeting #131	        											                         R2-2505567

Bengaluru, India, Aug 25th – 29th, 2025                                                  Revision of R2-2504527

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

According to the SA2 CR [4], a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Note that how UE handle the S&F Monitoring List is up to UE implementation.

S2-2502513 When an on-board MME supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update. NOTE 2: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O. The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served by an MME operating in S&F Mode. For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with asatellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

The satellite ID in the monitoring list can be helpful in the cell selection and cell reselection. UE can prioritize the cell in the monitoring list. To align with SA2, the detail can be up to UE implementation.

Proposal 1: It is up to UE implementation to prioritize the cell in the monitoring list during the cell selection and cell reselection.

In the last RAN2 meeting R2#130, the following working assumption was made:

Working Assumption: 1. In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

However, we should notice that the S&F mode should be satellite specific rather than cell specific. Therefore, the S&F mode status for neighbour cells should be broadcast in the SIB33. The current satellite ID list in SIB3/5 can be used to associate with a specific satellite in SIB33.

Proposal 2: If the WA on S&F mode indicator for neighbour cells is confirmed, the neighbour cell S&F mode status should be broadcast in SIB33.

Neighbor cell S&F mode status can help improve the cell selection and reselection, similar to the S&F monitoring list. Since how UE behaves when receiving the S&F Monitoring List is up to UE implementation, we should take the same approach for the neighbor the S&F mode status.

Proposal 3: How to use the neighbour cell S&F mode status is up to UE implementation.

Regarding the FFS S&F transition time of neighbor cell, no significant benefit is foreseen. Given the limit remaining time for RAN2 discussion, support for neighbor cell transition time should not be included in Rel-19.

Proposal 4: Neighbor cell transition time for S&F mode status is NOT supported in Rel-19.

Some companies are also interested in introducing paging relaxing in S&F mode. But paging relaxing can be risky, we might need more time to ensure it does not have negative impact to the UE. There could also be impacts on RAN3. Considering the limit remaining time for RAN2 discussion, the potential complexity and the uncertain risks, we suggest not supporting paging relaxing for S&F in Rel-19.

Proposal 5: Paging relaxing in S&F mode is NOT supported in Rel-19.

# Conclusion

Proposal 1: It is up to UE implementation to prioritize the cell in the monitoring list during the cell selection and cell reselection.

Proposal 2: If the WA on S&F mode indicator for neighbour cells is confirmed, the neighbour cell S&F mode status should be broadcast in SIB33.

Proposal 3: How to use the neighbour cell S&F mode status is up to UE implementation.

Proposal 4: Neighbor cell transition time for S&F mode status is NOT supported in Rel-19.

Proposal 5: Paging relaxing in S&F mode is NOT supported in Rel-19.

# References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3

TS 23.401 v19.1.0 General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

S2-2502513, “Support of S&F Monitoring List in S&F Mode”, Huawei, HiSilicon, Samsung CR	Rel-19	23.401	5GSAT_Ph3-ARC