3GPP TSG-RAN WG2 Meeting #131	R2- 2505578

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Concluding XR rate control for Rel-19

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#130:

Single or multiple QoS flows (MAC-05) - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. ID for flow identification (MAC-04) - We rule out explicit signalling of PDU session ID + QFI - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping Procedure details and prohibit timer configuration (RRC-1, MAC-08, MAC-10) - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. Available bit rate query (MAC-12) - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. Impact of Dual-Connectivity (MAC-9) - No optimization is needed to address DC case for XR rate control MAC CE - We capture in stage-2 that XR rate indication is for application and not for MAC entity. DL rate control (MAC-11) - RAN2 will not specify DL rate control and DL rate control query

This contribution discusses the open issues related to XR rate control summarized during the post email discussion [505].

[MAC-3]	Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

[MAC-4]	Format of the UL Rate Control MAC CE.

[MAC-5]	Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

Additionally, it raises an issue regarding the start/restart condition of the prohibit timer.

# 2	Discussion

## 2.1	[MAC-3/4] Format of the UL Rate Control MAC CE

Given that support of DL rate control is excluded in the last meeting, we note that UL rate control MAC CE requires two fields – QoS flow ID information to identify a QoS flow and Bit rate index.

For the QoS flow ID information, RAN2 ruled out using PDU session ID + QFI, while two options remain under consideration:

ID for flow identification (MAC-04) We rule out explicit signalling of PDU session ID + QFI We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

First, we need to determine the maximum number of QoS flows for which the rate control could be indicated, as this affects how the QoS flow ID information is signalled. TS 38.331 specifies that the maximum number of QFI (maxNrofQFIs) is 64, meaning 6 bits are needed to flexibly indicate any QoS flows. However, not all QoS flows are applicable for rate control – this is why we obtain information from the CN which QoS flows are eligible for rate control. Furthermore, in practice, only a small subset of QoS flows within a logical channel is expected to require bit rate adjustment, e.g., one or two QoS flows, since gain of rate control would be flow-dependent and gNB is likely to selectively control the bit rate by considering the QoS flow type.

Therefore, it seems sufficient and reasonable to assume that a maximum of two QoS flows within a logical channel can be signalled in the MAC CE.

Proposal 1 (MAC-3/4): RAN2 assumes that a maximum of two QoS flows within a logical channel require uplink rate control for MAC CE design.

With Proposal 1, we will discuss how to convey the QoS flow information in the MAC CE.

Option 1. Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

TS 38.331 specifies that the maximum number of DRBs (maxDRB) is 29 and the maximum Logical Channel ID (maxLC-ID) is 32. Therefore, 6-bit field is required to indicate either the DRB or the LCH ID in the MAC CE.

For QFI, as the largest QoS flow ID (maxNrofQFIs) is 63, a 6-bits field is required to signal the QoS flow ID directly. However, if only 1 or 2 QoS flows are subject to rate control as in proposal 1, 1- or 2-bit field for QoS flow ID can be considered by using a bitmap. Assuming both the UE and the gNB know which QoS flows are rate controllable via MAC CE, each bit of the bitmap can represent a QoS flow in ascending order of the QoS flow ID among the rate controllable QoS flows within the logical channel.

Meanwhile, RAN2 agreed that "XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify". To signal the bitrate for multiple QoS flows belonging to the same LCH, repeating the LCID field for the QoS flows belonging to the same LCH would only unnecessarily increases the overhead by 1 octet per entry.

One possible option is to include the LCID field only once for the QoS flows that share the same LCID. For example, in the case of two QoS flows for rate control with the same LCID, this alternative would require 3 octets of MAC CE – 6 bits of LCID followed by 2 bits for QFI + 2 octets for Bit Rate. In contrast, including LCID for each QoS flow would result in 4 octets – 2 octets for each QoS flow. Figure 1 shows an example MAC CE format allowing omitting the LCID field for the QoS flows belonging to the same LCH. Figure 1 shows an example MAC CE format with option 1.

Figure 1. Example of option 1 with LCID field, bitmap QoS flow index, and Bit Rate field.

Option 2. Implicit, e.g. index or mapping

Index approach of Option 2 would assign an index to each rate controllable QoS flow across all LCHs (global indexing), e.g., in ascending order of LCH ID and QoS flow ID. For instance, if QoS flow 1 and 2 from LCH1 and QoS flow 2 from LCH2 are rate controllable, the index 1, 2, and 3 can be assigned to these QoS flows sequentially. The size of MAC CE would be 2N octets if the MAC CE delivers bit rate information for N QoS flows. Figure 2 depicts an example MAC CE format of index approach of option 2 assuming 6-bits of QoS flow index field.

Figure 2. Example of option 2 with index approach – QoS flow index field and Bit Rate field.

Mapping approach of Option 2 would have a bitmap field to indicate the presence of Bit Rate field of a QoS flow of a LCH, e.g., in ascending order of LCH ID and QoS flow ID (global mapping). This would require fixed size of bitmap field depending on the maximum number of QoS flows that we want to control in total. For instance, if we assume maximum 8 QoS flows are rate controllable in total, 1 octet of bitmap for QoS flow indication is required. For this option, therefore, we might need to additionally determine the maximum number of QoS flows that we want to control per UE, i.e., not per LCH. Figure 3 shows an example MAC CE format of option 2 with mapping approach.

Figure 3. Example of option 2 with mapping approach – Bitmap of QoS flow index field and Bit Rate field.

From overhead perspective, option 2 with mapping approach is the most efficient since it reduces QFI signalling size by introducing a global mapping across LCHs. However, this option would be favoured only when the total number of controllable QoS flows (across LCHs) is small, e.g., 8, to keep the bitmap field size within a reasonable limit, e.g., 1 or 2 octets.

Option2 with index approach and option 1 would offer similar overhead when controlling the bitrate for multiple QoS flows from different LCHs. However, when the QoS flows belong to the same LCH, option 1 achieves lower overhead by omitting repeated LCID field.

Overall, we consider option 1 to be overhead efficient and sufficiently scalable solution as it leverages LCID and per LCH QoS flow indexing without requiring global indexing/mapping across the LCHs and can support maximum two QoS flows for every LCH.

Proposal 2 (MAC-3/4): UL rate control MAC CE includes 6 bits of LCID field, 2 bits of QoS flow index field, and 8 bits of Bit Rate field:

-	LCID field indicates the presence of Bit Rate information for the LCH i.

-	QoS flow index field indicates the presence of Bit Rate information for the QoS flow j belonging to the LCH i.

-	Bit Rate field indicates the Bit Rate information of the QoS flow j of LCH i

Proposal 3 (MAC-4): The LCID field is omitted for the rest of QoS flows belonging to the same LCH.

## 2.2	[MAC-5] Truncation of UL rate control MAC CE

The open issue [MAC-5] is to Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

In legacy RBR procedure, a 2-octet MAC CE is defined to indicate the bit rate value for a single logical channel. Although there may be a scenario where queries are pending for multiple logical channels, the specification does not define any special handling for such cases. Since the trigger of query itself is a UE implementation, it should be possible to avoid triggering of queries for multiple LCHs simultaneously, and such situation are unlikely to occur frequently in practice.

Unlike RBR MAC CE, the UL rate control MAC CE itself can indicate the bit rates for multiple flows, thereby reducing overhead already. Moreover, as the query serves as an auxiliary mechanism for UL rate control, the likelihood of queries being triggered simultaneously for multiple flows – resulting in tens of octets – is considered low.

Therefore, additional optimization is deemed not necessary.

Proposal 4 (MAC-5): The UL rate control MAC CE can be transmitted when available UL-SCH resources can accommodate the UL rate control MAC CE including all the pending queries plus its subheader.

## 2.3	Prohibit timer

During post email discussion [506], it is agreed that the value of the prohibit timers are the same to all QoS flows.

In addition to the prohibit timer value, we would like to discuss the conditions for starting or restarting the prohibit timer.

When the gNB transmits an uplink rate control MAC CE to the UE, it would be less efficient for the UE to query soon, as it could still be a congestion situation and may only cause a ping-pong effect. Therefore, it would be beneficial to prevent the UE from querying in such case.

Proposal 5: For XR rate control, RAN2 discusses whether the prohibit timer starts or restarts when the UE receives the uplink rate control MAC CE from the gNB.

# 3	Conclusion

This document has made the following observations:

Proposal 1 (MAC-3/4): RAN2 assumes that a maximum of two QoS flows within a logical channel require uplink rate control for MAC CE design.

Proposal 2 (MAC-3/4): UL rate control MAC CE includes 6 bits of LCID field, 2 bits of QoS flow index field, and 8 bits of Bit Rate field:

-	LCID field indicates the presence of Bit Rate information for the LCH i.

-	QoS flow index field indicates the presence of Bit Rate information for the QoS flow j belonging to the LCH i.

-	Bit Rate field indicates the Bit Rate information of the QoS flow j of LCH i

Proposal 3 (MAC-4): The LCID field is omitted for the rest of QoS flows belonging to the same LCH.

Proposal 4 (MAC-5): The UL rate control MAC CE can be transmitted when available UL-SCH resources can accommodate the UL rate control MAC CE for all the pending queries plus its subheader.

Proposal 5: For XR rate control, RAN2 discusses whether the prohibit timer starts or restarts when the UE receives the uplink rate control MAC CE from the gNB.