3GPP TSG-RAN WG2 Meeting #131	R2-2505582

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.1.2.2

Source:	NEC

Title:	Remaining open issues on LCM for UE-sided model for Beam Management

Document for:	Discussion, Decision

Introduction

In RAN2#130 meeting, a good progress was achieved on LCM for UE-sided model for Beam Management use case.

This contribution further discusses remaining RRC open issues on LCM for UE-sided model for Beam Management use case, which are summarized as the output of the email discussion, [POST130][026][AI PHY] 38.331 CR (Ericsson).

Discussion

Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters.

Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models.

Based on the agreements reached at the RAN2#129 meeting, we understand that the time duration for an AI functionality to become available for inference, when the UE reports applicability, is implicitly included in the applicability determination processing performed on the UE side. Therefore, we believe there is no need to discuss this issue from the RAN2 point of view. If a requirement discussion is truly necessary, the appropriate working group would be RAN4.

Agreements at RAN2#129 meeting:

・Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

・Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

・If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

・Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

(RRC-15) There is no need to discuss this issue from RAN2 point of view. If a requirement discussion is truly necessary, the appropriate working group would be RAN4.

Open issue RRC-40: Configuration for Option B for applicability reporting

Issue description: RAN2 is still to confirm the assumption that option B is configured in otherConfig.

RAN2#129bis agreed that:

RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Subsequently, RAN1#121 agreed that:

Agreement For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, Note: this doesn’t imply the associated ID is always present

In the RAN1 LS (R1-2503242 LS on Rel-19 higher layers parameters list Post RAN1#121,attachenment is R1-2503243, copied below), the RAN2 parent IE of all Rel-19 AI/ML-related higher-layer parameters is CSI-ReportConfig (i.e., the list of inference-related parameters is fully contained within the existing CSI-ReportConfig and is common to both Option A and Option B).

Table 1 Consolidated RRC parameters for Rel-19 AI/ML PHY

(RRC-40) The list of inference-related parameters is fully contained within the existing CSI-ReportConfig and is common to both Option A and Option B.

Based on the above, we prefer to adhere to the agreement in RAN2#129bis to reconsider CSI-ReportConfig for Option B, as there is no compelling reason to insist on using otherConfig, which would introduce redundancy.

(RRC-40) For Option B, one set or multiple sets of inference related parameters for applicability report are configured via CSI-ReportConfig.

Another thing needs to be discussed is that whether option A and option B result in different UE behavior. As agreed in RAN2#129 meeting, activation of semi-persistent and aperiodic CSI reporting follow legacy CSI framework, as copied below.

・Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

Since the activation of the semi-persistent and aperiodic CSI reporting of applicable functionality is fully controlled by the NW after receiving applicability report, the UE behavior for the activation of semi-persistent and aperiodic CSI reporting is therefore common to both Option A and Option B.

(RRC-40) UE behavior on activation of semi-persistent and aperiodic CSI reporting is fully controlled by the NW and is common to both Option A and Option B.

On the other hand, UE behavior on activation of the periodic CSI reporting is summarized in the following table.

Table 2 UE behavior on activation of the periodic CSI reporting of applicable functionality

|  | Option A | Option B |
| --- | --- | --- |
| Step 3 | one or more of CSI-ReportConfig for inference configuration | One set or multiple sets of inference related parameters for applicability report only (not for inference) via CSI-ReportConfig |
| Step 4 | UE reports applicability for all the above one or more CSI-ReportConfig | UE reports applicability for all the set(s) of inference related parameters |
|  | the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). |  |
| Step 5 | (optional) NW can configure CSI-ReportConfig for inference configuration in RRCReconfiguration, | NW configures CSI-ReportConfig for inference configuration in RRCReconfiguration, |
|  |  | periodic CSI Report is considered as activated after RRCReconfigurationComplete. |

As shown in Table 2, for Option A, the UE autonomously activates the applicable functionalities upon reporting them via RRCReconfigurationComplete, whereas for Option B, the UE does not activate them autonomously.

(RRC-40) UE behavior on activation of the periodic CSI reporting is different for Option A and Option B.

Therefore, to indicate whether Option A or Option B is configured via CSI-ReportConfig, a flag is needed. With this new flag, a unified signalling procedure can support both applicability report for Option A and Option B.

(RRC-40) For periodic CSI reporting, support a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig.

Conclusion and Proposal

We have the following observations and proposals:

Observation 1	(RRC-15) There is no need to discuss this issue from RAN2 point of view. If a requirement discussion is truly necessary, the appropriate working group would be RAN4.

Observation 2	(RRC-40) The list of inference-related parameters is fully contained within the existing CSI-ReportConfig and is common to both Option A and Option B.

Observation 3	(RRC-40) UE behavior on activation of semi-persistent and aperiodic CSI reporting is fully controlled by the NW and is common to both Option A and Option B.

Observation 4	(RRC-40) UE behavior on activation of the periodic CSI reporting is different for Option A and Option B.

Proposal 1	(RRC-40) For Option B, one set or multiple sets of inference related parameters for applicability report are configured via CSI-ReportConfig.

Proposal 2 (RRC-40) For periodic CSI reporting, support a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference-related parameters is contained in that CSI-ReportConfig.

Reference

- Report of 3GPP TSG RAN WG2 meeting #130